3GPP TSG-CT WG1 Meeting #159	C1-260496

# Goa, India, 9th – 13th February 2026                                         revision of C1-260059

| CR-Form-v12.5.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.501 | CR | 7107 | rev | 1 | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PDU session ID in DL NAS TRANSPORT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO, Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GProtoc19 |  |  |  |  |  | Date: |  |  | 2026-02-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature) C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | PDU Session ID is an IE in the DL NAS TRANSPORT message and it is not in the header of the DL NAS TRANSPORT message. In addition, when the UE receives a invalid PDU session ID in the DL NAS TRANSPORT message, the abnormal case is missing, e.g. how to set the PDU session ID in the UL NAS TRANSPORT mesasge which includes 5GSM Status message. If the PDU session ID is not set accordingly to the PDU session ID received, the 5GSM STATUS message can not be routed to the correct SMF. Backward compatibility analysis: The changes are backwards compatible. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is clarified that the PDU Session ID is set to the received one in the UL NAS TRANSPORT when sending the 5GSM STATUS message. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is unclear how to set the PDU session ID in the UL NAS TRANSPORT mesasge which includes 5GSM Status message. The 5GSM STATUS message can not be sent to correct SMF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * First Change * * * *

7.3.2	PDU Session identity

The following network procedures shall apply for handling an unknown, erroneous, or unforeseen PDU session identity received in the header of a 5GSM message (specified as the header of a standard L3 message, see 3GPP TS 24.007 [11]) or in the DL NAS TRANSPORT message (see clause 5.4.5) as identified by 5GSM:

a)	If the network receives a PDU SESSION MODIFICATION REQUEST message which includes an unassigned or reserved PDU session identity value, the network shall respond with a PDU SESSION MODIFICATION REJECT message including 5GSM cause #43 "invalid PDU session identity".

b)	If the network receives PDU SESSION RELEASE REQUEST message which includes an unassigned or reserved PDU session identity value, the network shall respond with a PDU SESSION RELEASE REJECT message including 5GSM cause #43 "invalid PDU session identity".

c)	Upon receipt of an UL NAS TRANSPORT message, the network takes the following actions:

1)	If the Request type IE is set to "initial request" or "initial emergency request" and the message includes a reserved PDU session identity value, the network shall respond with a DL NAS TRANSPORT message with 5GMM cause #90 "payload was not forwarded";

2)	otherwise, if the message includes an unassigned PDU session identity value, a reserved PDU session identity value or an assigned PDU session identity value that does not match an existing PDU session, the network shall respond with a DL NAS TRANSPORT message with 5GMM cause #90 "payload was not forwarded".

d)	If the network receives a 5GSM message other than those listed in items a) through c) above in which the message includes a reserved PDU session identity value or an assigned value that does not match an existing PDU session, the network shall ignore the message.

The following UE procedures shall apply for handling an unknown, erroneous, or unforeseen PDU session identity received in the header of a 5GSM message or in the DL NAS TRANSPORT message (see clause 5.4.5) as identified by 5GSM:

a)	If the UE receives a 5GSM message which includes an unassigned or reserved PDU session identity value, the UE shall ignore the message.

b)	If the UE receives a 5GSM message or DL NAS TRANSPORT message which includes a PDU session identity belonging to any PDU session in state PDU SESSION INACTIVE in the UE, the UE shall respond with a 5GSM STATUS message including 5GSM cause #43 "invalid PDU session identity" and set the PDU session ID to the received PDU session ID in the UL NAS TRANSPORT message as specified in subclause 5.4.5.2.

* * * * End of Changes * * * *