3GPP TSG-CT1 Meeting #41	Tdoc C1-060605

Denver, USA, 13-17 February 2006

| CR-Form-v7.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.229 | CR | 1138 |  rev | 2 |  Current version: | 7.2.0 |  |
| For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Session termination by P-CSCF |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Source:  | Lucent Technologies |  |  |  |  |  |  |
| Work item code:  | IMSProtoc |  |  |  | Date:  |  | 6/2/2006 |
| Category:  | F |  |  |  | Release:  |  | Rel-7 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  | Use one of the following releases: Ph2 (GSM Phase 2) R96 (Release 1996) R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) |  |
| Reason for change:  |  | If the UE sends an SDP offer containing the media parameters that are not allowed by local policy, the session may be established and immediately terminated by a BYE request. i.e.: "When the P-CSCF receives a 200 (OK) response containing SDP offer, the P-CSCF shall examine the media parameters in the received SDP. If the P-CSCF finds any media parameters which are not allowed on the network by local policy, the P-CSCF shall forward the SDP offer and on the receipt of the ACK request containing the SDP answer, it shall immediately terminate the session as described in subclause 5.2.8.1.2." Subsequently, the UE cannot try to re-establish a session with permissible media parameters, since it doesn't know why the session was terminated. |  |  |  |  |  |
| Summary of change:  |  | Include in the BYE request the Reason header the 488 (Not Acceptable Here) response code that will enable the UE to re-establish the failed session. |  |  |  |  |  |
| Consequences if  not approved: |  | The user - upon initiating or modifying a call - will not know the reason why is the network cleared the call. |  |  |  |  |  |

| Clauses affected:  | 5.2.8.1.2 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Y | N |  |  |
| Other specs  |  | X | Other core specifications  |  |
| affected: |  | X | Test specifications |  |
|  |  | X | O&M Specifications |  |
| Other comments:  |  |  |  |  |

How to create CRs using this form:

Comprehensive information and tips about how to create CRs can be found at http://www.3gpp.org/specs/CR.htm.  Below is a brief summary:

1)	Fill out the above form. The symbols above marked  contain pop-up help information about the field that they are closest to.

2)	Obtain the latest version for the release of the specification to which the change is proposed. Use the MS Word "revision marks"  feature (also known as "track changes") when making the changes. All 3GPP specifications can be downloaded from the 3GPP server under ftp://ftp.3gpp.org/specs/ For the latest version, look for the directory name with the latest date e.g. 2001-03 contains the specifications resulting from the March 2001 TSG meetings.

3)	With "track changes" disabled, paste the entire CR form (use CTRL-A to select it) into the specification just in front of the clause containing the first piece of changed text.  Delete those parts of the specification which are not relevant to the change request.

##### 5.2.8.1.2	Release of an existing session

Upon receipt of an indication that the radio/bearer interface resources are no longer available for a session (e.g. abort session request from PDF) or upon detecting that the SDP offer conveyed in a SIP response contained parameters which are not allowed according to the local policy (as specified in the subclause 6.2), the P-CSCF shall release the respective  dialog by applying the following steps:

1)	if the P-CSCF serves the calling user of the session it shall generate a BYE request based on the information saved for the related dialog, including:

-	a Request-URI, set to the stored Contact header provided by the called user;

-	a To header, set to the To header value as received in the 200 (OK) response for the initial INVITE request;

-	a From header, set to the From header value as received in the initial INVITE request;

-	a Call-ID header, set to the Call-Id header value as received in the initial INVITE request;

-	a CSeq header, set to the current CSeq value stored for the direction from the calling to the called user, incremented by one;

-	a Route header, set to the routeing information towards the called user as stored for the dialog;

-	a Reason header that contains:

-	a 503 (Service Unavailable) response code, if radio/bearer interface resources are no longer available; or

-	a 488 (Not Acceptable Here) response code, if a SDP offer conveyed in a SIP response contained parameters which are not allowed according to the local policy; and

-	further headers, based on local policy.

2)	If the P-CSCF serves the called user of the session it shall generate a BYE request based on the information saved for the related dialog, including:

-	a Request-URI, set to the stored Contact header provided by the calling user;

-	a To header, set to the From header value as received in the initial INVITE request;

-	a From header, set to the To header value as received in the 200 (OK) response for the initial INVITE request;

-	a Call-ID header, set to the Call-Id header value as received in the initial INVITE request;

-	a CSeq header, set to the current CSeq value stored for the direction from the called to the calling user, incremented by one;

-	a Route header, set to the routeing information towards the calling user as stored for the dialog;

-	a Reason header that contains:

-	a 503 (Service Unavailable) response code, if radio/bearer interface resources are no longer available; or

-	a 488 (Not Acceptable Here) response code, if SDP payload contained parameters which are not allowed according to the local policy; and

-	further headers, based on local policy.

3)	send the so generated BYE request towards the indicated user.

4)	upon receipt of the 2xx responses for the BYE request, shall delete all information related to the dialog and the related multimedia session.