3GPP TSG-CT WG3 Meeting #108e	C3-201262

# E-Meeting, 19th – 28th February 2020

Source:	Samsung

Title:	Pseudo-CR on SEAL – Configuration Management API

Spec:	3GPP TS 29.549 v1.0.0

Agenda item:	16.28

Document for:	Decision

1. Introduction

SS_UserProfileRetrieval API has been specified in TS 29.549. This contribution proposes the Open API specification of SS_UserProfileRetrieval API. As there are no call back scenarios this API the notifications related to API definition needs to be clarified as none.

2. Reason for Change

Open API specification of SS_UserProfileRetrieval API is unspecified.

3. Conclusions

1. Added SS_UserProfileRetrieval Apen API specification and included YAML file.

2. Clarify the notifications for the API with none.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 29.549 v1.0.0.

This contribution proposes new Open API specification for SS_UserProfileRetrieval API in the Annex A.

* * * First Change * * * *

#### 7.3.1.3	Notifications

#### None.

* * * 2nd Change * * * *

Annex A (normative):
OpenAPI specification

This is a normative annex clause to specify the Open API representation of all SEAL APIs defined in this specification.

## A.X	SS_UserProfileRetrieval API

openapi: 3.0.0

info:

title: SS_UserProfileRetrieval

description: |

API for SEAL Configuration management.

© 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

version: "1.0.0.alpha-1"

externalDocs:

description: 3GPP TS 29.549 V1.0.0 Service Enabler Architecture Layer for Verticals (SEAL);Application Programming Interface (API) specification

url: http://www.3gpp.org/ftp/Specs/archive/29_series/29.549/

servers:

- url: '{apiRoot}/configuration-management/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in subclause 6.5 of 3GPP TS 29.549

paths:

/{valServiceId}:

get:

description: Retrieve a VAL User or VAL UE's profile information belonging to a VAL service.

parameters:

- name: valServiceId

in: path

description: String identifying an individual VAL service resource

required: true

schema:

type: string

- name: valUserId

in: query

description: String identifying a VAL User. Shall be present if valUEId query parameter is not present.

schema:

type: string

- name: valUEId

in: query

description: String identifying a VAL UE. Shall be present if valUserId query parameter is not present.

schema:

type: string

responses:

'200':

description: The Profile information of the VAL User or VAL UE corresponding to the identifier in the request.

content:

application/json:

schema:

$ref: '#/components/schemas/ProfileDoc'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

schemas:

ProfileDoc:

type: object

properties:

profileInformation:

type: string

description: Profile information associated with the valUserId or valUEId.

valUserId:

type: string

description: Unique identifier of a VAL user.

valUEId:

type: string

description: Unique identifuer of a VAL UE.

required:

- profileInformation

oneOf:

- required: [valUserId]

- required: [valUEId]

* * * End of Changes * * * *