3GPP TSG-CT WG3 Meeting #116e	C3-213431

E-Meeting, 19th – 28th May 2021													(Revision of C3-213212)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.061 | CR | 0538 | rev | 2 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to support L2TP in Diameter message flow |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | BEPoP |  |  |  |  |  | Date: |  |  | 2021-05-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | CT4 has been studied and agreed L2TP supporting for CUPS in WI BEPoP, TR 29.820 has also concluded to support L2TP tunneling over N6/SGi for 5GC/EPS is to be standardized based on the solution#8 as described in 6.8 in Rel-17, and CT3 scope has been added in WI BEPoP. Meanwhile, SA2 LS Reply on the support of L2TP with CUPS in rel-17 to support L2TP tunnelling over N6/SGi for 5GS and EPS, with TS 23.214 CR 0076 approved. Hence the related L2TP support and attributes for Diameter messages need to be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding Diameter attributes within the grouped "Tunneling" AVP and the basic attributes in Diameter AAA message to support DN AAA server providing L2TP information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing the Diameter attributes in the related Diameter message to support L2TP in this specification, and cannot be referred by TS 29.561. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16a.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.214 CR 0076 |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

# Proposed changes:

# *** 1st Change ***

### 16a.4.2	AAA Command

The AAA command, defined in Diameter NASREQ (IETF RFC 4005 [67]), is indicated by the Command-Code field set to 265 and the ‘R’ bit cleared in the Command Flags field., It is sent by the Diameter server to the GGSN/P-GW in response to the AAR command.

The relevant AVPs that are of use for the Gi/Sgi interface are detailed in the ABNF description below. Other valid AVPs for this command are not used for Gi/Sgi purposes and should be ignored by the receiver or processed according to the relevant specifications.

The "Tunneling" AVP may include the "Tunnel-Type" with value 3 to represent L2TP tunnel type, "Tunnel-Medium-Type" and "Tunnel-Server-Endpoint" AVPs. If more than one set of these "Tunneling" AVPs are provided, the optional "Tunnel-Preference" AVP may be provided in each set to identify the relative preference. The Tunnel-Password AVP may be used to authenticate to a remote server.

NOTE:	The other optional AVPs within the "Tunneling" AVPs can be referred to the IETF RFC 4005 [67] with implementation specific.

Editor’s Note: Whether the Tunnel-Password can be included will be confirmed by SA3.

The bold marked AVPs in the message format indicate optional AVPs for Gi/Sgi, or modified existing AVPs.

Message Format:

<AA-Answer> ::= < Diameter Header: 265, PXY >

< Session-Id >

{ Auth-Application-Id }

{ Auth-Request-Type }

{ Result-Code }

{ Origin-Host }

{ Origin-Realm }

[ User-Name ]

[ Service-Type ]

*	[ Class ]

[ Acct-Interim-Interval ]

[ Error-Message ]

[ Error-Reporting-Host ]

[ Failed-AVP ]

[ Idle-Timeout ]

[ Authorization-Lifetime ]

[ Auth-Grace-Period ]

[ Auth-Session-State ]

[ Re-Auth-Request-Type ]

[ Multi-Round-Time-Out ]

[ Session-Timeout ]

*	[ Reply-Message ]

[ Origin-State-Id ]

*	[ Filter-Id ]

[ Port-Limit ]

[ Prompt ]

[ Callback-Id ]

[ Callback-Number ]

*	[ Framed-Compression ]

[ Framed-Interface-Id ]

[ Framed-IP-Address ]

*	[ Framed-IPv6-Prefix ]

[ Framed-IPv6-Pool ]

*	[ Framed-IPv6-Route ]

*	[ Delegated-IPv6-Prefix ]

[ Framed-IP-Netmask ]

*	[ Framed-Route ]

[ Framed-Pool ]

[ Framed-IPX-Network ]

[ Framed-MTU ]

[ Framed-Protocol ]

[ Framed-Routing ]

*	[ Login-IP-Host ]

*	[ Login-IPv6-Host ]

[ Login-LAT-Group ]

[ Login-LAT-Node ]

[ Login-LAT-Port ]

[ Login-LAT-Service ]

[ Login-Service ]

[ Login-TCP-Port ]

*	[ NAS-Filter-Rule ]

*	[ QoS-Filter-Rule ]

*	[ Tunneling ]

*	[ Redirect-Host ]

[ Redirect-Host-Usage ]

[ Redirect-Max-Cache-Time ]

*	[ Proxy-Info ]

[ 3GPP-IPv6-DNS-Servers ]

*	[ External-Identifier]

*	[ AVP ]

*** End of Changes ***