3GPP TSG-CT3 Meeting #119bis-e	C3-220241

# E-Meeting, 17th – 21st January 2022

Source:	Huawei, HiSilicon

Title:	MSGG_N3GDelivery Service Operation

Spec:	3GPP TS 29.538 v0.2.0

Agenda item:	17.34

Document for:	Approval

1. Introduction

This pCR is to propose the Non-3GPP Message Gateway Terminating Message and Delivery status report part of TS 29.538.

2. Reason for Change

As specified in TS 23 554, Non-3GPP Message Gateway Terminating Message and Delivery status report part is needed.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 29.538 v0.2.0.

* * * First Change * * * *

##### 6.3.2.4.1	General

This service operation corresponds to clause 9.2.2.1.2 as defined in 3GPP TS 23.554 [2], is used by MSGin5G Server to deliver MSGin5G message to the Non-3GPP Message Gateway.

* * * Next Change * * * *

##### 6.3.2.4.2	Non-3GPP Message Gateway Terminating Message Delivery

Figure 6.3.2.4.2-1: Non-3GPP Message Gateway Terminating Message Delivery

When the MSGin5G Server needs to send the message to the Non-3GPP Message Gateway, the MSGin5G Server shall send the HTTP POST request towards the "deliver-message" resource as shown in figure 6.3.2.4.2-1.

The MSGin5G Server shall send a POST request to the resource with an N3gMessageDelivery object in the request body.

The N3gMessageDelivery data type shall include:

-	the Originating UE Service ID/AS Service ID within the "oriAddr" attribute;

-	the Recipient UE Service ID/AS Service ID within the "destAddr" attribute;

-	the Message ID within the "msgId" attribute; and

may include:

-	the Application ID within the "appId" attribute;

-	the indication whether the message delivery status report is required within the "isDelivStatReq" attribute;

-	the Payload within the "payload" attribute;

-	the Message is segmented within the "isSegmented" attribute; and

-	the message segment parameters within the "segParams" attribute, this attribute may include:

-	the segmentation set identifier within the "segId" attribute;

-	the total number of message segments within the "totalSegCount" attribute;

-	the message segment number within the "segNumb" attribute; and

-	the last segment flag within the "lastSegFlag" attribute.

Editor's Note:	whether the above attributes are the final version is FFS.

When the Non-3GPP Message Gateway receives the HTTP POST request from the MSGin5G Server, the Non-3GPP Message Gateway shall respond to the MSGin5G Server with a 204 No Content message.

If errors occur when processing the HTTP POST request, the Non-3GPP Message Gateway shall apply error handling procedures as specified in subclause 9.2.6.

* * * Next Change * * * *

##### 6.3.2.5.1	General

This service operation corresponds to clause 9.2.2.2.2 as defined in 3GPP TS 23.554 [2], is used by MSGin5G Server to deliver the delivery status report to the Non-3GPP Message Gateway.

* * * Next Change * * * *

##### 6.3.2.5.2	Non-3GPP Message Gateway Terminating Message Delivery Status Report

Figure 6.3.2.5.2-1: Non-3GPP Message Gateway Terminating Delivery Status Report

When the MSGin5G Server needs to send the delivery status report to the Non-3GPP Message Gateway, the MSGin5G Server shall send the HTTP POST request towards the "deliver-report" resource as shown in figure 6.3.2.5.2-1.

The MSGin5G Server shall send a POST request to the resource with a DeliveryStatusReport object in the request body.

The DeliveryStatusReport data type shall include:

-	the Originating UE Service ID/AS Service ID within the "oriAddr" attribute;

-	the Recipient UE Service ID within the "destAddr" attribute;

-	the Message ID within the "msgId" attribute;and

-	the delivery status within the "deliveryStatus" attribute; and

may include:

-	the failure cause within the "failureCause" attribute.

Editor's Note:	whether the above attributes are the final version is FFS.

When the Non-3GPP Message Gateway receives the HTTP POST request from the MSGin5G Server, the Non-3GPP Message Gateway shall respond to the MSGin5G Server with a 204 No Content message.

If errors occur when processing the HTTP POST request, the Non-3GPP Message Gateway shall apply error handling procedures as specified in subclause 9.2.6.

* * * End of Changes * * * *