3GPP TSG-CT3 Meeting #119bis-e	C3-22xxxx

E-Meeting, 17th – 21th January 2022											(Revision of C3-22xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.522 | CR |  | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Methods and resource of management of 5G access stratum time distribution |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | IIoT |  |  |  |  |  | Date: |  |  | 2022-01-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | NEF service supporting management of 5G access stratum time distribution is defined in clause 5.2.6.25 and clause 4.15.9.4 of TS 23.502. |  |  |  |  |  |  |  |  |
| Summary of change: |  | T Methods and resource supporting management of 5G access stratum time distribution are defined |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Management of 5G access stratum time distribution is not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.15.1.1, 5.15.1.x(new), 5.15.1.x.1(new) , 5.15.1.x.2(new) , 5.15.1.x.3(new) , 5.15.1.x.3.1(new) , 5.15.1.x.3.2(new) , 5.15.1.x.3.3(new) , 5.15.1.x.4(new) , 5.15.1.x.4.1(new) , 5.15.1.x.4.2(new) , 5.15.1.x.4.2.1(new) , 5.15.1.x.4.2.2(new) , 5.15.1.y(new), 5.15.1.y.1(new), 5.15.1.y.2(new), 5.15.1.y.3(new), 5.15.1.y.3.1(new), 5.15.1.y.3.2(new), 5.15.1.y.3.3(new), 5.15.1.y.3.4(new), 5.15.4.2, 5.15.4.3.x1(new), 5.15.4.3.x2(new), 5.15.4.3.x3(new), 5.15.4.3.x4(new), |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR does not impact the OpenAPI file. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

# Proposed changes:

# *** 1st Change ***

5.15.1.1	Overview

All resource URIs of this API should have the following root:

- {apiRoot}/3gpp-time-sync/v1

"apiRoot" is set as described in subclause 5.2.4 in 3GPP TS 29.122 [4]. "apiName" shall be set to "3gpp-time-sync" and "apiVersion" shall be set to "v1" for the current version defined in the present document. All resource URIs in the subclauses below are defined relative to the above root URI.

This subclause describes the structure for the Resource URIs as shown in figure 5.15.1.1-1 and the resources and HTTP methods used for the TimeSyncExposure API.

Figure 5.15.1.1-1: Resource URI structure of the TimeSyncExposure API

Table 5.15.1.1-1 provides an overview of the resources and HTTP methods applicable for the TimeSyncExposure API.

Table 5.15.1.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
| Time Synchronization Exposure Subscriptions | /{afId}/subscriptions | GET | Read all subscriptions for a given AF |
|  |  | POST | Create a new subscription to time synchronization exposure |
| Individual Time Synchronization Exposure Subscription | /{afId}/subscriptions/{subscriptionId} | GET | Read a subscription to time synchronization exposure |
|  |  | PUT | Modify all of the properties of an existing subscription to time synchronization exposure |
|  |  | DELETE | Delete a subscription to time synchronization exposure |
| Time Synchronization Exposure Configurations | /{afId}/subscriptions/{subscriptionId}/configurations | GET | Read all configurations for a given AF and subscription |
|  |  | POST | Create a new configuration to time synchronization exposure |
| Individual Time Synchronization Exposure Configuration | /{afId}/subscriptions/{subscriptionId}/configurations/{instanceReference} | GET | Read a configuration to time synchronization exposure |
|  |  | PUT | Modify all of the properties of an existing configuration to time synchronization exposure |
|  |  | DELETE | Delete a configuration to time synchronization exposure |
| ASTI Configurations | /{afId}/asti-configurations | GET | Read all configurations of 5G access stratum time distribution for a given AF and subscription |
|  |  | POST | Create a new configuration of 5G access stratum time distribution. |
|  |  | retrieve (POST) | Retrieval of the status of access stratum time distribution for a list of UEs. |
| Individual ASTI Configuration | /{afId}/asti-configurations/{astiConfigId} | PUT | Modify a new configuration of 5G access stratum time distribution. |
|  |  | DELETE | DELETE a new configuration of 5G access stratum time distribution. |

# *** NextChange ***

5.15.1.x	Resource: ASTI Configurations

5.15.1.x.1	Introduction

This resource allows an AF to read all active configurations of 5G access stratum time distribution for a given AF, or allows an AF to create a new new configuration of 5G access stratum time distribution.

5.15.1.x.2	Resource Definition

Resource URI: {apiRoot}/3gpp-time-sync/v1/{afId}/asti-configurations

This resource shall support the resource URI variables defined in table 5.15.1.x.2-1.

Table 5.15.1.x.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | Subclause 5.2.4 of 3GPP TS 29.122 [4]. |
| afId | string | Identifier of the AF. |

5.15.1.x.3	Resource Methods

5.15.1.x.3.1	General

The following subclauses specify the resource methods supported by the resource as described in subclause 5.15.1.x.3.

5.15.1.x.3.2	GET

The GET method allows to read all active configurations of 5G access stratum time distribution for a given AF and subscription. The AF shall initiate the HTTP GET request message and the NEF shall respond to the message.

This method shall support the URI query parameters specified in table 5.15.1.x.3.2-1.

Table 5.15.1.x.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  |  |  |

This method shall support the request data structures specified in table 5.15.1.x.3.2-2 and the response data structures and response codes specified in table 5.15.1.x.3.2-3.

Table 5.15.1.x.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N/A |  |  |  |

Table 5.15.1.x.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(AccessTimeDistributionData) | M | 0..N | 200 OK | The configuration information for the AF in the request URI are returned. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection, during configuration retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in subclause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection, during configuration retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in subclause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.15.1.x.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.15.1.x.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

5.15.1.x.3.3	POST

The POST method creates a new configuration of 5G access stratum time distribution for a given AF. The AF shall initiate the HTTP POST request message and the NEF shall respond to the message. The NEF shall construct the URI of the created resource.

This method shall support the request data structures specified in table 5.15.1.x.3.3-1 and the response data structures and response codes specified in table 5.15.1.x.3.3-2.

Table 5.15.1.x.3.3-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AccessTimeDistributionData | M | 1 | Parameters to create a configuration of 5G access stratum time distribution. |

Table 5.15.1.2.4.3-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AccessTimeDistributionData | M | 1 | 201 Created | The configuration was created successfully. The URI of the created resource shall be returned in the "Location" HTTP header. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.15.1.x.3.3-3: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/3gpp-time-sync/v1/{afId}/subscriptions/{subscriptionId}/asti-configurations/{astiConfigId} |

5.15.1.x.4	Resource Custom Operations

5.15.1.x.4.1	Overview

Table 5.15.1.x.4.1-1: Custom operations

| Operation Name | Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| retrieve | /asti-configurations/retrieve | retrieve (POST) | Request the status of the 5G access stratum time distribution for a list of UEs. |

5.15.1.x.4.2	Operation: retrieve

5.15.1.x.4.2.1	Description

This custom operation retrieves the status of the access stratum time distribution for a list of UEs.

5.15.1.x.4.2.2	Operation Definition

This operation shall support the request data structures specified in table 5.15.1.x.4.2.2-1 and the response data structure and response codes specified in table 5.15.1.x.4.2.2-2.

Table 5.15.1.x.4.2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| StatusRequestData | M | 1 | Parameters to be sent by the NF service consumer when the status of the 5G access stratum time distribution for a list of UEs is requested. |

Table 5.15.1.x.4.2.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| StatusResponseData | M | 1 | 200 OK | Status of the 5G access stratum time distribution for a list of UEs is returned. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] shall also apply. |  |  |  |  |

# *** NextChange ***

5.15.1.y	Resource: Individual ASTI Configuration

5.15.1.y.1	Introduction

This resource allows an AF to read/modify/cancel a configuration of 5G access stratum time distribution with the NEF.

5.15.1.y.2	Resource Definition

Resource URI: {apiRoot}/3gpp-time-sync/v1/{afId}/asit-configurations/{astiConfigId}

This resource shall support the resource URI variables defined in table 5.15.1.y.2-1.

Table 5.15.1.y.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | Subclause 5.2.4 of 3GPP TS 29.122 [4]. |
| afId | string | Identifier of the AF. |
| astiConfigId | string | Identifier of the configuration resource |

5.15.1.y.3	Resource Methods

5.15.1.y.3.1	General

The following subclauses specify the resource methods supported by the resource as described in subclause 5.15.1.y.2.

5.15.1.y.3.2	GET

The GET method allows to read the active configuration for a given AF, subscription Id and configuration Id. The AF shall initiate the HTTP GET request message and the NEF shall respond to the message.

This method shall support the URI query parameters specified in table 5.15.1.y.3.2-1.

Table 5.15.1.y.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  |  |  |

This method shall support the request data structures specified in table 5.15.1.y.3.2-2 and the response data structures and response codes specified in table 5.15.1.y.3.2-3.

Table 5.15.1.y.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N/A |  |  |  |

Table 5.15.1.y.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AccessTimeDistributionData | M | 1 | 200 OK | The configuration information for the AF in the request URI are returned. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection, during configuration retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in subclause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection, during configuration retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in subclause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.15.1.y.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.15.1.y.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

5.15.1.y.3.3	PUT

The PUT method modifies an existing configuration resource to update a configuration. The AF shall initiate the HTTP PUT request message and the NEF shall respond to the message.

This method shall support the request data structures specified in table 5.15.1.y.3.3-1 and the response data structures and response codes specified in table 5.15.1.y.3.3-2.

Table 5.15.1.y.3.3-1: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AccessTimeDistributionData | M | 1 | Modify an existing configuration of 5G access stratum time distribution. |

Table 5.15.1.y.3.3-2: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AccessTimeDistributionData | M | 1 | 200 OK | The configuration was updated successfully. |
| N/A |  |  | 204 No Content | The configuration was updated successfully. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection, during configuration modification. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in subclause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection, during configuration modification. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in subclause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the PUT method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.15.1.y.3.3-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.15.1.y.3.3-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

5.15.1.y.3.4	DELETE

The DELETE method deletes the configuration of 5G access stratum time distribution for a given AF. The AF shall initiate the HTTP DELETE request message and the NEF shall respond to the message.

This method shall support the URI query parameters specified in table 5.15.1.y.3.4-1.

Table 5.15.1.y.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  |  |  |

This method shall support the request data structures specified in table 5.15.1.y.3.4-2 and the response data structures and response codes specified in table 5.15.1.y.3.4-3.

Table 5.15.1.y.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N/A |  |  |  |

Table 5.15.1.y.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| N/A |  |  | 204 No Content | The configuration was terminated successfully. |
| N/A |  |  | 307 Temporary Redirect | Temporary redirection, during configuration termination. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in subclause 5.2.10 of 3GPP TS 29.122 [4]. |
| N/A |  |  | 308 Permanent Redirect | Permanent redirection, during configuration termination. The response shall include a Location header field containing an alternative URI of the resource located in an alternative NEF. Redirection handling is described in subclause 5.2.10 of 3GPP TS 29.122 [4]. |
| NOTE: The mandatory HTTP error status codes for the DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.15.1.y.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

Table 5.15.1.y.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative NEF. |

# *** NextChange ***

5.15.4.2	Reused data types

The data types reused by the TimeSyncExposure API from other specifications are listed in table 5.15.4.2-1.

Table 5.15.4.2-1: Re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| AsTimeDistributionParam | 3GPP TS 29.565 [50] | Contains the 5G access stratum time distribution parameters. |
| Datetime | 3GPP TS 29.122 [4] |  |
| Dnn | 3GPP TS 29.571 [8] | Identifies a DNN. |
| ExternalGroupId | 3GPP TS 29.122 [4] | External Group Identifier for a user group. |
| NotificationMethod | 3GPP TS 29.508 [26] |  |
| Gpsi | 3GPP TS 29.571 [8] | Identifies a GPSI. |
| Snssai | 3GPP TS 29.571 [8] | Identifies the S-NSSAI. |
| Supi | 3GPP TS 29.571 [8] | Identifies a SUPI. |
| SupportedFeatures | 3GPP TS 29.571 [8] | Used to negotiate the applicability of the optional features defined in table 5.15.5-1. |
| Uinteger | 3GPP TS 29.571 [8] | Unsigned integer. |
| Uri | 3GPP TS 29.571 [8] | Identifies a referenced resource. |

# *** NextChange ***

5.15.4.3.x1	Type: AccessTimeDistributionData

Table 5.15.4.3.x1-1: Definition of type AccessTimeDistributionData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| gpsis | array(Gpsi) | C | 1..N | Identifies a list of UE(s). (NOTE 1) |  |
| exterGroupId | ExternalGroupId | C | 0.1 | Represents a group of users. (NOTE 1) |  |
| anyUeInd | boolean | C | 0..1 | Identifies whether the AF request applies to any UE (i.e. all UEs). This attribute shall set to "true" if applicable for any UE, otherwise, set to "false". (NOTE 1) |  |
| dnn | Dnn | C | 0..1 | Identifies a DNN, a full DNN with both the Network Identifier and Operator Identifier, or a DNN with the Network Identifier only. (NOTE 2) |  |
| snssai | Snssai | C | 0..1 | Identifies an S-NSSAI. (NOTE 2) |  |
| asTimeDisParam | AsTimeDistributionParam | M | 1 | 5G access stratum time distribution parameters |  |
| suppFeat | SupportedFeatures | C | 0..1 | Represents the features supported by the NF service consumer. This parameter shall be supplied by the NF service consumer in the POST request and the response that requested the creation of an Individual ASTI Configuration resource. |  |
| NOTE 1: One of "gpsis", "externalGroupId" or "anyUeInd" attribute shall be provided. NOTE 2: Any of it shall be presented if "anyUeInd" attribute is presented. |  |  |  |  |  |

# *** NextChange ***

5.15.4.3.x2	Type: StatusRequestData

Table 5.15.4.3.x2-1: Definition of type StatusRequestData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| gpsis | array(Gpsi) | M | 1..N | Identifies a list of UE(s). |  |
| dnn | Dnn | O | 0..1 | Identifies a DNN, a full DNN with both the Network Identifier and Operator Identifier, or a DNN with the Network Identifier only. |  |
| snssai | Snssai | O | 0..1 | Identifies an S-NSSAI. |  |

# *** NextChange ***

5.15.4.3.x3	Type: StatusResponseData

Table 5.15.4.3.x3-1: Definition of type StatusResponseData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| inactiveUes | array(Gpsi) | O | 1..N | Indicate the UE(s) whose status of the access stratum time distribution is inactive. |  |
| activeUes | array(ActiveUe) | O | 1..N | Contains the UE identifier(s) whose status of the access stratum time distribution is active and the optional requested time synchronization error budget. |  |

# *** NextChange ***

5.15.4.3.x4	Type: ActiveUe

Table 5.15.4.3.x4-1: Definition of type ActiveUe

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| gpsi | Gpsi | M | 1 | Indicate the UE whose status of the access stratum time distribution is active. |  |
| timeSyncErrBudget | Uinteger | O | 0..1 | Indicates the time synchronization error budget in terms of time units of microseconds. |  |

# *** End of Changes ***