TSG-CT WG3 Meeting #119bis-e	C3-220170

# E-Meeting, 17th – 21st January 2022

Source:	Huawei

Title:	Support of time synchronization error budget

Spec:	3GPP TS 29.565 v1.0.0

Agenda item:	17.16

Document for:	Decision

1. Introduction

<Introduction part (optional)>

2. Reason for Change

As defined in Table 4.15.9.3-1 of 23.502, the AF may provide the Time synchronization error budget when it activates the time synchronization service.

3. Conclusions

Add “timeSyncErrBdgt” attribute in the TimeSyncExposureConfig

4. Proposal

It is proposed to agree the following changes to 3GPP TS 29.565.

* * * First Change * * * *

5.2.2.5.2	Creating a new configuration

Figure 5.2.2.5.2-1 illustrates the creation of a configuration.

Figure 5.2.2.5.2-1: Creation of a configuration

To create a configuration, the NF service consumer shall send an HTTP POST message to the TSCTSF to the URI "{apiRoot}/ntsctsf-time-sync/<apiVersion>/subscriptions/{subscriptionId}/configurations". The HTTP POST message shall include the TimeSyncExposureConfig data structure as request body, as shown in figure 5.2.2.5.2-1, step 1. The TimeSyncExposureConfig data structure shall include:

-	the user plane node Id within the "upNodeId" attribute;

-	the requested PTP instance within the "reqPtpIns" attribute;

-	the time domian within the "timeDom" attribute;

-	the notification URI within the "configNotifUri" attribute;

-	the notification correlation Id within the "configNotifId" attribute;

and may include:

- the "gmEnable" attribute set to true if the AF requests 5GS to act as a grandmaster for PTP or gPTP;

- the time synchronization error budget within the "timeSyncErrBdgt" attribute;

- the gandmaster priority with the "gmPrio" attribute; and

- the temporal validity condition within the "tempValidity" attribute.

Upon receipt of the HTTP request from the NF service consumer, if the request is authorized, the TSCTSF shall:

-	create a new resource, which represents a new "Individual Time Synchronization Exposure Configuration" instance, addressed by a URI as defined in subclause 6.1.3.5 and containing a TSCTSF created resource identifier;

-	send an HTTP "201 Created" response with TimeSyncExposureConfig data structure as response body and a Location header field containing the URI of the created Individual Time Synchronization Exposure Configuration resource, i.e. "{apiRoot}/ntsctsf-time-sync/<apiVersion>/subscriptions/{subcriptionId}/configuration/{configurationId}", as shown in figure 5.2.2.5.2-1, step 2;

-	use the {subscriptionId} within the requested URI and user plane node ID within the "upNodeId" attribute in the request to determine the target UEs and corresponding AF-sessions, and then contact with the PCF(s) to configure and initialize the PTP instance in the DS-TT(s) and NW-TT as defined in 3GPP TS 23.502 [3], clause 4.15.9.3.2, step 5-6;

Editor's Note:	Error responses are FFS.

Editor's Note:	The handling of the time synchronization error budget is FFS.

* * * Next Change * * * *

5.2.2.6.2	Updating an existing configuration

Figure 5.2.2.6.2-1 illustrates the updating of an existing configuration.

Figure 5.2.2.6.2-1: Update of a configuration

To update a configuration, the NF service consumer shall send an HTTP PUT request to the resource "{apiRoot}/ntsctsf-time-sync/<apiVersion>/subscriptions/{subscriptionId}/configurations/{configurationId" representing an existing "Individual Time Synchronization Exposure Configuration" resource, as shown in figure 5.2.2.6.2-1, step 1, to modify the configuration.

The TimeSyncExposureConfig data structure provided in the request body shall include:

-	the user plane node Id within the "upNodeId" attribute;

NOTE 1:		The user plane node Id cannot be changed during the modification.

-	the requested PTP instance within the "reqPtpIns" attribute;

-	the time domain within the "timeDom" attribute;

NOTE 2:	The user plane node Id, the requested PTP instance and the time domain cannot be changed during the modification.

-	the notification URI within the "configNotifUri" attribute;

-	the notification correlation Id within the "configNotifId" attribute;

NOTE 2:		If the notification URI or notification correlation Id is not changed the previously value is included.

and may include:

- the "gmEnable" attribute set to true if the AF requests 5GS to act as a grandmaster for PTP or gPTP;

- the time synchronization error budget within the "timeSyncErrBdgt" attribute;

- the gandmaster priority with the "gmPrio" attribute; and

- the temporal validity condition within the "tempValidity" attribute.

Upon receipt of the corresponding HTTP PUT message, if the request is authorized, theTSCTSF shall:

-	update the existing "Individual Time Synchronization Exposure Configuration" resource;

-	send a HTTP response including "200 OK" status code with TimeSyncExposureConfig data structure or "204 No Content" status code, as shown in figure 5.2.2.6.2-1, step 2;

-	use the {subscriptionId} within the requested URI and user plane node ID within the "upNodeId" attribute in the request to determine the target UEs and corresponding AF-sessions, and then contact with the PCF(s) to configure and initialize the PTP instance in the DS-TT(s) and NW-TT as defined in 3GPP TS 23.502 [3], clause 4.15.9.3.2, step 5-6.

Editor's Note:	Error/Redirect responses are FFS.

Editor's Note:	The handling of the time synchronization error budget is FFS.

* * * End of Changes * * * *