3GPP TSG-CT3 Meeting #120-e	C3-221661

# E-Meeting, 15th – 25th February 2022                                                    (revision of C3-22xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.222 | CR | 0227 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of info and externalDocs fields |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2022-02-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | CRs modifying the CAPIF APIs have been agreed and the version number of the corresponding OpenAPI file thus needs to be incremented following the rules in TS 29.501, subclause 4.3.1. The following are the agreed CRs the update the various CAPIF OpenAPI files for the present release CAPIF_Publish_Service_API - TS 29.222, CR#0221: Backward compatible correction - TS 29.222, CR#0226: Backward compatible feature CAPIF_API_Invoker_Management_API - TS 29.222, CR#0225: Backward compatible feature CAPIF_Security_API - TS 29.222, CR#0222: Backward compatible feature CAPIF_API_Provider_Management_API - TS 29.222, CR#0224: Backward compatible feature As the present release is not frozen, the alpha part of the API version needs to be incremented, and the TS version in the externalDocs field to 17.4.0. In addition, the description field covering TS information is updated to cover proper line breaks structure, and the URL of the External Docs field is updated to https. |  |  |  |  |  |  |  |  |
| Summary of change: |  | CAPIF_Publish_Service_API - Update API version from 1.2.0-alpha.3 to 1.2.0-alpha.4 - Copyright year to 2022 - Update of the line breaks in the description field of info field - Update externalDocs to 17.4.0 - Update http URL to https for externalDocs field CAPIF_API_Invoker_Management_API - Update API version from 1.2.0-alpha.3 to 1.2.0-alpha.4 - Copyright year to 2022 - Update of the line breaks in the description field of info field - Update externalDocs to 17.4.0 - Update http URL to https for externalDocs field CAPIF_Security_API - Update API version from 1.2.0-alpha.1 to 1.2.0-alpha.2 - Copyright year to 2022 - Update of the line breaks in the description field of info field - Update externalDocs to 17.4.0 - Update http URL to https for externalDocs field CAPIF_API_Provider_Management_API - Update API version from 1.1.0-alpha.2 to 1.1.0-alpha.3 - Copyright year to 2022 - Update of the line breaks in the description field of info field - Update externalDocs to 17.4.0 - Update http URL to https for externalDocs field |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect API version number and TS version in externalDocs field. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3, A.5, A.6, A.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## A.3	CAPIF_Publish_Service_API

openapi: 3.0.0

info:

title: CAPIF_Publish_Service_API

description: |

API for publishing service APIs.

© 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

version: "1.2.0-alpha.4"

externalDocs:

description: 3GPP TS 29.222 V17.4.0 Common API Framework for 3GPP Northbound APIs

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.222/

servers:

- url: '{apiRoot}/published-apis/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in subclause 7.5 of 3GPP TS 29.222.

paths:

# APF published API

/{apfId}/service-apis:

post:

description: Publish a new API.

parameters:

- name: apfId

in: path

required: true

schema:

$ref: '#/components/schemas/apfId'

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceAPIDescription'

responses:

'201':

description: Service API published successfully The URI of the created resource shall be returned in the "Location" HTTP header.

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceAPIDescription'

headers:

Location:

description: 'Contains the URI of the newly created resource, according to the structure: {apiRoot}/published-apis/v1/{apfId}/service-apis/{serviceApiId}'

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

get:

description: Retrieve all published APIs.

parameters:

- name: apfId

in: path

required: true

schema:

$ref: '#/components/schemas/apfId'

responses:

'200':

description: Definition of all service API(s) published by the API publishing function.

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceAPIDescription'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

# Individual APF published API

/{apfId}/service-apis/{serviceApiId}:

get:

description: Retrieve a published service API.

parameters:

- name: serviceApiId

in: path

required: true

schema:

$ref: '#/components/schemas/serviceApiId'

- name: apfId

in: path

required: true

schema:

$ref: '#/components/schemas/apfId'

responses:

'200':

description: Definition of all service API published by the API publishing function.

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceAPIDescription'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

description: Update a published service API.

parameters:

- name: serviceApiId

in: path

required: true

schema:

$ref: '#/components/schemas/serviceApiId'

- name: apfId

in: path

required: true

schema:

$ref: '#/components/schemas/apfId'

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceAPIDescription'

responses:

'200':

description: Definition of service API updated successfully.

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceAPIDescription'

'204':

description: No Content

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

description: Unpublish a published service API.

parameters:

- name: serviceApiId

in: path

required: true

schema:

$ref: '#/components/schemas/serviceApiId'

- name: apfId

in: path

required: true

schema:

$ref: '#/components/schemas/apfId'

responses:

'204':

description: The individual published service API matching the serviceAPiId is deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

# Components

components:

schemas:

# Data types uses as path variables

apfId:

type: string

description: Identification of the API publishing function.

serviceApiId:

type: string

description: String identifying an individual published service API.

# Data Type for representations

ServiceAPIDescription:

type: object

description: Represents the description of a service API as published by the APF.

properties:

apiName:

type: string

description: API name, it is set as {apiName} part of the URI structure as defined in subclause 4.4 of 3GPP TS 29.501.

apiId:

type: string

description: API identifier assigned by the CAPIF core function to the published service API. Shall not be present in the HTTP POST request from the API publishing function to the CAPIF core function. Shall be present in the HTTP POST response from the CAPIF core function to the API publishing function and in the HTTP GET response from the CAPIF core function to the API invoker (discovery API).

aefProfiles:

type: array

items:

$ref: '#/components/schemas/AefProfile'

minItems: 1

description: AEF profile information, which includes the exposed API details (e.g. protocol).

description:

type: string

description: Text description of the API

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

shareableInfo:

$ref: '#/components/schemas/ShareableInformation'

serviceAPICategory:

type: string

apiSuppFeats:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

pubApiPath:

$ref: '#/components/schemas/PublishedApiPath'

ccfId:

type: string

description: CAPIF core function identifier.

required:

- apiName

InterfaceDescription:

type: object

description: Represents the description of an API's interface.

properties:

ipv4Addr:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Ipv4Addr'

ipv6Addr:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Ipv6Addr'

port:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Port'

securityMethods:

type: array

items:

$ref: '#/components/schemas/SecurityMethod'

minItems: 1

description: Security methods supported by the interface, it take precedence over the security methods provided in AefProfile, for this specific interface.

oneOf:

- required: [ipv4Addr]

- required: [ipv6Addr]

AefProfile:

type: object

description: Represents the AEF profile data.

properties:

aefId:

type: string

description: Identifier of the API exposing function

versions:

type: array

items:

$ref: '#/components/schemas/Version'

minItems: 1

description: API version

protocol:

$ref: '#/components/schemas/Protocol'

dataFormat:

$ref: '#/components/schemas/DataFormat'

securityMethods:

type: array

items:

$ref: '#/components/schemas/SecurityMethod'

minItems: 1

description: Security methods supported by the AEF

domainName:

type: string

description: Domain to which API belongs to

interfaceDescriptions:

type: array

items:

$ref: '#/components/schemas/InterfaceDescription'

minItems: 1

description: Interface details

aefLocation:

$ref: '#/components/schemas/AefLocation'

required:

- aefId

- versions

oneOf:

- required: [domainName]

- required: [interfaceDescriptions]

Resource:

type: object

description: Represents the API resource data.

properties:

resourceName:

type: string

description: Resource name

commType:

$ref: '#/components/schemas/CommunicationType'

uri:

type: string

description: Relative URI of the API resource, it is set as {apiSpecificResourceUriPart} part of the URI structure as defined in subclause 4.4 of 3GPP TS 29.501.

custOpName:

type: string

description: it is set as {custOpName} part of the URI structure for a custom operation associated with a resource as defined in subclause 4.4 of 3GPP TS 29.501.

operations:

type: array

items:

$ref: '#/components/schemas/Operation'

minItems: 1

description: Supported HTTP methods for the API resource. Only applicable when the protocol in AefProfile indicates HTTP.

description:

type: string

description: Text description of the API resource

required:

- resourceName

- commType

- uri

CustomOperation:

type: object

description: Represents the description of a custom operation.

properties:

commType:

$ref: '#/components/schemas/CommunicationType'

custOpName:

type: string

description: it is set as {custOpName} part of the URI structure for a custom operation without resource association as defined in subclause 4.4 of 3GPP TS 29.501.

operations:

type: array

items:

$ref: '#/components/schemas/Operation'

minItems: 1

description: Supported HTTP methods for the API resource. Only applicable when the protocol in AefProfile indicates HTTP.

description:

type: string

description: Text description of the custom operation

required:

- commType

- custOpName

Version:

type: object

description: Represents the API version information.

properties:

apiVersion:

type: string

description: API major version in URI (e.g. v1)

expiry:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

resources:

type: array

items:

$ref: '#/components/schemas/Resource'

minItems: 1

description: Resources supported by the API.

custOperations:

type: array

items:

$ref: '#/components/schemas/CustomOperation'

minItems: 1

description: Custom operations without resource association.

required:

- apiVersion

ShareableInformation:

type: object

description: Indicates whether the service API and/or the service API category can be shared to the list of CAPIF provider domains.

properties:

isShareable:

type: boolean

description: Set to "true" indicates that the service API and/or the service API category can be shared to the list of CAPIF provider domain information. Otherwise set to "false".

capifProvDoms:

type: array

items:

type: string

minItems: 1

description: List of CAPIF provider domains to which the service API information to be shared.

required:

- isShareable

PublishedApiPath:

type: object

description: Represents the published API path within the same CAPIF provider domain.

properties:

ccfIds:

type: array

items:

type: string

minItems: 1

description: A list of CCF identifiers where the service API is already published.

AefLocation:

description: The location information (e.g. civic address, GPS coordinates, data center ID) where the AEF providing the service API is located.

type: object

properties:

civicAddr:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/CivicAddress'

geoArea:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/GeographicArea'

dcId:

type: string

description: Identifies the data center where the AEF providing the service API is located.

Protocol:

anyOf:

- type: string

enum:

- HTTP_1_1

- HTTP_2

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: >

Possible values are

- HTTP_1_1: HTTP version 1.1

- HTTP_2: HTTP version 2

CommunicationType:

anyOf:

- type: string

enum:

- REQUEST_RESPONSE

- SUBSCRIBE_NOTIFY

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: >

Possible values are

- REQUEST_RESPONSE: The communication is of the type request-response

- SUBSCRIBE_NOTIFY: The communication is of the type subscribe-notify

DataFormat:

anyOf:

- type: string

enum:

- JSON

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: >

Possible values are

- JSON: JavaScript Object Notation

SecurityMethod:

anyOf:

- type: string

enum:

- PSK

- PKI

- OAUTH

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: >

Possible values are

- PSK: Security method 1 (Using TLS-PSK) as described in 3GPP TS 33.122

- PKI: Security method 2 (Using PKI) as described in 3GPP TS 33.122

- OAUTH: Security method 3 (TLS with OAuth token) as described in 3GPP TS 33.122

Operation:

anyOf:

- type: string

enum:

- GET

- POST

- PUT

- PATCH

- DELETE

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: >

Possible values are

- GET: HTTP GET method

- POST: HTTP POST method

- PUT: HTTP PUT method

- PATCH: HTTP PATCH method

- DELETE: HTTP DELETE method

* * * Next Change * * * *

## A.5	CAPIF_API_Invoker_Management_API

openapi: 3.0.0

info:

title: CAPIF_API_Invoker_Management_API

description: |

API for API invoker management.

© 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

version: "1.2.0-alpha.4"

externalDocs:

description: 3GPP TS 29.222 V17.4.0 Common API Framework for 3GPP Northbound APIs

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.222/

servers:

- url: '{apiRoot}/api-invoker-management/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in subclause 7.5 of 3GPP TS 29.222

paths:

/onboardedInvokers:

post:

description: Creates a new individual API Invoker profile.

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/APIInvokerEnrolmentDetails'

callbacks:

notificationDestination:

'{request.body#/notificationDestination}':

post:

description: Notify the API Invoker about the onboarding completion

requestBody:  # contents of the callback message

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/OnboardingNotification'

responses:

'204':

description: No Content (successful onboarding notification)

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

responses:

'201':

description: API invoker on-boarded successfully.

content:

application/json:

schema:

$ref: '#/components/schemas/APIInvokerEnrolmentDetails'

headers:

Location:

description: 'Contains the URI of the newly created resource, according to the structure: {apiRoot}/api-invoker-management/v1/onboardedInvokers/{onboardingId}'

required: true

schema:

type: string

'202':

description: The CAPIF core has accepted the Onboarding request and is processing it.

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/onboardedInvokers/{onboardingId}:

delete:

description: Deletes an individual API Invoker.

parameters:

- name: onboardingId

in: path

description: String identifying an individual on-boarded API invoker resource

required: true

schema:

type: string

responses:

'204':

description: The individual API Invoker matching onboardingId was offboarded.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

description: Updates an individual API invoker details.

parameters:

- name: onboardingId

in: path

description: String identifying an individual on-boarded API invoker resource

required: true

schema:

type: string

requestBody:

description: representation of the API invoker details to be updated in CAPIF core function

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/APIInvokerEnrolmentDetails'

callbacks:

notificationDestination:

'{request.body#/notificationDestination}':

post:

description: Notify the API Invoker about the API invoker update completion

requestBody:  # contents of the callback message

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/OnboardingNotification'

responses:

'204':

description: No Content (successful API invoker update notification)

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

responses:

'200':

description: API invoker details updated successfully.

content:

application/json:

schema:

$ref: '#/components/schemas/APIInvokerEnrolmentDetails'

'202':

description: The CAPIF core has accepted the API invoker update details request and is processing it.

'204':

description: API invoker’s information updated successfully, with no content to be sent in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

schemas:

OnboardingInformation:

type: object

description: Represents on-boarding information of the API invoker.

properties:

apiInvokerPublicKey:

type: string

description: The API Invoker’s public key

apiInvokerCertificate:

type: string

description: The API Invoker’s generic client certificate, provided by the CAPIF core function.

onboardingSecret:

type: string

description: The API Invoker’s onboarding secret, provided by the CAPIF core function.

required:

- apiInvokerPublicKey

APIList:

type: array

items:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/ServiceAPIDescription'

minItems: 1

description: The list of service APIs that the API Invoker is allowed to invoke

APIInvokerEnrolmentDetails:

type: object

properties:

apiInvokerId:

type: string

description: API invoker ID assigned by the CAPIF core function to the API invoker while on-boarding the API invoker. Shall not be present in the HTTP POST request from the API invoker to the CAPIF core function, to on-board itself. Shall be present in all other HTTP requests and responses.

readOnly: true

onboardingInformation:

$ref: '#/components/schemas/OnboardingInformation'

notificationDestination:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

requestTestNotification:

type: boolean

description: Set to true by Subscriber to request the CAPIF core function to send a test notification as defined in in subclause 7.6. Set to false or omitted otherwise.

websockNotifConfig:

$ref: 'TS29122_CommonData.yaml#/components/schemas/WebsockNotifConfig'

apiList:

$ref: '#/components/schemas/APIList'

apiInvokerInformation:

type: string

description: Generic information related to the API invoker such as details of the device or the application.

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- onboardingInformation

- notificationDestination

description: Information about the API Invoker that requested to onboard

OnboardingNotification:

type: object

description: Represents a notification of on-boarding or update result.

properties:

result:

type: boolean

description: Set to "true" indicate successful on-boarding. Otherwise set to "false"

resourceLocation:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

apiInvokerEnrolmentDetails:

$ref: '#/components/schemas/APIInvokerEnrolmentDetails'

apiList:

$ref: '#/components/schemas/APIList'

required:

- result

* * * Next Change * * * *

## A.6	CAPIF_Security_API

openapi: 3.0.0

info:

title: CAPIF_Security_API

description: |

API for CAPIF security management.

© 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

version: "1.2.0-alpha.2"

externalDocs:

description: 3GPP TS 29.222 V17.4.0 Common API Framework for 3GPP Northbound APIs

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.222/

servers:

- url: '{apiRoot}/capif-security/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in subclause 7.5 of 3GPP TS 29.222.

paths:

/trustedInvokers/{apiInvokerId}:

get:

parameters:

- name: apiInvokerId

in: path

description: Identifier of an individual API invoker

required: true

schema:

type: string

- name: authenticationInfo

in: query

description: When set to 'true', it indicates the CAPIF core function to send the authentication information of the API invoker. Set to false or omitted otherwise.

schema:

type: boolean

- name: authorizationInfo

in: query

description: When set to 'true', it indicates the CAPIF core function to send the authorization information of the API invoker. Set to false or omitted otherwise.

schema:

type: boolean

responses:

'200':

description: The security related information of the API Invoker based on the request from the API exposing function.

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceSecurity'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'414':

$ref: 'TS29122_CommonData.yaml#/components/responses/414'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

parameters:

- name: apiInvokerId

in: path

description: Identifier of an individual API invoker

required: true

schema:

type: string

requestBody:

description: create a security context for an API invoker

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceSecurity'

callbacks:

notificationDestination:

'{request.body#/notificationDestination}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/SecurityNotification'

responses:

'204':

description: No Content (successful notification)

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

responses:

'201':

description: Successful created.

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceSecurity'

headers:

Location:

description: 'Contains the URI of the newly created resource, according to the structure: {apiRoot}/capif-security/v1/trustedInvokers/{apiInvokerId}'

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'414':

$ref: 'TS29122_CommonData.yaml#/components/responses/414'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

parameters:

- name: apiInvokerId

in: path

description: Identifier of an individual API invoker

required: true

schema:

type: string

responses:

'204':

description: No Content (Successful deletion of the existing subscription)

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/trustedInvokers/{apiInvokerId}/update:

post:

parameters:

- name: apiInvokerId

in: path

description: Identifier of an individual API invoker

required: true

schema:

type: string

requestBody:

description: Update the security context (e.g. re-negotiate the security methods).

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceSecurity'

responses:

'200':

description: Successful updated.

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceSecurity'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/trustedInvokers/{apiInvokerId}/delete:

post:

parameters:

- name: apiInvokerId

in: path

description: Identifier of an individual API invoker

required: true

schema:

type: string

requestBody:

description: Revoke the authorization of the API invoker for APIs.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/SecurityNotification'

responses:

'204':

description: Successful revoked.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/securities/{securityId}/token:

post:

parameters:

- name: securityId

in: path

description: Identifier of an individual API invoker

required: true

schema:

type: string

requestBody:

required: true

content:

application/x-www-form-urlencoded:

schema:

$ref: '#/components/schemas/AccessTokenReq'

responses:

'200':

description: Successful Access Token Request

content:

application/json:

schema:

$ref: '#/components/schemas/AccessTokenRsp'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

description: Error in the Access Token Request

content:

application/json:

schema:

$ref: '#/components/schemas/AccessTokenErr'

components:

schemas:

ServiceSecurity:

type: object

description: Represents the details of the security method for each service API interface. When included by the API invoker, it indicates the preferred method of security. When included by the CAPIF core function, it indicates the security method to be used for the service API interface.

properties:

securityInfo:

type: array

items:

$ref: '#/components/schemas/SecurityInformation'

minimum: 1

notificationDestination:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

requestTestNotification:

type: boolean

description: Set to true by API invoker to request the CAPIF core function to send a test notification as defined in in subclause 7.6. Set to false or omitted otherwise.

websockNotifConfig:

$ref: 'TS29122_CommonData.yaml#/components/schemas/WebsockNotifConfig'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- securityInfo

- notificationDestination

SecurityInformation:

type: object

description: Represents the interface details and the security method.

properties:

interfaceDetails:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/InterfaceDescription'

aefId:

type: string

description: Identifier of the API exposing function

prefSecurityMethods:

type: array

items:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/SecurityMethod'

minItems: 1

description: Security methods preferred by the API invoker for the API interface.

selSecurityMethod:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/SecurityMethod'

authenticationInfo:

type: string

description: Authentication related information

authorizationInfo:

type: string

description: Authorization related information

required:

- prefSecurityMethods

oneOf:

- required: [interfaceDetails]

- required: [aefId]

SecurityNotification:

type: object

description: Represents revoked authorization notification details.

properties:

apiInvokerId:

type: string

description: String identifying the API invoker assigned by the CAPIF core function.

aefId:

type: string

description: String identifying the AEF.

apiIds:

type: array

items:

type: string

minItems: 1

description: Identifier of the service API

cause:

$ref: '#/components/schemas/Cause'

required:

- apiInvokerId

- apiIds

- cause

AccessTokenReq:

format: x-www-form-urlencoded

description: Represents the access token request information.

properties:

grant_type:

type: string

enum:

- client_credentials

client_id:

type: string

client_secret:

type: string

scope:

type: string

required:

- grant_type

- client_id

AccessTokenRsp:

type: object

description: Represents the access token response information.

properties:

access_token:

type: string

description: JWS Compact Serialized representation of JWS signed JSON object (AccessTokenClaims)

token_type:

type: string

enum:

- Bearer

expires_in:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DurationSec'

scope:

type: string

required:

- access_token

- token_type

- expires_in

AccessTokenClaims:

type: object

description: Represents the claims data structure for the access token.

properties:

iss:

type: string

scope:

type: string

exp:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DurationSec'

required:

- iss

- scope

- exp

AccessTokenErr:

type: object

description: Represents an error in the access token request.

properties:

error:

type: string

enum:

- invalid_request

- invalid_client

- invalid_grant

- unauthorized_client

- unsupported_grant_type

- invalid_scope

error_description:

type: string

error_uri:

type: string

required:

- error

Cause:

anyOf:

- type: string

enum:

- OVERLIMIT_USAGE

- UNEXPECTED_REASON

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: >

Possible values are

- OVERLIMIT_USAGE: The revocation of the authorization of the API invoker is due to the overlimit usage of the service API

- UNEXPECTED_REASON: The revocation of the authorization of the API invoker is due to unexpected reason.

* * * Next Change * * * *

## A.11	CAPIF_API_Provider_Management_API

openapi: 3.0.0

info:

title: CAPIF_API_Provider_Management_API

description: |

API for API provider domain functions management.

© 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

version: "1.1.0-alpha.3"

externalDocs:

description: 3GPP TS 29.222 V17.4.0 Common API Framework for 3GPP Northbound APIs

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.222/

servers:

- url: '{apiRoot}/api-provider-management/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in subclause 7.5 of 3GPP TS 29.222

paths:

/registrations:

post:

description: Registers a new API Provider domain with API provider domain functions profiles.

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/APIProviderEnrolmentDetails'

responses:

'201':

description: API provider domain registered successfully

content:

application/json:

schema:

$ref: '#/components/schemas/APIProviderEnrolmentDetails'

headers:

Location:

description: 'Contains the URI of the newly created resource, according to the structure: {apiRoot}/api-provider-management/v1/registrations/{registrationId}'

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/registrations/{registrationId}:

delete:

description: Deregisters API provider domain by deleting API provider domain and functions.

parameters:

- name: registrationId

in: path

description: String identifying an registered API provider domain resource.

required: true

schema:

type: string

responses:

'204':

description: The API provider domain matching registrationId is deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

description: Updates an API provider domain's registration details.

parameters:

- name: registrationId

in: path

description: String identifying an registered API provider domain resource.

required: true

schema:

type: string

requestBody:

description: Representation of the API provider domain registration details to be updated in CAPIF core function.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/APIProviderEnrolmentDetails'

responses:

'200':

description: API provider domain registration details updated successfully.

content:

application/json:

schema:

$ref: '#/components/schemas/APIProviderEnrolmentDetails'

'204':

description: No Content

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

schemas:

APIProviderEnrolmentDetails:

type: object

description: Represents an API provider domain's enrolment details.

properties:

apiProvDomId:

type: string

description: API provider domain ID assigned by the CAPIF core function to the API management function while registering the API provider domain. Shall not be present in the HTTP POST request from the API Management function to the CAPIF core function, to on-board itself. Shall be present in all other HTTP requests and responses.

readOnly: true

regSec:

type: string

description: Security information necessary for the CAPIF core function to validate the registration of the API provider domain. Shall be present in HTTP POST request from API management function to CAPIF core function for API provider domain registration.

apiProvFuncs:

type: array

items:

$ref: '#/components/schemas/APIProviderFunctionDetails'

minItems: 1

description: A list of individual API provider domain functions details. When included by the API management function in the HTTP request message, it lists the API provider domain functions that the API management function intends to register/update in registration or update registration procedure. When included by the CAPIF core function in the HTTP response message, it lists the API domain functions details that are registered or updated successfully.

apiProvDomInfo:

type: string

description: Generic information related to the API provider domain such as details of the API provider applications.

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

failReason:

type: string

description: Registration or update specific failure information of failed API provider domain function registrations.Shall be present in the HTTP response body if atleast one of the API provider domain function registration or update registration fails.

required:

- regSec

APIProviderFunctionDetails:

type: object

description: Represents API provider domain function's details.

properties:

apiProvFuncId:

type: string

description: API provider domain functionID assigned by the CAPIF core function to the API provider domain function while registering/updating the API provider domain. Shall not be present in the HTTP POST request from the API management function to the CAPIF core function, to register itself. Shall be present in all other HTTP requests and responses.

regInfo:

$ref: '#/components/schemas/RegistrationInformation'

apiProvFuncRole:

$ref: '#/components/schemas/ApiProviderFuncRole'

apiProvFuncInfo:

type: string

description: Generic information related to the API provider domain function such as details of the API provider applications.

required:

- regInfo

- apiProvFuncRole

RegistrationInformation:

type: object

description: Represents registration information of an individual API provider domain function.

properties:

apiProvPubKey:

type: string

description: Public Key of API Provider domain function.

apiProvCert:

type: string

description: API provider domain function's client certificate

required:

- apiProvPubKey

ApiProviderFuncRole:

anyOf:

- type: string

enum:

- AEF

- APF

- AMF

- type: string

description: >

This string provides forward-compatiblity with future extensions to the enumeration but is not used to encode content defined in the present version of this API.

description: >

Possible values are

- AEF: API provider function is API Exposing Function.

- APF: API provider function is API Publishing Function.

- AMF: API Provider function is API Management Function.

* * * End of Change * * * *