3GPP TSG CT WG3 Meeting #127e	C3-231147

# E-Meeting, 17th – 21st April 2023														(revision of C3-22xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.522 | CR | 0865 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update MBSGroupMsgDelivery API |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | KDDI, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS_Ph2 |  |  |  |  |  | Date: |  |  | 2023-04-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to Step 2 in clause 7.5.1 of TS23.247, NEF translates the MBS service area (geographical area information or civic address information) to Cell ID list or TAI list, if geographical area information or civic address information was provided by the AF as MBS service area. However, there is no conditional description for extMbsServArea" attribute and the "mbsServArea" attribute in TS29.522, The “UsageThreshold” data type for “groupMsgDelPayload” attribute is not defined in TS29.522. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR proposes to - Add conditional description for extMbsServArea" attribute and the "mbsServArea" attribute.s. - Add the “ExternalMbsServiceArea” data type to re-used Data Types - Add the editor's note to indicate the definition of “groupMsgDelPayload” attribute is FFS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The stage 2 requirement is not implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.29.7.2, 5.29.5.1, 5.29.5.2.3, 5.29.5.2.4, A.27 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces a backwards compatible feature in the OpenAPI file of the MBSGroupMsgDelivery API. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** 1st Changes ***

##### 4.4.29.7.2	Procedure for MBS Group Message Delivery Creation

This procedure is used by an AF to request MBS Group Message Delivery.

In order to request MBS Group Message Delivery, an AF shall send a Nnef_MBSGroupMsgDelivery_Create request message to the NEF using the HTTP POST method, with the Request body including the MbsGroupMsgDelReq data structure that shall contain:

-	within the "mbsGroupMsgDelData" attribute, the group message delivery data that shall contain:

-	within the "extMbsServArea" attribute or the "mbsServArea" attribute, the MBS service area;

-	within the "suppFeat" attribute, the list of supported features, if feature negotiation needs to take place.

The NEF shall then check whether the AF is authorized to perform this operation or not, as defined in clause 6.1.1 of 3GPP TS 23.247 [53]. If the AF is authorized, the NEF performs MBS User Service and MBS User Data Ingest Session provisioning towards the MBSF using the Object Distribution Method, as specified in 3GPP TS 26.502 [65] and 3GPP TS 29.580 [66]. If the AF provided the MBS service area in the form of geographical area(s) or civic address(es), the NEF may translate this information into a list of TAI(s) and/or a list of cell ID(s) before when sending the MBS service area information as part of MBS User Data Ingest Session provisioning towards the MBSF.

Upon reception of a successful response from the MBSF, as defined in 3GPP TS 29.580 [66], the NEF shall send a Nnef_MBSGroupMsgDelivery_Create response to the AF with an HTTP "201 Created" status code and the response body including the MbsGroupMsgDelResp data structure that shall contain:

-	within the "mbsGroupMsgDelData" attribute, the representation of the created "Individual MBS Group Message Delivery" resource;

-	within the "delStatus" attribute, the status of the Group Message Delivery; and

-	within the "suppFeat" attribute, the list of supported features, if feature negotiation needs to take place.

On failure or if the NEF receives an error response from the MBSF, the NEF shall take proper error handling actions, as specified in clause 5.29.7, and respond to the AF with an appropriate error status code. If the NEF received within an error response a "ProblemDetails" data structure with a "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error, when applicable.

# *** 2nd Changes ***

#### 5.29.5.1	General

This clause specifies the application data model supported by the MBSGroupMsgDelivery API. Table 5.29.5.1-1 specifies the data types defined for the MBSGroupMsgDelivery API.

Table 5.29.5.1-1: MBSGroupMsgDelivery specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| MbsGroupMsgDelReq | 5.29.5.2.2 | Represents the full set of parameters to initiate an MBS Group Message Delivery request. |  |
| MbsGroupMsgDelData | 5.29.5.2.3 | Represents MBS Group Message Delivery data. |  |
| MbsGroupMsgDelResp | 5.29.5.2.4 | Represents the response to an MBS Group Message Delivery request. |  |
| MbsGroupMsgDelPatch | 5.29.5.2.5 | Represents the parameters to request the modification of an existing MBS Group Message Delivery. |  |
| MbsGroupMsgDelStatusNotif | 5.29.5.2.6 | Represents a notification on the status of MBS Group Message Delivery. |  |

Table 5.29.5.1-2 specifies data types re-used by the MBSGroupMsgDelivery API from other specifications, including a reference to their respective specifications, and when needed, a short description of their use within the MBSGroupMsgDelivery API.

Table 5.29.5.1-2: MBSGroupMsgDelivery API re-used Data Types

| Data type | Reference |  | Comments |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- |
| ExternalMbsServiceArea |  | 3GPP TS 29.571 [8] |  | Represents an external MBS service area. |  |  |
| MbsServiceArea | 3GPP TS 29.571 [8] |  | Represents an MBS service area. |  |  |  |
| Tai | 3GPP TS 29.571 [8] |  | Represents a TAI. |  |  |  |
| DateTime | 3GPP TS 29.122 [4] |  | Represents a date and a time. |  |  |  |
| ExternalGroupId | 3GPP TS 29.122 [4] |  | Represents an external Group Identifier. |  |  |  |
| Uri | 3GPP TS 29.122 [4] |  | Contains a Uri. |  |  |  |

# *** 3rd Change ***

##### 5.29.5.2.3	Type: MbsGroupMsgDelData

Table 5.29.5.2.3-1: Definition of type MbsGroupMsgDelReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| extGroupId | ExternalGroupId | M | 1 | Identifies the targeted group of UEs. |  |
| groupMsgDelPayload | UsageThreshold | M | 1 | Contains the payload of the requested MBS Group Message Delivery. |  |
| mbsServArea | MbsServiceArea | M | 1 | Represents the MBS service area. (NOTE) |  |
| extMbsServArea | ExternalMbsServiceArea | M | 1 | Represents the external MBS service area. (NOTE) |  |
| startTime | DateTime | M | 1 | Represents the start time of the MBS Group Message Delivery. |  |
| stopTime | DateTime | M | 1 | Represents the end time of the MBS Group Message Delivery. |  |
| notificationUri | Uri | M | 1 | The notification URI via which the AF desires to receive notifications on the status of an MBS Group Message Delivery. |  |
| NOTE: These attributes are mutually exclusive. Either one of them may be present. |  |  |  |  |  |

Editor’s Note: The definition of "groupMsgDelPayload" attribute is FFS.

# *** 4th Change ***

##### 5.29.5.2.4	Type: MbsGroupMsgDelPatch

Table 5.29.5.2.4-1: Definition of type MbsGroupMsgDelPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| groupMsgDelPayload | UsageThreshold | O | 0..1 | Contains uri to the payload of the requested MBS Group Message Delivery. |  |
| mbsServArea | MbsServiceArea | O | 0..1 | Represents the MBS service area. (NOTE) |  |
| extMbsServArea | ExternalMbsServiceArea | O | 0..1 | Represents the external MBS service area. (NOTE) |  |
| startTime | DateTime | O | 0..1 | Represents the start time when the MBS Group Message Delivery starts. |  |
| stopTime | DateTime | O | 0..1 | Represents the stop time when the MBS Group Message Delivery stops. |  |
| notificationUri | Uri | O | 0..1 | The notification URI via which the AF desires to receive notifications on the status of an MBS Group Message Delivery. |  |
| NOTE: These attributes are mutually exclusive. Either one of them may be present. |  |  |  |  |  |

Editor’s Note: The definition of "groupMsgDelPayload" attribute is FFS.

# *** 5th Change ***

# A.27	MBSGroupMsgDelivery API

openapi: 3.0.0

info:

title: 3gpp-mbs-group-msg

version: 1.0.0-alpha.1

description: |

API for MBS Group Message Delivery.

© 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.522 V18.1.0; 5G System; Network Exposure Function Northbound APIs.

url: 'https://www.3gpp.org/ftp/Specs/archive/29_series/29.522/'

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/3gpp-mbs-group-msg/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 5.2.4 of 3GPP TS 29.122.

paths:

/deliveries:

get:

summary: Retrieve all the active MBS Group Message Deliveries.

operationId: GetMbsGroupMsgDeliveries

tags:

- MBS Group Message Deliveries

responses:

'200':

description: >

OK. All the active MBS Group Message Deliveries managed by the NEF are returned.

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/MbsGroupMsgDelData'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

post:

summary: Request the creation of an MBS Group Message Delivery.

operationId: CreateMbsGroupMsgDelivery

tags:

- MBS Group Message Deliveries

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/MbsGroupMsgDelReq'

responses:

'201':

description: >

Created. Successful case. An MBS Group Message Delivery is created.

content:

application/json:

schema:

$ref: '#/components/schemas/MbsGroupMsgDelResp'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

callbacks:

MbsGroupMsgDelStatusNotif:

'{$request.body#/notifUri}':

post:

requestBody:

description: >

Represents the status of MBS Group Message Delivery.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/MbsGroupMsgDelStatusNotif'

responses:

'204':

description: No content. The notification is successfully received.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/deliveries/{delRef}:

parameters:

- name: delRef

in: path

description: >

Identifier of the Individual MBS Group Message Delivery resource.

required: true

schema:

type: string

put:

summary: Request the modification of an existing MBS Group Message Delivery.

operationId: ModifyMbsGroupMsgDelivery

tags:

- Individual MBS Group Message Delivery

requestBody:

required: true

content:

application/json-patch+json:

schema:

$ref: '#/components/schemas/MbsGroupMsgDelPatch'

responses:

'200':

description: >

OK. Successful case. The MBS Group Message Delivery is modified.

content:

application/json:

schema:

$ref: '#/components/schemas/MbsGroupMsgDeliveryResp'

'204':

description: No Content. Successful case, the requested group message delivery is      successfully updated.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Deletes an existing Individual MBS Group Message Delivery resource.

operationId: DeleteMbsGroupMsgDelivery

tags:

- Individual MBS Group Message Delivery

responses:

'204':

description: >

No Content. The targeted MBS Group Message Delivery is successfully deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

# STRUCTURED DATA TYPES

MbsGroupMsgDelReq:

description: >

Represents the full set of parameters to initiate an MBS group message delivery request.

type: object

properties:

mbsGroupMsgDelData:

$ref: '#/components/schemas/MbsGroupMsgDelData'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- mbsGroupMsgDelData

MbsGroupMsgDelData:

description: Represents MBS Group Message Delivery data.

type: object

properties:

externalGroupId:

$ref: 'TS29122_CommonData.yaml#/components/schemas/ExternalGroupId'

mbsServArea:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MbsServiceArea'

extMbsServArea:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ExternalMbsServiceArea'

startTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

stopTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

required:

- externalGroupId

- startTime

- stopTime

oneOf:

- required: [mbsServArea]

- required: [extMbsServArea]

MbsGroupMsgDelResp:

description: >

Represents a response to a requested group message delivery.

type: object

properties:

mbsGroupMsgDelData:

$ref: '#/components/schemas/MbsGroupMsgDelData'

delStatus:

type: boolean

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- mbsGroupMsgDelData

- delStatus

MbsGroupMsgDelPatch:

description: Represents the requested modifications to an existing MBS Group Message Delivery.

type: object

properties:

externalGroupId:

$ref: 'TS29122_CommonData.yaml#/components/schemas/ExternalGroupId'

mbsServArea:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MbsServiceArea'

extMbsServArea:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ExternalMbsServiceArea'

startTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

stopTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

not:

required: [mbsServArea, extMbsServArea]

MbsGroupMsgDelStatusNotif:

description: Represents status notification information for an MBS group message delivery.

type: object

properties:

delStatus:

type: boolean

required:

- delStatus

# SIMPLE DATA TYPES

#

#

# ENUMERATIONS

# *** End of Changes ***