3GPP TSG-CT WG3 Meeting #127e	C3-231728

E-meeting, 17th – 21st April, 2023													(Revision of C3-231402)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.522 | CR | 0906 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MBS Group Message Delivery service description |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, ZTE, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS_Ph2 |  |  |  |  |  | Date: |  |  | 2023-03-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | MBS Group Message Delivery to deliver a payload to a group of UEs located in a particular geographical area when MBS broadcast is used is defined in TS 23.247. The corresponding implementation of the new service in 29.522 according to the SA2#154AHe agreed TS 23.247 CR 0161 and TS 23.501 CR 3755 are ready for procedures, API definition and OpenAPI file, while the basic service description is still missing for the implemented MBSGroupMsgDelivery API. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding service descriptions for MBS Group Message Delivery. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not aligned with stage 2 normative requirement, missing service description for Nnef_MBSGroupMsgDelivery Service API. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1, 4.3.1, 5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR does not impact the OpenAPI files. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

# Proposed changes:

# *** 1st Change ***

## 4.1	Overview

The NEF Northbound interface is between the NEF and the AF. It specifies RESTful/RPC APIs that allow the AF to access the services and capabilities provided by 3GPP network entities and securely exposed by the NEF.

This document also specifies the procedures triggered at the NEF by API requests from the AF and by event notifications received from 3GPP network entities.

The stage 2 level requirements and signalling flows for the NEF Northbound interface are defined in 3GPP TS 23.502 [2], 3GPP TS 23.247 [53] for MBS specific aspects and 3GPP TS 26.531 [59] for data reporting provisioning and Media Streaming Event Exposure specific aspects.

The NEF Northbound interface supports the following procedures:

1)	Procedures for Monitoring

2)	Procedures for Device Triggering

3)	Procedures for resource management of Background Data Transfer

4)	Procedures for CP Parameters, Network Configuration Parameters Provisioning, 5G LAN Parameters Provisioning, ACS Configuration Parameter Provisioning, Location Privacy Indication Parameters Provisioning and ECS address provisioning

5)	Procedures for PFD Management

6)	Procedures for Traffic Influence

7)	Procedures for changing the chargeable party at session set up or during the session

8)	Procedures for setting up an AF session with required QoS

9)	Procedures for MSISDN-less Mobile Originated SMS

10)	Procedures for non-IP data delivery

11)	Procedures for analytics information exposure

12)	Procedure for applying BDT policy

13)	Procedures for Enhanced Coverage Restriction Control

14)	Procedures for IPTV Configuration

15)	Procedures for Service Parameter Provisioning

16)	Procedures for RACS Parameter Provisioning

17)	Procedures for Mobile Originated Location Request

18)	Procedures for AKMA

19)	Procedures for AF triggered Access and Mobility Influence

20)	Procedures for AF triggered Access and Mobility Policy Authorization

21)	Procedures for Time Synchronization Exposure

22)	Procedures for EAS Deployment information provisioning

23)	Procedures for TMGI allocation, deallocation, expiry timer refresh and timer expiry notification

24)	Procedures for MBS session management and parameters provisioning.

25) Procedures for Data Reporting.

26) Procedures for Data Reporting Provisioning.

27)	Procedures for AF specific UE ID retrieval.

28) Procedures for Media Streaming Event Exposure.

29) Procedures for MBS User Service management.

30) Procedures for MBS User Data Ingest Session management.

31) Procedures for MBS Group Message Delivery management.

Which correspond to the following services respectively, supported by the NEF as defined in 3GPP TS 23.502 [2] or 3GPP TS 26.531 [59]:

1)	Nnef_EventExposure service and Nnef_APISupportCapability service

2)	Nnef_Trigger service

3)	Nnef_BDTPNegotiation service

4)	Nnef_ParameterProvision service

5)	Nnef_PFDManagement service

6)	Nnef_TrafficInfluence service

7)	Nnef_ChargeableParty service

8)	Nnef_AFsessionWithQoS service

9)	Nnef_MSISDN-less_MO_SMS service

10)	Nnef_NIDDConfiguration and Nnef_NIDD services

11)	Nnef_AnalyticsExposure service

12)	Nnef_ApplyPolicy service

13)	Nnef_ECRestriction service

14)	Nnef_IPTVConfiguration service

15)	Nnef_ServiceParameter service

16)	Nnef_UCMFProvisioning service

17)	Nnef_Location service

18)	Nnef_AKMA service

19)	Nnef_AMInfluence

20)	Nnef_AMPolicyAuthorization service

21)	Nnef_TimeSynchronization and Nnef_ASTI services

22)	Nnef_EASDeployment service

23)	Nnef_MBSTMGI service

24)	Nnef_MBSSession service

25) Nnef_DataReporting

26) Nnef_DataReportingProvisioning

27)	Nnef_UEId service

28) Nnef_MSEventExposure service

29) Nnef_MBSUserService

30) Nnef_MBSUserDataIngestSession

31) Nnef_ MBSGroupMsgDelivery

NOTE 1:	For Nnef_PFDManagement service, only the Nnef_PFDManagement_Create/Update/Delete service operations are applicable for the NEF Northbound interface.

NOTE 2:	For Nnef_NIDD service, NF consumer other than the AF does not use the NEF Northbound interface.

NOTE 3:	For Nnef_NIDDConfiguration service, the Nnef_NIDDConfiguration_Trigger service operation is only applicable for the NEF Northbound interface.

NOTE 4:	The Nnef_APISupportCapability service is only applicable in the MonitoringEvent API when the monitoring type sets to "API_SUPPORT_CAPABILITY".

NOTE 5:	The Nnef_MSEventExposure service maps to the Nnef_EventExposure service and is applicable for the case where the event consumer AF in the Application Service Provider is deployed outside the trusted domain, as described in 3GPP TS 26.531 [59], and the subscribed event is set to "MS_QOE_METRICS", "MS_CONSUMPTION", "MS_NET_ASSIST_INVOCATION", "MS_DYN_POLICY_INVOCATION", or "MS_ACCESS_ACTIVITY".

# *** 2nd Change ***

### 4.3.1	NEF

The Network Exposure Function (NEF) is a functional element that supports the following functionalities:

-	The NEF shall securely expose network capabilities and events provided by 3GPP NFs to AF.

-	The NEF shall provide means for the AF to securely provide information to 3GPP network and may authenticate, authorize and assist in throttling the AF.

-	The NEF shall be able to translate the information received from the AF to the one sent to internal 3GPP NFs, and vice versa.

-	The NEF shall support to expose information (collected from other 3GPP NFs) to the AF.

-	The NEF may support a PFD Function which allows the AF to provision PFD(s) and may store and retrieve PFD(s) in the UDR. The NEF further provisions PFD(s) to the SMF.

-	The NEF may support the time synchronization exposure function to the AF.

-	The NEF may provide means for the AF to influence access and mobility management related policies.

-	The NEF may provide means for the AF to provide inputs that can be used by the PCF for deciding access and mobility management related policies.

-	The NEF may provide means for the AF to provide the EAS Deployment information.

-	The NEF may provide means for the AF to retrieve AF specific UE ID.

-	The NEF may provide means for an untrusted event consumer AF to perform Media Streaming Event Exposure monitoring.

-	The NEF may provide means for the AF to request and manage MBS Group Message Delivery.

A specific NEF instance may support one or more of the functionalities described above and consequently an individual NEF may support a subset of the APIs specified for capability exposure.

NOTE:	The NEF can access the UDR located in the same PLMN as the NEF.

# *** 3rd Change ***

## 5.1	Introduction

The NEF Northbound APIs are a set of APIs defining the related procedures and resources for the interaction between the NEF and the AF.

Tables 5.1-1 summarizes the APIs defined in this specification.

Table 5.1-1: API Descriptions

| Service Name | Clause defined | Description | OpenAPI Specification File | API Name | Annex |
| --- | --- | --- | --- | --- | --- |
| TrafficInfluence | 5.4 | Traffic Influence API | TS29522_TrafficInfluence.yaml | 3gpp-traffic-influence | A.2 |
| NiddConfigurationTrigger | 5.5 | NIDD (Non-IP Data Delivery) Configuration Trigger API | TS29522_NiddConfigurationTrigger.yaml | 3gpp-nidd-configuration-trigger | A.3 |
| AnalyticsExposure | 5.6 | Analytics Exposure API | TS29522_AnalyticsExposure.yaml | 3gpp-analyticsexposure | A.4 |
| 5GLANParameterProvision | 5.7 | 5G LAN Parameter Provision API | TS29522_5GLANParameterProvision.yaml | 3gpp-5glan-pp | A.5 |
| ApplyingBdtPolicy | 5.8 | Applying BDT Policy API | TS29522_ApplyingBdtPolicy.yaml | 3gpp-applying-bdt-policy | A.6 |
| IPTVConfiguration | 5.9 | IPTV Configuration API | TS29522_IPTVConfiguration.yaml | 3gpp-iptvconfiguration | A.7 |
| LpiParameterProvision | 5.10 | LPI (Location Privacy Indicator) Parameter Provision API | TS29522_LpiParameterProvision.yaml | 3gpp-lpi-pp | A.8 |
| ServiceParameter | 5.11 | Service Parameter API | TS29522_ServiceParameter.yaml | 3gpp-service-parameter | A.9 |
| ACSParameterProvision | 5.12 | ACS Parameter Provision API | TS29522_ACSParameterProvision.yaml | 3gpp-acs-pp | A.10 |
| MoLcsNotify | 5.13 | MO LCS Notify API | TS29522_MoLcsNotify.yaml | 3gpp-mo-lcs-notify | A.11 |
| AKMA | 5.14 | AKMA API | TS29522_AKMA.yaml | 3gpp-akma | A.12 |
| TimeSyncExposure | 5.15 | Time Sync Exposure API | TS29522_TimeSyncExposure.yaml | 3gpp-time-sync-exposure | A.13 |
| EcsAddressProvision | 5.16 | ECS Address Provision API | TS29522_EcsAddressProvision.yaml | 3gpp-ecs-address-provision | A.14 |
| AMPolicyAuthorization | 5.17 | AM Policy Authorization API | TS29522_AMPolicyAuthorization.yaml | 3gpp-am-policyauthorization | A.15 |
| AMInfluence | 5.18 | AM Influence API | TS29522_AMInfluence.yaml | 3gpp-am-influence | A.16 |
| MBSTMGI | 5.19 | MBS TMGI API | TS29522_MBSTMGI.yaml | 3gpp-mbs-tmgi | A.17 |
| MBSSession | 5.20 | MBS Session API | TS29522_MBSSession.yaml | 3gpp-mbs-session | A.18 |
| EASDeployment | 5.21 | EAS Deployment API | TS29522_EASDeployment.yaml | 3gpp-eas-deployment | A.19 |
| ASTI | 5.22 | ASTI API | TS29522_ASTI.yaml | 3gpp-asti | A.20 |
| DataReporting | 5.23 | DataReporting API | TS29522_DataReporting.yaml | 3gpp-data-reporting | A.21 |
| DataReportingProvisioning | 5.24 | DataReportingProvisioning API | TS29522_DataReportingProvisioning.yaml | 3gpp-data-reporting-provisioning | A.22 |
| UEId | 5.25 | UE ID API | TS29522_UEId.yaml | 3gpp-ueid | A.23 |
| MBSUserService | 5.26 | MBSUserService API | TS29522_MBSUserService.yaml | 3gpp-mb-us | A.24 |
| MBSUserDataIngestSession | 5.27 | MBSUserDataIngestSession API | TS29522_ MBSUserDataIngestSession.yaml | 3gpp-mb-ud-ingest | A.25 |
| MSEventExposure | 5.28 | MSEventExposure API | TS29522_MSEventExposure.yaml | 3gpp-event-exposure | A.26 |
| MBSGroupMsgDelivery | 5.29 | MBSGroupMsgDelivery API | TS29522_MBSGroupMsgDelivery.yaml | 3gpp-mbs-group-msg-delivery | A.27 |

*** End of Changes ***