3GPP TSG CT WG3 Meeting #140	C3-251163

# Wuhan, CN, 7th – 11st April, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.165 | CR | 1041 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of data channel multiplexing |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  | 2025-03-31 |
| Category: | B |  |  |  |  |  | Release: |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  |  | 3GPP TS 23.228 defines the procedure of data channel multiplexing. This CR proposes the support of data channel multiplexing in NNI interface. |  |  |  |  |  |  |
| Summary of change: |  |  | This CR proposes to: - Add the description of data channel mulitplexing. |  |  |  |  |  |  |
| Consequences if not approved: |  |  | Not support the feature of data channel mulitplexing. |  |  |  |  |  |  |
| Clauses affected: |  | 33, 33.1(new), 33.2 (new) |  |  |  |  |  |  |  |
|  |  | Y |  | N |  |  |  |  |  |
| Other specs |  |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |
| affected: |  |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |
| (show related CRs) |  |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |
| Other comments: |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |

* * * * Start of changes * * * *

33	IMS data channel

33.1	General

Based on inter-operator agreement, the MMTEL service with IMS data channel as described in 3GPP TS 23.228 [4], 3GPP TS 24.186 [222] and 3GPP TS 26.114 [11] may be supported at the II-NNI.

If the MMTEL service with IMS data channel is supported, the procedures specified in 3GPP TS 24.186 [222] shall be applied and the capabilities below shall be provided at the II-NNI.

The "+sip.app-subtype" media feature tag with a value of "webrtc-datachannel" in the Contact header field parameter of the REGISTER request shall be supported at the roaming II-NNI.

The feature-capability indicator "g.3gpp.datachannel" (defined in 3GPP TS 24.186 [222] clause B.1.1) in the Feature-Caps header field of 200 (OK) response to REGISTER request shall be supported at the roaming II-NNI.

The "+sip.app-subtype" media feature tag with a value of "webrtc-datachannel" in the Contact header field parameter of INVITE and UPDATE requests and in 18x and 2xx responses to INVITE and UPDATE requests shall be supported at the II-NNI.

The "+sip.app-subtype" media feature tag (defined in IETF RFC 5688 [81]) with a value "webrtc-datachannel" in the Accept-Contact header field parameter of initial INVITE request may be supported at the II-NNI.

The "m=" line set to "application <port number> UDP/DTLS/SCTP webrtc-datachannel" and associated parameters in the media attribute lines "a=dcmap", "a=3gpp-qos-hint", "a=3gpp-req-app" and "a=3gpp-bdc-used-by" of the SDP body shall be supported at the II-NNI. For the "bootstrap" data channel as defined in table 6.2.10.1-2 of 3GPP TS 26.114 [11] only stream ID values 100 and 110 shall be supported at the II-NNI.

NOTE:	The "bootstrap" data channels with stream ID values 0 and 10 are strictly local between the UE and its local network.

33.2	Support of data channel multiplexing

If the data channel multiplexing is supported, the procedures specified in 3GPP TS 24.186 [222] shall be applied and the capabilities below shall be provided at the II-NNI.

The "+g.3gpp.dc-mux" media feature tag in the Contact header field (defined in 3GPP TS 26.114 [11]) of the REGISTER request shall be supported at the roaming II-NNI.

The feature-capability indicator "g.3gpp.dc-mux" (defined in 3GPP TS 24.186 [222] clause B.1.2) in the Feature-Caps header field of 200 (OK) response to REGISTER request shall be supported at the roaming II-NNI.

The "+g.3gpp.dc-mux" media feature tag in the Contact header field parameter of INVITE and UPDATE requests and in 18x and 2xx responses to INVITE and UPDATE requests shall be supported at the II-NNI.

* * * * End of changes * * * *