3GPP TSG-CT WG3 Meeting #78bis	C3-144308

# Sophia Antipolis, France, 20 - 24 October 2014

| PCR-Form-v0.3 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| PSEUDO CR |  |  |  |  |  |
|  | Spec. number: | 29.217 | Current version: | 0.0.0 |  |

| Title: | RUCI report over Np reference point |  |
| --- | --- | --- |
| Source: | China Telecom,ZTE |  |
| Work item code: | UPCON-DOTCON-CT |  |
| Reason for change: | Initial definition of RUCI report over Np reference point |  |
| Summary of change: | Define RUCI report in 29.217 |  |
| Consequences if not agreed: | The related function missing on stage 3 |  |
| Other specs |  |  |
| affected(if any): |  |  |
| Other comments |  |  |

Additional discussion(if needed):

…

Proposed changes:

*** 1st Change ***

References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.203: "Policy and charging control architecture".

[xx]	IETF RFC 4005: “Diameter Network Access Server Application”.

[yy]	IETF RFC 4006: “Diameter Credit Control Application”.

[zz]	3GPP TS 29.229: “Cx and Dx interfaces based on Diameter protocol; Protocol details”.

*** 2nd Change ***

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

ARR	Aggregated_RUCI_Report_Request command

ARA	Aggregated_RUCI_Report_Answer command

RCAF	RAN Congestion Awareness Function

RUCI	RAN User Plane Congestion Information

NRR	Non-Aggregated_RUCI_Report_Request command

NRA	Non-Aggregated_RUCI_Report_Answer command

*** 3rd Change ***

4.4.1	RUCI Report

Two types of RUCI reports are used on Np for transfer of congestion information from RCAF to PCRF: Non-aggregated RUCI report and Aggregated RUCI report.

4.4.1.1	Non-aggregated RUCI report

For a Non-aggregated RUCI report, the RCAF shall send an NRR command to the PCRF by including the user id within the Subscription-Id AVP, PDN ID within the Called-Station-Id AVP and a congestion level set id witin the Congestion-Level-Set-Id AVP if the reporting restriction was provided earlier or a congestion level value within the Congestion-Level-Value AVP if the reporting restriction was not provided earlier at the command level.

Once the PCRF receives the NRR command, the PCRF shall store the related info and respond with an NRA command including the logical PCRF id within the Logical-PCRF-Id AVP. The PCRF may use the RUCI received from the RCAF as input for policy decisions

Editor's note:	It is FFS whether the reporting restriction can be provided in the NRA command.

Editor’s note: It is FFS whether the NRR/NRA command is stateful or stateless.

Editor's note:	In order to enable the reporting restriction provisioning, the PCRF needs to know the identity of the RCAF. If the NRR/NRA is stateless, the additional AVP to contain the RCAF identity is needed.

4.4.1.2	Aggregated RUCI report

For an Aggregated RUCI report, the RCAF shall aggregate the RUCIs of different user ids and PDN IDs that have same destination PCRF. The RCAF shall send an ARR command to the destination PCRF by including one or more Aggregated-RUCI-Report AVP with a congestion level set id within the Congestion-Level-Set-Id AVP if the reporting restriction was provided earlier or a congestion level value within the Congestion-Level-Value AVP if the reporting restriction was not provided earlier, the PDN ID within the Called-Station-ID AVP and the user id list in the Subscription-Id AVPs. The RCAF shall include the Auth-Session-State AVP set to NO_STATE_MAINTAINED.

NOTE: Each instance of Aggregated-RUCI-Report AVP aggregates the user id list of the subscribers that share the same level of congestion.

Editor's note:	It is FFS whether the Called-Station-ID AVP is included in the ARR command.

Editor's note: Inclusion of location information (ECGI, eNodeB ID) in Aggregate-RUCI-Report AVP is FFS

Once the PCRF receives the ARR command, the PCRF shall store the related info and respond with an ARA command. The PCRF may use the RUCI received from the RCAF as input for policy decisions.

Editor's note:	It is FFS whether the Supported-Features AVP is included in the ARR/ARA command..

*** 4th  Change ***

5.4	Np specific AVPs

Table 5.4.1 describes the Diameter AVPs defined for the Np reference point, their AVP Code values, types, possible flag values, whether or not the AVP may be encrypted and what access types (e.g. 3GPP-EPS, etc.) the AVP is applicable to. The Vendor-Id header of all AVPs defined in the present document shall be set to 3GPP (10415).

Table 5.4.1: Np specific Diameter AVPs

|  |  |  |  | AVP Flag rules (NOTE x) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Attribute Name | AVP Code | Clause defined | Value Type (NOTE y) | Must | May | Should not | Must not | May Encr. | Acc. Type | Applicability |
| Aggregated-RUCI-Report | xxxx | 5.4.x | Grouped | V, M | P |  |  | Y | 3GPP-EPS | Both |
| Congestion-Level-Value | yyyy | 5.4.y | Unsigned32 | V, M | P |  |  | Y | 3GPP-EPS | Both |
| Logical-PCRF-Id | ## zzzz | ## 5.4.z | ## UTF8String | ## V, M | ## P |  |  | Y | 3GPP-EPS | Both |
| NOTE x: The AVP header bit denoted as ‘M’, indicates whether support of the AVP is required. The AVP header bit denoted as ‘V’, indicates whether the optional Vendor-ID field is present in the AVP header. For further details, see RFC 3588 [a]. NOTE y: The value types are defined in RFC 3588 [a]. ## NOTE z: AVPs marked with “Both” are applicable to both charging control and policy control. |  |  |  |  |  |  |  |  |  |  |

*** 5th Change ***

5.4.x	Aggregated-RUCI-Report AVP

The Aggregated-RUCI-Report AVP (AVP code xxxx) is of type Grouped, and it is used to contain the congestion level value or congestion level set id for a set of users which have same PCRF for the same PDN ID.

The congestion-Level-Value AVP contains the congestion level value if the PCRF did not provide the reporting restriction earlier for the user id and PDN ID.

The Congestion-Level-Set-Id AVP contains the congestion level set identifier between the PCRF and the RCAF if the PCRF provided the reporting restriction earlier for the user id and PDN ID.

The Called-Station-Id AVP contains the PDN ID.

The Subscription-Id AVP contains the corresponding user id list for the Congestion-Level-Value or Congestion-Level-Set-Id.

Aggregated-RUCI-Report ::= < AVP Header: xxxx >

[ Congestion-Level-Value ]

[ Congestion-Level-Set-Id ]

[ Called-Station-Id ]

*[ Subscription-Id ]

*[ AVP ]

Editor's note:	It is FFS whether the Called-Station-ID AVP is included in the Aggregated-RUCI-Report.

Editor's note:	It is FFS whether the Subscription-Id AVP can be replaced by a new defined AVP to save the space of the message.

Editor's note: Inclusion of location information (ECGI, eNodeB ID) in Aggregate-RUCI-Report AVP is FFS.

Editor’s note: Other structure of the Aggregated-RUCI-Report needs to be considered.

*** 6th Change ***

5.4.Y	Congestion-Level-Value  AVP

The Congestion-Level-Value AVP (AVP code yyyy) is of type Unsigned32, and it is used to indicate the congestion level of the cell where the UE is located.

*** 7th Change ***

5.4.z	Logical-PCRF-Id	AVP

The Logical-PCRF-Id AVP (AVP code zzzz) is of type UTF8String, and it is used to uniquely identify a PCRF. It has the following ABNF: <FQDN>[;<optional>], where <FQDN> is the Fully Qualified Domain Name and <optional> may be implementation specific.

Editor’s note: It is FFS how this AVP is used.

***8th Change ***

5.5	Np re-used AVPs

5.5.1 General

Table 5.5.1 lists the Diameter AVPs re-used by the Np reference point from other existing Diameter Applications, reference to their respective specifications, short description of their usage within the Np reference point, the applicability of the AVPs to a specific access, and which supported features the AVP is applicable to. AVPs from existing Diameter Applications, except for the AVPs from Diameter base protocol, do not need to be supported. The AVPs from Diameter base protocol are not included in table 5.5.1, but they are re-used for the Np reference point. Unless otherwise stated, re-used AVPs shall maintain their ‘M’, ‘P’ and ‘V’ flag settings. Where RADIUS VSAs are re-used, unless otherwise stated, they shall be translated to Diameter AVPs as described in RFC 4005 [xx] with the exception that the ‘M’ flag shall be set and the ‘P’ flag may be set.

Table 5.5.1: Np re-used Diameter AVPs

| Attribute Name | Reference | Description | Acc. Type | Applicablity (note 1) |
| --- | --- | --- | --- | --- |
| Called-Station-Id | IETF RFC 4005 [xx] | The address the user is connected to (i.e. the PDN identifier). | 3GPP EPS | Both |
| Subscription-Id | IETF RFC 4006 [yy] | The identification of the subscription (i.e. IMSI) | 3GPP EPS | Both |
| Supported-Features | 3GPP TS 29.229 [zz] | If present, this AVP informs the destination host about the features that the origin host requires to successfully complete this command exchange | 3GPP EPS | Both |
| NOTE 1: AVPs marked with “CC” are applicable to charging control, AVPs marked with “PC” are applicable to policy control and AVPs marked with “Both” are applicable to both charging control and policy control. |  |  |  |  |

***9th Change ***

5.5.2	Use of the Supported-Features AVP on the Np reference point

When new functionality is introduced on the Np reference point, it should be defined as optional. If backwards incompatible changes cannot be avoided, the new functionality shall be introduced as a new feature and support advertised with the Supported-Features AVP. Unless otherwise stated, the use of the Supported-Features AVP on the Np reference point shall be compliant to the usage of the Supported-Features AVP on the Cx reference point and consistent with the procedures for the dynamic discovery of supported features as defined in clause 7.2 of 3GPP TS 29.229 [zz].

When extending the application by adding new AVPs for a feature, the new AVPs shall have the M bit cleared and the AVP shall not be defined mandatory in the command ABNF.

As defined in 3GPP TS 29.229 [zz], the Supported-Features AVP is of type grouped and contains the Vendor-Id, Feature-List-ID and Feature-List AVPs. On the all reference points as specified in this specificaion, the Supported-Features AVP is used to identify features that have been defined by 3GPP and hence, for features defined in this document, the Vendor-Id AVP shall contain the vendor ID of 3GPP (10415). If there are multiple feature lists defined for the reference point, the Feature-List-ID AVP shall differentiate those lists from one another.

Editor’s Note: Depending on if stateless or stateful Diameter application is chosen this subclause may need to be updated.

The Table 5.5.1.1 defines the features applicable to the Np reference point for the feature list with a Feature-List-ID of 1.

The table below defines the features applicable to the Np interface for the feature list with a Feature-List-ID of 1.

# Table 5.5.1.1: Features of Feature-List-ID 1 used in Np

| Feature bit | Feature | M/O | Description |
| --- | --- | --- | --- |
| 0 | ReportRestriction | O | This feature indicates the support of reporting restriction |
| Feature bit: The order number of the bit within the Feature-List AVP where the least significant bit is assigned number “0”. Feature: A short name that can be used to refer to the bit and to the feature, e.g. “EPS”. M/O: Defines if the implementation of the feature is mandatory (“M”) or optional (“O”) in this 3GPP Release. Description: A clear textual description of the feature. |  |  |  |

***10th Change ***

5.7.x	Non-Aggregated-RUCI-Report-Request (NRR) command

The NRR command, indicated by the Command-Code field set to xxxxxx and the 'R' bit set in the Command Flags field, is sent by the RCAF to the PCRF as part of the Non-aggregated RUCI report procedure.

Message Format:

<NR-Request> ::= <Diameter Header: xxxxxx, REQ, PXY >

< Session-Id >

{ Auth-Application-Id }

{ Auth-Session-State }

{ Origin-Host }

{ Origin-Realm }

{ Destination-Realm }

[ Destination-Host ]

[ Origin-State-Id ]

[ Subscription-Id ]

[ Called-Station-Id ]

[ Congestion-Level-Value ]

[ Congestion-Level-Set-Id ]

*[ Proxy-Info ]

*[ Route-Record ]

*[ Supported-Features ]

*[ AVP ]

***11th Change ***

5.7.y	Non-Aggregated-RUCI-Report-Answer (NRA) command

The NRA command, indicated by the Command-Code field set to xxxxxx and the 'R' bit cleared in the Command Flags field, is sent by the PCRF to the RCAF as part of the Non-aggregated RUCI report procedure.

Message Format:

<NR-Answer> ::=  < Diameter Header: xxxxxx, PXY >

< Session-Id >

{ Auth-Application-Id }

{ Auth-Session-State }

{ Origin-Host }

{ Origin-Realm }

[ Result-Code ]

[ Experimental-Result ]

[ Error-Message ]

[ Error-Reporting-Host ]

*[ Failed-AVP ]

[ Logical-PCRF-Id ]

[ Origin-State-Id ]

*[ Redirect-Host ]

[ Redirect-Host-Usage ]

[ Redirect-Max-Cache-Time ]

*[ Proxy-Info ]

*[ Supported-Features ]

*[ AVP ]

***12th Change ***

5.7.z	Aggregated-RUCI-Report-Request (ARR) command

The ARR command, indicated by the Command-Code field set to xxxxxx and the 'R' bit set in the Command Flags field, is sent by the RCAF to the PCRF as part of the Aggregated RUCI report procedure.

Message Format:

<AR-Request> ::= <Diameter Header: xxxxxx, REQ, PXY >

< Session-Id >

{ Auth-Application-Id }

{ Auth-Session-State }

{ Origin-Host }

{ Origin-Realm }

{ Destination-Realm }

[ Destination-Host ]

[ Origin-State-Id ]

*[ Aggregated-RUCI-Report ]

*[ Proxy-Info ]

*[ Route-Record ]

*[ Supported-Features ]

*[ AVP ]

***12th Change ***

5.7.a	Aggregated-RUCI-Report-Answer (ARA) command

The ARA command, indicated by the Command-Code field set to xxxxxx and the 'R' bit cleared in the Command Flags field, is sent by the PCRF to the RCAF as part of the Aggregated RUCI report procedure.

Message Format:

<AR-Answer> ::=  < Diameter Header: xxxxxx, PXY >

< Session-Id >

{ Auth-Application-Id }

{ Auth-Session-State }

{ Origin-Host }

{ Origin-Realm }

[ Result-Code ]

[ Experimental-Result ]

[ Error-Message ]

[ Error-Reporting-Host ]

*[ Failed-AVP ]

[ Origin-State-Id ]

*[ Redirect-Host ]

[ Redirect-Host-Usage ]

[ Redirect-Max-Cache-Time ]

*[ Proxy-Info ]

*[ Supported-Features ]

*[ AVP ]

*** End of Changes ***