3GPP TSG CT WG4 Meeting #85bis	C4-185072

Sophia Antipolis, France, 09st – 13th July 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.244 | CR | 0120 | rev | - | Current version: | 14.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Esstential correction on Monitoring Time |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | CUPS-CT |  |  |  |  |  |  | Date: |  | 2018-06-27 |
| Category: | A |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For offline charging, in many deployments, operators may define multiple tariff change during a day, e.g. 8:00 in the morning, 18:00 in the afternoon, and 23:00 in the midnight. To consider other use case, there may need more tariff change, e.g. for usage monitoring for Gx. Currently, the PFCP spec allows CP function to provision only one Monitoring Time, which is only one tariff change. In such case, the CP function shall be prepared, once the first tariff change is passed, the CP should provide another monitoring time, which is much less efficient and introducing more signalling. In some cases, if two monitoring time are too close, it may not work. In addition, such tariff change should be applicable for Sxa interface. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to create a new IE called additional Monitoring Time which includes a monitoring time, Subsequent time/volume threshold/quota, and this IE may have multiple instances representing multiple monitoring times. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 2 requirements are not fulfilled.. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.2.1, 5.4.10, 7.5.2.4, 7.5.4.4, 8.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | Actually we can directly create a new group IE additional monitoring time, to substitute the the current IEs (Monitoring Time, Subsequent Volume/Time Threshold/Quotas) in the message level. |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 5.2.2.2.1	General

When provisioning a URR, the CP function shall provide the reporting trigger(s) in the Reporting Triggers IE of the URR which shall cause the UP function to generate and send a Usage Report for this URR to the CP function. When adding or removing reporting trigger(s) to or from the URR, the CP function shall provide the new complete list of applicable reporting triggers in the Reporting Triggers IE in the PFCP Session Modification Request message.

For the volume-based measurement method, the CP function may provision:

-	the Volume Threshold IE, to request the UP function to generate a usage report when the measured traffic reaches the threshold;

-	the Volume Quota IE, to request the UP function to stop forwarding packets (or only allow forwarding of some limited user plane traffic, based on operator policy in the UP function) and, if no Volume Threshold is provisioned, to also generate a usage report, when the measured traffic reaches the quota;

-	the Dropped DL Traffic Threshold IE, to request the UP function to generate a usage report when the downlink traffic that is being dropped reaches the threshold; and/or

NOTE 1:	The Dropped DL Traffic Threshold can be armed in a SGW-U for triggering the PGW Pause of Charging feature (see 3GPP TS 23.401 [14]).

-	a Measurement Information with the 'Measurement Before QoS Enforcement' flag set to 1, to request the UP function to measure the traffic usage before any QoS enforcement.

For the time-based measurement method, the CP function may provision:

-	a Time Threshold IE, to request the UP function to generate a usage report when the measured traffic reaches the threshold;

-	a Time Quota, to request the UP function to stop forwarding packets (or only allow forwarding of some limited user plane traffic, based on operator policy in the UP function) and, if no Time Threshold is provisioned, to also generate a usage report, when the measured traffic reaches the quota; and/or

-	an Inactivity Detection Time, to request the UP function to suspend the time measurement when no packets are received during the provisioned Inactivity Detection Time. The time measurement shall then be resumed by the UP function when subsequent traffic is received. If an Inactivity Detection Time value of zero is provided, or if no Inactivity Detection Time has been provided by the CP function, the time measurement shall be performed continuously from the point when the first packet is received until the time-based usage measurement is stopped. See Figure 5.2.2.2-1:

Figure 5.2.2.2-1: IDT based charging

NOTE 2:	The Inactivity Detection Time can be set to the Quota Consumption Timer if received.

-	For EPC, a Time Quota Mechanism, including a Base Time Interval Type, which is either Continuous Time Period (CTP) or Discrete Time Period (DTP), and a Base Time Interval (BTI), to the UP function. See subclause 6.5.7 in 3GPP TS 32.299[18].

-	For CTP (Continuous Time Period), the time measurement starts from the time that traffic has occurred up to the first Base Time Interval (BTI) which contains no traffic. The time measurement shall include the last Base Time Interval, i.e. the one which contained no traffic. The time measurement resumes by the UP function when subsequent traffic is received. See Figure 5.2.2.2-2:

Figure 5.2.2.2-2: CTP based charging

-	For DTP (Descrete Time Period), the time measurement starts from the time that traffic has occurred up to the Base Time Interval end. The time measurement shall be resumed by the UP function when subsequent traffic is received. See Figure 5.2.2.2-3:

Figure 5.2.2.2-3: DTP based charging

When the time-based measurement method applies, and when the Envelope Reporting is required for EPC, the CP function shall request the UP function to report the usage by setting the reporting trigger to Envelope Closure in addition to other Reporting Trigger(s), in the Reporting Triggers IE. The CP function may indicate the UP function to report for just time, time and volume, time and events, or time and volume and number of events by setting Measurement Method accordingly. The CP function may set the Reduced Application Detection Information flag in the Measurement Information of the URR, when requesting the detection of start and stop of an application solely for the purpose of envelope reporting for EPC.

The CP function may provision a Volume Threshold, a Volume Quota, or both (and/or respectively a Time Threshold, a Time Quota, or both).

When both a Volume (or Time) Threshold and a Volume (or Time) Quota are provisioned, the UP function shall send a usage report only when reaching the Volume (or Time) Threshold; when subsequently reaching the Volume (or Time) Quota, the UP function shall stop forwarding packets (or only allow forwarding of some limited user plane traffic, based on operator policy in the UP function) without sending a new usage report to the CP function.

NOTE 3:	For online charging, the Volume Threshold (or Time Threshold) can be set in a PGW-U or TDF-U to the value of the granted volume (or time) quota minus the volume (or time) quota threshold, such as to get a usage report from the UP function when the volume (or time) based credit falls below the remaining quota thresholds provided by the OCS.

NOTE 4:	The Volume Quota or Time Quota can be armed in a PGW-U or TDF-U for online charging to enable the traffic to be forwarded up to an intermediate or final quotas granted by the OCS. The CP function can provision both a Volume (or Time) Threshold and a Volume (or Time) Threshold to request the UP function to:
-	send a usage report when the consumed resources reach the volume (or time) usage threshold provided by the OCS, and 
-	to stop forwarding packets (or only allow forwarding of some limited user plane traffic, based on operator policy in the UP function), without sending a second usage report, when the granted volume (or time) quota is exhausted.

For all the measurement methods (i.e. volume, time or event), the CP function may also provision:

-	a Quota Holding Time, to request the UP function to send a usage report and to also stop forwarding packets (or only allow forwarding of some limited user plane traffic, based on operator policy in the UP function) when no packets have been received for the duration indicated in this parameter;

NOTE 5:	A Quota Holding Time can be armed in a PGW-U or TDF-U for online charging to request the UP function to send a Usage Report when the Quota Holding Time provided by the OCS (see 3GPP TS 32.299 [18]) expires. The UP function can be instructed in the same Usage Reporting Rule with the Report Triggers – START to generate a new Usage Report upon receiving any subsequent packets associated with this URR.

-	a Monitoring Time IE and zero or more Additional Monitoring IEs, to request the UP function to measure the network resources usage before and after the monitoring time in separate counts and to re-apply the volume and time thresholds at the monitoring time. The CP function may additionally provision a Subsequent Volume (or Time) Threshold IE and/or a Subsequent Volume (or Time) Quota IE, for a volume (or time) based measurement. When being provisioned with a Monitoring Time, the UP function shall:

-	reset its usage thresholds at the monitoring time to the value provided in the Subsequent Volume (or Time) Threshold IE, if provisioned in the URR, or to the remaining value of the Volume (or Time) threshold used before the monitoring time (i.e. excluding the already accumulated volume or time usage);

-	shall indicate the usage up to the Monitoring time and usage after the Monitoring time in the first usage report after the Monitoring Time is reached;

-	a Measurement Period, indicating the period to generate periodic usage reports to the CP function.

If the UP function indicated support of the Quota Action feature in the UP Function Features IE, when the CP function provisions a Volume Quota or Time Quota in a URR, the CP function may also provision the "FAR ID for Quota Action" IE identifying the substitute FAR the UP function shall apply, for the traffic associated to this URR, when exhausting any of these quotas. This FAR may require the UP function to drop the packets or redirect the traffic towards a redirect destination as specified in subclause 5.4.7.

The CP function may request at any time the UP function to activate or deactivate a network resources usage measurement and the sending of corresponding Usage Reports, using the Inactive Measurement flag of the Measurement Information IE of the URR.

NOTE 6:	This can be used in a PGW-U for the PGW Pause of Charging procedure (see 3GPP TS 23.401 [14]).

* * * Next Change * * * *

### 5.4.10	Charging

The charging requirements for online and offline charging in the PS domain specified in 3GPP TS 32.251 [17] shall be preserved with a split SGW, PGW and TDF architecture.

Charging is supported by the CP function by activating in the UP function the measurement and reporting of the accumulated usage of network resources per:

-	IP-CAN bearer, for an SGW;

-	IP-CAN bearer, IP-CAN session and/or individual or group of service data flows, for a PGW;

-	TDF session and/or individual or group of applications, for a TDF.

See subclauses 5.3 and 7.8.4 of 3GPP TS 23.214 [2].

The CP function shall control the usage measurement and reporting in the UP function by:

-	creating the necessary PDR(s) to represent the service data flow, application, bearer or session, if not already existing;

-	creating URR(s) for each Charging Key, combination of Charging Key and Service ID, or combination of Charging Key, Sponsor ID and Application Service Provider Id;

-	associating the URR(s) to the relevant PDRs defined for the PFCP session, for usage reporting at IP-CAN bearer, IP-CAN session, TDF session, SDF or application level.

For online charging, the CP function shall provision the URR with the Volume (or Time) Quota, and with the Volume (or Time) Quota if a quota threshold was received from the OCS, as specified in subclause 5.2.2.2. Besides, when the OCS provides a final quota and requests to redirect the traffic towards a redirect destination when exhausing this quota, the CP function shall redirect the traffic towards a redirect destination as specified in subclause 5.4.7 upon being notified that the final quota has been reached; to permit HTTP traffic redirection, the UP function should have at least one HTTP packet, e.g. the UP function may store one packet when reaching the Volume (or Time) Quota. An example call flow is depicted in Annex C.2.1.1.

To avoid the risk of signalling storms between the CP and UP functions at times of tariff change, the CP function may include the Monitoring Time IE and zero or more Additional Monitoring IEs in the URR and set it to the time of tariff change to request the UP function to report separately the resource usage before and after the time of tariff change (see e.g. subclause 6.3.7.1 of 3GPP TS 32.299 [18]).

* * * Next Change * * * *

#### 7.5.2.4	Create URR IE within PFCP Session Establishment Request

The Create URR grouped IE shall be encoded as shown in Figure 7.5.2.4-1.

Table 7.5.2.4-1: Create URR IE within PFCP Session Establishment Request

| Octet 1 and 2 |  | Create URR IE Type = 6 (decimal) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Octets 3 and 4 |  | Length = n |  |  |  |  |  |
| Information elements | P | Condition / Comment | Appl. |  |  |  | IE Type |
|  |  |  | Sxa | Sxb | Sxc | N4 |  |
| URR ID | M | This IE shall uniquely identify the URR among all the URRs configured for this PFCP session. | X | X | X | X | URR ID |
| Measurement Method | M | This IE shall indicate the method for measuring the network resources usage, i.e. whether the data volume, duration (i.e. time), combined volume/duration, or event shall be measured. | X | X | X | X | Measurement Method |
| Reporting Triggers | M | This IE shall indicate the trigger(s) for reporting network resources usage to the CP function, e.g. periodic reporting or reporting upon reaching a threshold, or envelope closure. | X | X | X | X | Reporting Triggers |
| Measurement Period | C | This IE shall be present if periodic reporting is required. When present, it shall indicate the period for generating and reporting usage reports. | X | X | X | X | Measurement Period |
| Volume Threshold | C | This IE shall be present if volume-based measurement is used and reporting is required upon reaching a volume threshold. When present, it shall indicate the traffic volume value after which the UP function shall report network resources usage to the CP function for this URR. | X | X | X | X | Volume Threshold |
| Volume Quota | C | This IE shall be present if volume-based measurement is used and the CP function needs to provision a Volume Quota in the UP function (see subclause 5.2.2.2) When present, it shall indicate the Volume Quota value. | - | X | X | X | Volume Quota |
| Time Threshold | C | This IE shall be present if time-based measurement is used and reporting is required upon reaching a time threshold. When present, it shall indicate the time usage after which the UP function shall report network resources usage to the CP function for this URR. | X | X | X | X | Time Threshold |
| Time Quota | C | This IE shall be present if time-based measurement is used and the CP function needs to provision a Time Quota in the UP function (see subclause 5.2.2.2) When present, it shall indicate the Time Quota value | - | X | X | X | Time Quota |
| Quota Holding Time | C | This IE shall be present, for a time, volume or event-based measurement, if reporting is required and packets are no longer permitted to pass on when no packets are received during a given inactivity period. When present, it shall contain the duration of the inactivity period. | - | X | X | X | Quota Holding Time |
| Dropped DL Traffic Threshold | C | This IE shall be present if reporting is required when the DL traffic being dropped exceeds a threshold. When present, it shall contain the threshold of the DL traffic being dropped. | X | - | - |  | Dropped DL Traffic Threshold |
| Monitoring Time | O | When present, this IE shall contain the time at which the UP function shall re-apply the volume or time threshold. | X | X | X | X | Monitoring Time |
| Subsequent Volume Threshold | O | This IE may be present if the Monitoring Time IE is present and volume-based measurement is used. When present, it shall indicate the traffic volume value after which the UP function shall report network resources usage to the CP function for this URR for the period after the Monitoring Time. | X | X | X | X | Subsequent Volume Threshold |
| Subsequent Time Threshold | O | This IE may be present if the Monitoring Time IE is present and time-based measurement is used. When present, it shall indicate the time usage after which the UP function shall report network resources usage to the CP function for this URR for the period after the Monitoring Time. | X | X | X | X | Subsequent Time Threshold |
| Subsequent Volume Quota | O | This IE may be present if Monitoring Time IE is present and volume-based measurement is used (see subclause 5.2.2.2). When present, it shall indicate the Volume Quota value which the UP function shall use for this URR for the period after the Monitoring Time. | X | X | X | X | Subsequent Volume Quota |
| Subsequent Time Quota | O | This IE may be present if Monitoring Time IE is present and time-based measurement is used (see subclause 5.2.2.2) When present, it shall indicate the Time Quota value which the UP function shall use for this URR for the period after the Monitoring Time. | X | X | X | X | Subsequent Time Quota |
| Inactivity Detection Time | C | This IE shall be present if time-based measurement is used and the time measurement need to be suspended when no packets are received during a given inactivity period. When present, it shall contain the duration of the inactivity period. | - | X | X | X | Inactivity Detection Time |
| Linked URR ID | C | This IE shall be present if linked usage reporting is required. When present, this IE shall contain the linked URR ID which is related with this URR (see subclause 5.2.2.4). Several IEs with the same IE type may be present to represent multiple linked URRs which are related with this URR. | - | X | X | X | Linked URR ID |
| Measurement Information | C | This IE shall be included if any of the following flag is set to 1. Applicable flags are: - Measurement Before QoS Enforcement Flag: this flag shall be set to 1 if the traffic usage before any QoS Enforcement is requested to be measured. - Inactive Measurement Flag: this flag shall be set to 1 if the measurement shall be paused (inactive). The measurement shall be performed (active) if the bit is set to 0 or if the Measurement Information IE is not present in the Create URR IE. - Reduced Application Detection Information Flag: this flag may be set to 1, if the Reporting Triggers request to report the start or stop of application, to request the UP function to only report the Application ID in the Application Detection Information, e.g. for envelope reporting. | - - | X X | X - | X X | Measurement Information |
| Time Quota Mechanism | C | This IE shall be present if time-based measurement based on CTP or DTP is used. | - | X | - | - | Time Quota Mechanism |
| Aggregated URRs | C | This IE shall be included if the URR is used to support a Credit Pool. Several IEs with the same IE type may be present to provide multiple aggregated URRs. | - | X | - |  | Aggregated URRs |
| FAR ID for Quota Action | C | This IE may be present if the Volume Quota IE and/or the Time Quota IE is provisioned in the URR and the UP Function indicated support of the Quota Action feature. When present, it shall contain the identifier of the substitute FAR the UP function shall apply, for the traffic associated to this URR, when exhausting any of these quotas. See NOTE 1. | - | X | X | X | FAR ID |
| Ethernet Inactivity Timer | C | This IE shall be present if Ethernet traffic reporting is used and the SMF requests the UP function to also report inactive UE MAC addresses. When present, it shall contain the duration of the Ethernet inactivity period. | - | - | - | X | Ethernet Inactivity Timer |
| Additional Monitoring Time | C | When present, this IE shall contain the time at which the UP function shall re-apply the volume or time threshold/quota provisioned in the IE. Several IEs with the same IE type may be present to provide multiple Monitoring Times. | X | X | X | X | Additional Monitoring Time |
| NOTE 1: The substitute FAR used when exhausting a Volume Quota or Time Quota may be set to drop the packets or redirect the traffic towards a redirect destination as specified in subclause 5.4.7. |  |  |  |  |  |  |  |

Editor's Note:	In 5GC Pause of charging support is FFS (Dropped down link packets).

# Table 7.5.2.4-2: Aggregated URRs

| Octet 1 and 2 |  | Aggregated URRs = 118 (decimal) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Octets 3 and 4 |  | Length = n |  |  |  |  |  |
| Information elements | P | Condition / Comment | Appl. |  |  |  | IE Type |
|  |  |  | Sxa | Sxb | Sxc | N4 |  |
| Aggregated URR ID | M | This IE shall be present for the aggregated URR ID of the URR sharing the credit pool. | - | X | - | - | Aggregated URR ID |
| Multiplier | M | This IE shall be included to measure the abstract service units the traffic of the corresponding aggregated URR consumes from the credit pool. | - | X | - | - | Multiplier |

# Table 7.5.2.4-3: Additional Monitoring Time

| Octet 1 and 2 |  | Additional Monitoring Time = xx (decimal) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Octets 3 and 4 |  | Length = n |  |  |  |  |  |
| Information elements | P | Condition / Comment | Appl. |  |  |  | IE Type |
|  |  |  | Sxa | Sxb | Sxc | N4 |  |
| Monitoring Time | C | This IE shall be present and contain the time at which the UP function shall re-apply the volume or time threshold/quota. | X | X | X | X | Monitoring Time |
| Subsequent Volume Threshold | O | This IE may be present if the Monitoring Time IE is present and volume-based measurement is used. When present, it shall indicate the traffic volume value after which the UP function shall report network resources usage to the CP function for this URR for the period after the Monitoring Time. | X | X | X | X | Subsequent Volume Threshold |
| Subsequent Time Threshold | O | This IE may be present if the Monitoring Time IE is present and time-based measurement is used. When present, it shall indicate the time usage after which the UP function shall report network resources usage to the CP function for this URR for the period after the Monitoring Time. | X | X | X | X | Subsequent Time Threshold |
| Subsequent Volume Quota | O | This IE may be present if Monitoring Time IE is present and volume-based measurement is used (see subclause 5.2.2.2). When present, it shall indicate the Volume Quota value which the UP function shall use for this URR for the period after the Monitoring Time. | X | X | X | X | Subsequent Volume Quota |
| Subsequent Time Quota | O | This IE may be present if Monitoring Time IE is present and time-based measurement is used (see subclause 5.2.2.2) When present, it shall indicate the Time Quota value which the UP function shall use for this URR for the period after the Monitoring Time. | X | X | X | X | Subsequent Time Quota |

* * * Next Change * * * *

#### 7.5.4.4	Update URR IE within PFCP Session Modification Request

The Update URR grouped IE shall be encoded as shown in Figure 7.5.4.4-1.

Table 7.5.4.4-1: Update URR IE within PFCP Session Modification Request

| Octet 1 and 2 |  | Update URR IE Type = 13 (decimal) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Octets 3 and 4 |  | Length = n |  |  |  |  |  |
| Information elements | P | Condition / Comment | Appl. |  |  |  | IE Type |
|  |  |  | Sxa | Sxb | Sxc | N4 |  |
| URR ID | M | This IE shall uniquely identify the URR among all the URRs configured for that PFCP session | X | X | X | X | URR ID |
| Measurement Method | C | This IE shall be present if the measurement method needs to be modified. When present, this IE shall indicate the method for measuring the network resources usage, i.e. whether the data volume, duration (i.e. time), combined volume/duration, or event shall be measured. | X | X | X | X | Measurement Method |
| Reporting Triggers | C | This IE shall be present if the reporting triggers needs to be modified. When present, this IE shall indicate the trigger(s) for reporting network resources usage to the CP function, e.g. periodic reporting or reporting upon reaching a threshold, or envelope closure. | X | X | X | X | Reporting Triggers |
| Measurement Period | C | This IE shall be present if the Measurement Period needs to be modified. When present, it shall indicate the period for generating and reporting usage reports. | X | X | X | X | Measurement Period |
| Volume Threshold | C | This IE shall be present if the Volume Threshold needs to be modified. When present, it shall indicate the traffic volume value after which the UP function shall report network resources usage to the CP function for this URR. | X | X | X | X | Volume Threshold |
| Volume Quota | C | This IE shall be present if the Volume Quota needs to be modified. When present, it shall indicate the Volume Quota value. | - | X | X | X | Volume Quota |
| Time Threshold | C | This IE shall be present if the Time Threshold needs to be modified. When present, it shall indicate the time usage after which the UP function shall report network resources usage to the CP function for this URR. | X | X | X | X | Time Threshold |
| Time Quota | C | This IE shall be present if the Time Quota needs to be modified. When present, it shall indicate the Time Quota value. | - | X | X | X | Time Quota |
| Quota Holding Time | C | This IE shall be present if the Quota Holding Time needs to be modified. When present, it shall contain the duration of the Quota Holding Time. | - | X | X | X | Quota Holding Time |
| Dropped DL Traffic Threshold | C | This IE shall be present if the Dropped DL Threshold needs to be modified. When present, it shall contain the threshold of the DL traffic being dropped. | X | - | - | X | Dropped DL Traffic Threshold |
| Monitoring Time | C | This IE shall be present if the Monitoring Time needs to be modified. When present, this IE shall contain the time at which the UP function shall re-apply the volume or time threshold. | - | X | X | X | Monitoring Time |
| Subsequent Volume Threshold | C | This IE shall be present if the Subsequent Volume Threshold needs to be modified and volume-based measurement is used. When present, it shall indicate the traffic volume value after which the UP function shall report network resources usage to the CP function for this URR for the period after the Monitoring Time. | - | X | X | X | Subsequent Volume Threshold |
| Subsequent Time Threshold | C | This IE shall be present if the Subsequent Time Threshold needs to be modified. When present, it shall indicate the time usage value after which the UP function shall report network resources usage to the CP function for this URR for the period after the Monitoring Time. | - | X | X | X | Subsequent Time Threshold |
| Subsequent Volume Quota | C | This IE shall be present if the Subsequent Volume Quota needs to be modified. When present, it shall indicate the Volume Quota value which the UP function shall use for this URR for the period after the Monitoring Time. | - | X | X | X | Subsequent Volume Quota |
| Subsequent Time Quota | C | This IE shall be present if the Subsequent Time Quota needs to be modified. When present, it shall indicate the Time Quota value which the UP function shall use for this URR for the period after the Monitoring Time. | - | X | X | X | Subsequent Time Quota |
| Inactivity Detection Time | C | This IE shall be present if the Inactivity Detection Time needs to be modified. When present, it shall indicate the duration of the inactivity period after which time measurement needs to be suspended when no packets are received during this inactivity period. | - | X | X | X | Inactivity Detection Time |
| Linked URR ID | C | This IE shall be present if linked usage reporting is required. When present, this IE shall contain the linked URR ID which is related with this URR (see subclause 5.2.2.4). Several IEs with the same IE type may be present to represent multiple linked URRs which are related with this URR. | - | X | X | X | Linked URR ID |
| Measurement Information | C | This IE shall be included if any of the following flag is set to 1. Applicable flags are: - Inactive Measurement Flag: this flag shall be set to 1 if the measurement shall be paused (inactive). The measurement shall be performed (active) if the bit is set to 0 or if the Measurement Information IE is not present in the Update URR IE. - Reduced Application Detection Information Flag: this flag may be set to 1, if the Reporting Triggers request to report the start or stop of application, to request the UP function to only report the Application ID in the Application Detection Information, e.g. for envelope reporting. | - | X | - | X | Measurement Information |
| Time Quota Mechanism | C | This IE shall be present if time-based measurement based on CTP or DTP needs to be modified. | - | X | - | - | Time Quota Mechanism |
| Aggregated URRs | C | This IE shall be included if the Aggregated URRs IE needs to be modified. See Table 7.5.2.4-1. Several IEs with the same IE type may be present to provision multiple aggregated URRs. When present, this IE shall provide the complete list of the aggregated URRs. | - | X | - |  | Aggregated URRs |
| FAR ID for Quota Action | C | This IE shall be present if the FAR ID for Quota Action IE needs to be modified. This IE may be present if the Volume Quota IE or the Time Quota IE is newly provisioned in the URR and the UP Function indicated support of the Quota Action. When present, it shall contain the identifier of the substitute FAR the UP function shall apply, for the traffic associated to this URR, when exhausting any of these quotas. See NOTE 1. | - | X | X | X | FAR ID |
| Ethernet Inactivity Timer | C | This IE shall be present if the Ethernet Inactivity Timer needs to be modified. When present, it shall contain the duration of the Ethernet inactivity period. | - | - | - | X | Ethernet Inactivity Timer |
| Additional Monitoring Time | O | This IE shall be present if the additional Monitoring Time needs to be modified. When present, this IE shall contain the time at which the UP function shall re-apply the volume or time threshold/quota. See Table 7.5.2.4-3. | X | X | X | X | Additional Monitoring Time |
| NOTE 1: The substitute FAR used when exhausting a Volume Quota or Time Quota may be set to drop the packets or redirect the traffic towards a redirect destination as specified in subclause 5.4.7. |  |  |  |  |  |  |  |

* * * Next Change * * * *

### 8.1.2	Information Element Types

A PFCP message may contain several IEs. In order to have forward compatible type definitions for the PFCP IEs, all of them shall be TLV (Type, Length, Value) coded. PFCP IE type values are specified in the Table 8.1.2-1. The last column of this table indicates whether the IE is:

-	Fixed Length: the IE has a fixed set of fields, and a fixed number of octets;

-	Variable Length: the IE has a fixed set of fields, and has a variable number of octets.
For example, the last octets may be numbered similar to "5 to (n+4)". In this example, if the value of the length field, n, is 0, then the last field is not present;

-	Extendable: the IE has a variable number of fields, and has a variable number of octets.
The last fields are typically specified with the statement: "These octet(s) is/are present only if explicitly specified". The legacy receiving entity shall ignore the unknown octets.

An IE of any of the above types may have a null length as specified in subclause 5.6.3. This shall not be considered as an error by the receiving PFCP entity.

In order to improve the efficiency of troubleshooting, it is recommended that the IEs should be arranged in the signalling messages as well as in the grouped IEs, according to the order the IEs are listed in the message definition table or grouped IE definition table in section 7. However the receiving entity shall be prepared to handle the messages with IEs in any order.

Within IEs, certain fields may be described as spare. These bits shall be transmitted with the value set to 0. To allow for future features, the receiver shall not evaluate these bits.

# Table 8.1.2-1: Information Element Types

| IE Type value (Decimal) | Information elements | Comment / Reference | Number of Fixed Octets |
| --- | --- | --- | --- |
| 0 | Reserved |  |  |
| 1 | Create PDR | Extendable / Table 7.5.2.2-1 | Not Applicable |
| 2 | PDI | Extendable / Table 7.5.2.2-2 | Not Applicable |
| 3 | Create FAR | Extendable / Table 7.5.2.3-1 | Not Applicable |
| 4 | Forwarding Parameters | Extendable / Table 7.5.2.3-2 | Not Applicable |
| 5 | Duplicating Parameters | Extendable / Table 7.5.2.3-3 | Not Applicable |
| 6 | Create URR | Extendable / Table 7.5.2.4-1 | Not Applicable |
| 7 | Create QER | Extendable / Table 7.5.2.5-1 | Not Applicable |
| 8 | Created PDR | Extendable / Table 7.5.3.2-1 | Not Applicable |
| 9 | Update PDR | Extendable / Table 7.5.4.2-1 | Not Applicable |
| 10 | Update FAR | Extendable / Table 7.5.4.3-1 | Not Applicable |
| 11 | Update Forwarding Parameters | Extendable / Table 7.5.4.3-2 | Not Applicable |
| 12 | Update BAR (PFCP Session Report Response) | Extendable / Table 7.5.9.2-1 | Not Applicable |
| 13 | Update URR | Extendable / Table 7.5.4.4 | Not Applicable |
| 14 | Update QER | Extendable / Table 7.5.4.5 | Not Applicable |
| 15 | Remove PDR | Extendable / Table 7.5.4.6 | Not Applicable |
| 16 | Remove FAR | Extendable / Table 7.5.4.7 | Not Applicable |
| 17 | Remove URR | Extendable / Table 7.5.4.8 | Not Applicable |
| 18 | Remove QER | Extendable / Table 7.5.4.9 | Not Applicable |
| 19 | Cause | Fixed / Subclause 8.2.1 | 1 |
| 20 | Source Interface | Extendable / Subclause 8.2.2 | 1 |
| 21 | F-TEID | Extendable / Subclause 8.2.3 | q-4 |
| 22 | Network Instance | Variable Length / Subclause 8.2.4 | Not Applicable |
| 23 | SDF Filter | Extendable / Subclause 8.2.5 | v+2-4 |
| 24 | Application ID | Variable Length / Subclause 8.2.6 | Not Applicable |
| 25 | Gate Status | Extendable / Subclause 8.2.7 | 1 |
| 26 | MBR | Extendable / Subclause 8.2.8 | 10 |
| 27 | GBR | Extendable / Subclause 8.2.9 | 10 |
| 28 | QER Correlation ID | Extendable / Subclause 8.2.10 | 4 |
| 29 | Precedence | Extendable / Subclause 8.2.11 | 4 |
| 30 | Transport Level Marking | Extendable / Subclause 8.2.12 | 2 |
| 31 | Volume Threshold | Extendable /Subclause 8.2.13 | q+7-4 |
| 32 | Time Threshold | Extendable /Subclause 8.2.14 | 4 |
| 33 | Monitoring Time | Extendable /Subclause 8.2.15 | 4 |
| 34 | Subsequent Volume Threshold | Extendable /Subclause 8.2.16 | q+7-4 |
| 35 | Subsequent Time Threshold | Extendable /Subclause 8.2.17 | 4 |
| 36 | Inactivity Detection Time | Extendable /Subclause 8.2.18 | 4 |
| 37 | Reporting Triggers | Extendable /Subclause 8.2.19 | 2 |
| 38 | Redirect Information | Extendable /Subclause 8.2.20 | 8+a-4 |
| 39 | Report Type | Extendable / Subclause 8.2.21 | 1 |
| 40 | Offending IE | Fixed / Subclause 8.2.22 | 2 |
| 41 | Forwarding Policy | Extendable / Subclause 8.2.23 | k-4 |
| 42 | Destination Interface | Extendable / Subclause 8.2.24 | 1 |
| 43 | UP Function Features | Extendable / Subclause 8.2.25 | 1 |
| 44 | Apply Action | Extendable / Subclause 8.2.26 | 1 |
| 45 | Downlink Data Service Information | Extendable / Subclause 8.2.27 | 1 |
| 46 | Downlink Data Notification Delay | Extendable / Subclause 8.2.28 | 1 |
| 47 | DL Buffering Duration | Extendable / Subclause 8.2.29 | 1 |
| 48 | DL Buffering Suggested Packet Count | Variable / Subclause 8.2.30 | Not Applicable |
| 49 | PFCPSMReq-Flags | Extendable / Subclause 8.2.31 | 1 |
| 50 | PFCPSRRsp-Flags | Extendable / Subclause 8.2.32 | 1 |
| 51 | Load Control Information | Extendable / Table 7.5.3.3-1 | Not Applicable |
| 52 | Sequence Number | Fixed Length / Subclause 8.2.33 | 4 |
| 53 | Metric | Fixed Length / Subclause 8.2.34 | 1 |
| 54 | Overload Control Information | Extendable / Table 7.5.3.4-1 | Not Applicable |
| 55 | Timer | Extendable / Subclause 8.2 35 | 1 |
| 56 | Packet Detection Rule ID | Extendable / Subclause 8.2 36 | 2 |
| 57 | F-SEID | Extendable / Subclause 8.2 37 | p+15-4 |
| 58 | Application ID's PFDs | Extendable / Table 7.4.3.1-2 | Not Applicable |
| 59 | PFD context | Extendable / Table 7.4.3.1-3 | Not Applicable |
| 60 | Node ID | Extendable / Subclause 8.2.38 | o-4 |
| 61 | PFD contents | Extendable / Subclause 8.2.39 | v-4 |
| 62 | Measurement Method | Extendable / Subclause 8.2.40 | 1 |
| 63 | Usage Report Trigger | Extendable / Subclause 8.2.41 | 1 |
| 64 | Measurement Period | Extendable / Subclause 8.2.42 | 4 |
| 65 | FQ-CSID | Extendable / Subclause 8.2.43 | (q+1)-4 |
| 66 | Volume Measurement | Extendable / Subclause 8.2.44 | q+7-4 |
| 67 | Duration Measurement | Extendable / Subclause 8.2.45 | 4 |
| 68 | Application Detection Information | Extendable / Table 7.5.8.3-2 | Not Applicable |
| 69 | Time of First Packet | Extendable / Subclause 8.2.46 | 4 |
| 70 | Time of Last Packet | Extendable / Subclause 8.2.47 | 4 |
| 71 | Quota Holding Time | Extendable / Subclause 8.2.48 | 4 |
| 72 | Dropped DL Traffic Threshold | Extendable / Subclause 8.2.49 | m+7-4 |
| 73 | Volume Quota | Extendable / Subclause 8.2.50 | q+7-4 |
| 74 | Time Quota | Extendable / Subclause 8.2.51 | 4 |
| 75 | Start Time | Extendable / Subclause 8.2.52 | 4 |
| 76 | End Time | Extendable / Subclause 8.2.53 | 4 |
| 77 | Query URR | Extendable / Table 7.5.4.10-1 | Not Applicable |
| 78 | Usage Report (in Session Modification Response) | Extendable / Table 7.5.5.2-1 | Not Applicable |
| 79 | Usage Report (Session Deletion Response) | Extendable / Table 7.5.7.2-1 | Not Applicable |
| 80 | Usage Report (Session Report Request) | Extendable / Table 7.5.8.3-1 | Not Applicable |
| 81 | URR ID | Extendable / Subclause 8.2.54 | 4 |
| 82 | Linked URR ID | Extendable / Subclause 8.2.55 | 4 |
| 83 | Downlink Data Report | Extendable / Table 7.5.8.2-1 | Not Applicable |
| 84 | Outer Header Creation | Extendable / Subclause 8.2.56 | r+1-4 |
| 85 | Create BAR | Extendable / Table 7.5.2.6-1 | Not Applicable |
| 86 | Update BAR (Session Modification Request) | Extendable / Table 7.5.4.11-1 | Not Applicable |
| 87 | Remove BAR | Extendable / Table 7.5.4.12-1 | Not Applicable |
| 88 | BAR ID | Extendable / Subclause 8.2.57 | 1 |
| 89 | CP Function Features | Extendable / Subclause 8.2.58 | 1 |
| 90 | Usage Information | Extendable / Subclause 8.2.59 | 1 |
| 91 | Application Instance ID | Variable Length / Subclause 8.2.60 | Not Applicable |
| 92 | Flow Information | Extendable / Subclause 8.2.61 | m-4 |
| 93 | UE IP Address | Extendable / Subclause 8.2.62 | p+15-1 |
| 94 | Packet Rate | Extendable / Subclause 8.2.63 | p+2-4 |
| 95 | Outer Header Removal | Extendable / Subclause 8.2.64 | 1 |
| 96 | Recovery Time Stamp | Extendable / Subclause 8.2.65 | 4 |
| 97 | DL Flow Level Marking | Extendable / Subclause 8.2.66 | p+1-4 |
| 98 | Header Enrichment | Extendable / Subclause 8.2.67 | q-4 |
| 99 | Error Indication Report | Extendable / Table 7.5.8.4-1 | Not Applicable |
| 100 | Measurement Information | Extendable / Subclause 8.2.68 | 1 |
| 101 | Node Report Type | Extendable / Subclause 8.2.69 | 1 |
| 102 | User Plane Path Failure Report | Extendable / Table 7.4.5.1.2-1 | Not Applicable |
| 103 | Remote GTP-U Peer | Extendable / Subclause 8.2.70 | p+15-4 |
| 104 | UR-SEQN | Fixed Length / Subclause 8.2.71 | 4 |
| 105 | Update Duplicating Parameters | Extendable / Table 7.5.4.3-3 | Not Applicable |
| 106 | Activate Predefined Rules | Variable Length / Subclause 8.2.72 | Not Applicable |
| 107 | Deactivate Predefined Rules | Variable Length / Subclause 8.2.73 | Not Applicable |
| 108 | FAR ID | Extendable / Subclause 8.2.74 | 4 |
| 109 | QER ID | Extendable / Subclause 8.2.75 | 4 |
| 110 | OCI Flags | Extendable / Subclause 8.2.76 | 1 |
| 111 | PFCP Association Release Request | Extendable / Subclause 8.2.77 | 1 |
| 112 | Graceful Release Period | Extendable / Subclause 8.2.78 | 1 |
| 113 | PDN Type | Extendable / Subclause 8.2.79 | 1 |
| 114 | Failed Rule ID | Extendable / Subclause 8.2.80 | p-4 |
| 115 | Time Quota Mechanism | Extendable / Subclause 8.2.81 | 5 |
| 116 | User Plane IP Resource Information | Extendable / Subclause 8.2.82 | l+1-4 |
| 117 | User Plane Inactivity Timer | Extendable /Subclause 8.2.83 | 4 |
| 118 | Aggregated URRs | Extendable / Table 7.5.2.4-2 | Not Applicable |
| 119 | Multiplier | Fixed / Subclause 8.2.84 | 12 |
| 120 | Aggregated URR ID | Fixed / Subclause 8.2.85 | 4 |
| 121 | Subsequent Volume Quota | Extendable / Subclause 8.2.86 | q+7-4 |
| 122 | Subsequent Time Quota | Extendable / Subclause 8.2.87 | 4 |
| 123 | RQI | Extendable / Subclause 8.2.88 | 1 |
| 124 | QFI | Extendable / Subclause 8.2.89 | m-4 |
| 125 | Query URR Reference | Extendable / Subclause 8.2.90 | 4 |
| 126 | Additional Usage Reports Information | Extendable / Subclause 8.2.91 | 2 |
| 127 | Create Traffic Endpoint | Extendable / Table 7.5.2.7 | Not Applicable |
| 128 | Created Traffic Endpoint | Extendable / Table 7.5.3.5 | Not Applicable |
| 129 | Update Traffic Endpoint | Extendable / Table 7.5.4.13 | Not Applicable |
| 130 | Remove Traffic Endpoint | Extendable / Table 7.5.4.14 | Not Applicable |
| 131 | Traffic Endpoint ID | Extendable / Subclause 8.2.92 | 1 |
| 132 | Ethernet Packet Filter | Extendable / Table 7.5.2.2-3 | Not Applicable |
| 133 | MAC address | Extendable / Subclause 8.2.93 | s-1-4 |
| 134 | C-TAG | Extendable / Subclause 8.2.94 | 3 |
| 135 | S-TAG | Extendable / Subclause 8.2.95 | 3 |
| 136 | Ethertype | Extendable / Subclause 8.2.96 | 2 |
| 137 | Proxying | Extendable / Subclause 8.2.97 | 1 |
| 138 | Ethernet Filter ID | Extendable / Subclause 8.2.98 | 4 |
| 139 | Ethernet Filter Properties | Extendable / Subclause 8.2.99 | 1 |
| 140 | Suggested Buffering Packets Count | Extendable / Subclause 8.2.100 |  |
| 141 | User ID | Extendable / Subclause 8.2.101 | d-1-4 |
| 142 | Ethernet PDU Session Information | Extendable / Subclause 8.2.102 | 1 |
| 143 | Ethernet Traffic Information | Extendable / Table 7.5.8.3-3 | Not Applicable |
| 144 | MAC Addresses Detected | Extendable / Subclause 8.2.103 | 7 |
| 145 | MAC Addresses Removed | Extendable / Subclause 8.2.104 | 7 |
| 146 | Ethernet Inactivity Timer | Extendable / Subclause 8.2.105 | 4 |
| xx | Additional Monitoring Time | Extendable / Table 7.5.2.4-3 | Not Applicable |
| yy to 65535 | Spare. For future use. |  |  |

* * * End of Changes * * * *