3GPP TSG-CT WG6 Meeting #101e	C6-200618_DRAFT

# E-meeting; 25th – 28th August 2020

# Title:	Reply LS to GSMA regarding 3GPP TS 31.130 specification

# Response to:	LS C6-200540 regarding 3GPP TS 31.130 specification

# Release:	Release 15, Release 16

Source:	CT6

To:	TSG eSIMTP

Cc:	ETSI SCP TEC ????

Contact Person:

Name:	Sofia Massascusa

E-mail Address:	sofia.massascusa@st.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

CT6 thanks GSMA for their Liaison Statement regarding the issue highlighted for the UICC Conformance testing.

CT6 group analyzed the request and they discovered some issues defining these new events.

TS 31.130 and TS 102 241 usually define events not dedicated to test purpose, but events dedicated to applications on the field that implement services required by operators or third parties.

Defining new events as standard events implies that the UICC has to introduce and manage the events at operating system level, not only for testing purpose.

For the requested events READ BINARY EVENT and READ RECORD EVENT, it is necessary to think about how many times an applet registered on can be triggered, because the number of time files are read could wake up several applets, generating delay on card response, especially at activation time.

2. Actions: CT6 recommends TSG eSIMTP and TCA eUICC to take the above information into account.

3. Date of Next CT6 Meetings:

3GPP TSG CT6#102-e	???