| ETSI TC SCP TEC Meeting #32 Sophia Antipolis, France, 6-9 Apr 2010 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Tdoc SCPTEC(10)0171 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | EP SCP CR-Form-v1.7.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 102 223 |  |  |  | CR | 194 |  |  |  rev | - |  |  |  Current version: |  |  |  |  | 8.5.0 |  |  |  |
|  | For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Proposed change affects:  |  |  |  |  | smart card |  | X | terminal |  |  |  | X | server / network entity |  |  |  |  |  |  |  |  |
|  | Title:  |  | Correction to NMR multiple technologies |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Source:  |  | SCP TEC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Work item name:  |  | TEI8 |  |  |  |  |  |  |  |  |  |  |  | Date:  |  | 06/04/2010 |  |  |  |  |  |
|  | Category:  |  | F |  |  |  |  |  |  |  |  |  |  |  | Release:  |  | Rel-8 |  |  |  |  |  |
|  |  |  | Use one of the following categories: F (essential correction) A (corresponds to a correction in an earlier release) B (Addition of feature), C (Functional modification of feature) D (Editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) REL-4 (Release 4) REL-5 (Release 5) REL-6 (Release 6) REL-7 (Release 7) REL-8 (Release 8) REL-9 (Release 9) |  |  |  |  |  |  |
|  | Reason for change:  |  |  | In the description of Provide Local Information, Network Measurement Result for multiple access technologies, it is stated that TERMINAL RESPONSE will contain a BCCH channel list data object per access technology requested. However, there can be several NMR data objects but only one BCCH channel list data object in the TERMINAL RESPONSE, since it is only applicable for GERAN. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Summary of change:  |  |  | Clarify descriptive text in clause 6.8.7, that the presence of a BCCH channel list data object is conditional. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Consequences if  not approved: |  |  | Descriptive text in 6.8.7 misleading, can lead to wrong implementation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | New tag value defined within the CR? |  |  |  | If ticked, add document number of related CR to TS 101 220: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Clauses affected:  |  |  | 6.8.7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Other specs  |  |  |  | Other core specifications  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Affected: |  |  |  | Test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Other comments:  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.8.7	Local information

When the terminal issues a successful TERMINAL RESPONSE for a PROVIDE LOCAL INFORMATION command, it shall supply the requested local information:

- Where the UICC has requested location information, TERMINAL RESPONSE shall contain the location information data object.

- Where the UICC has requested location information for multiple access technologies, TERMINAL RESPONSE shall contain the Access Technology data object listing all current access technologies, followed by one location information data object for each current access technology in the same sequence. If no location information is available for an access technology, the respective data object shall have length zero.

- Where the UICC has requested the IMEI, TERMINAL RESPONSE shall contain the IMEI data object.

- Where the UICC has requested the Network Measurement Results the TERMINAL RESPONSE shall contain the NMR data object and the BCCH channel list data object if supported by the network access technology.

- Where the UICC has requested the Network Measurement Results for multiple access technologies, TERMINAL RESPONSE shall contain the Access Technology data object listing all current access technologies, followed by one NMR data object for each current access technology in the same sequence. If supported by a network access technology, a BCCH channel list data object shall immediately follow the NMR data object. If no NMR data or no BCCH channel list is available for an access technology, the respective data object shall have length zero.

- Where the UICC has requested the date, time and time zone the TERMINAL RESPONSE shall contain the Date-Time and Time zone data object.

- Where the UICC has requested the currently used language, the TERMINAL RESPONSE shall contain the Language data object.

- Where the UICC has requested the Timing Advance, the TERMINAL RESPONSE shall contain the Timing Advance data object if supported by the network access technology.

- Where the UICC has requested the Battery State, the TERMINAL RESPONSE shall contain the Battery State data object (if class "g" is supported).

- Where the UICC has requested the Access Technology (single access technology), the TERMINAL RESPONSE shall contain the Access Technology data object with length set to 1. If the terminal is currently connected via more than one technology, it shall select the most appropriate value.

- Where the UICC has requested Multiple Access Technologies, the TERMINAL RESPONSE shall include all access technologies that the terminal is currently connected to.

- Where the UICC has requested the ESN, TERMINAL RESPONSE shall contain the ESN data object.

- Where the UICC has requested the IMEISV, TERMINAL RESPONSE shall contain the IMEISV data object.

- Where the UICC has requested the Search Mode information, the TERMINAL RESPONSE shall contain the Search Mode data object.

- Where the UICC has requested the MEID, TERMINAL RESPONSE shall contain the MEID data object.

- Where the UICC has requested broadcast network information, TERMINAL RESPONSE shall contain the broadcast network information data object (if class "o" is supported).