3GPP TSG-CT6 Meeting #89bis	C6-180357

- Sophia-Antipolis, France, 10 - 13 July 2018

SA WG2 Meeting #128	S2-187599

2-6 July 2018, Vilnius, Lithuania	(was S2-186664)

Title:	Reply LS on GSMA question on 5G network slices

Reply to:	LS on S2-186367/GSMA (18SIMSG23_Doc_004r2)

Release:	Release 15

Work Item:	5GS_Ph1

Source:	SA2

To:	GSMA SIM Group

Cc:	CT6, SA3

Contact person:	Stefano Faccin

sfaccin@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: N/A

# 1	Overall description

SA2 thanks GSMA SIM Group for their LS on the need for re-authentication at the change of NSSAI.

GSMA SIM Group question: The SIM Group kindly requests 3GPP SA3/SA2/CT6 to clarify whether a change of NSSAI may force a re-authentication of the UE and subscriber. If so, under which circumstances this is likely to happen.

SA2 would like to indicate that a change of NSSAI does not force a re-authentication of UE and subscriber:

-	In release 15, the UE authenticates with the serving network via the AMF e.g. during a Registration Procedure. Within a PLMN, at any given point in time the UE is served by a single AMF, and a single 5G security context exists.

-	The UE may receive a new NSSAI (called Allowed NSSAI) in the Registration Procedure, or be provided an Allowed NSSAI during a UE Configuration Update procedure. In either case, UE re-authentication is not required in order for the network to provide a new Allowed NSSAI to the UE, although an authentication could be run if it was desired by the serving network.

-	The 5GS specifications allow for an AMF to re-authenticate a UE at AMF change.

# 2	Actions

To GSMA

ACTION: 	SA2 kindly asks GSMA to consider the answers above.

# 3	Dates of next TSG SA WG2 meetings

TSG SA WG2 Meeting 128bis	20 – 24 August 2018	Sophia-Antipolis, France

TSG SA WG2 Meeting 129	15 – 19 October 2018	Dongguan, China