3GPP TSG-CT Meeting #102	CP-233312

# Edinburgh, United Kingdom, 11th Dec 2023 - 12th Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 31.124 | CR | 0724 | rev | 2 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | x | ME | x | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction required for TC 27.22.14.2 (Seq 2.3) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple AB Denmark, China telecom, Datang Linktester Technology Co., Ltd., Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Comprion GmbH, Apple |  |  |  |  |  |  |  |  |  |
| Work item code: | UEConTest_R17 |  |  |  |  |  | Date: |  |  | 2023-12-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The present implementation of TC does not take into account that if the SET UP EVENT LIST Location status occurs before a device successfuly registers, the device will send its current state in ENVELOPE: EVENT DOWNLOAD - Location Status before a device registers. This will interfere with the expectation of the test case at Step 8. Hence an additional ENVELOPE: EVENT DOWNLOAD - Location Status needs to be handled if sent. NOTE: see TS 31.111, clause 7.5.4 for further explanation. Additionally in Clause 3.4 the Applicabilty Table needs to be corrected to have test sequence 2.3 listed before seq 2.4 for this TC 27.22.14.2. The proposed implementation in the agreed CR C6-230681 is not considering that the device could have registered to NG-RAN cell 1 before the ENVELOPE: EVENT DOWNLOAD – Location Status 2.3.0 is expected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Applicabilty Table is corrected to have test sequence 2.3 listed before seq 2.4 for this TC 27.22.14.2 Optional step 6b is added to handle ENVELOPE: EVENT DOWNLOAD - Location Status (no service) in case the device hasn’t already successfully registered to the network. Other minor typo corrections |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Devices that support sending location status and access technology that is already available will unfairly fail test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.4, 27.22.14.2 (seq 2.3, seq 2.4) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | also merging C6-230643 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Is revision of C6-230681, was C6-230638 |  |  |  |  |  |  |  |  |

[…]

***** start of changes *****

## 3.4	Applicability table

NOTE:	It is possible that the applicability of some tests indicated in table B.1 does not match with the value in the Release column, due to late definition of the test sequences. Tests should be performed without considering the Release column, but only based on the conditions indicated for each release.

…

| Test | Test sequence | Description | from Rel. | up to Rel. (see note 1) | Applicability | Terminal Profile | Network Dependency | Support | Additional test case execution parameter |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27.22.1 |  | PROFILE DOWNLOAD |  |  |  |  |  |  |  |
|  | 1 | PROFILE DOWNLOAD | R99 |  | M | E.1/1 | No |  |  |
| 27.22.2 |  | Contents of the TERMINAL PROFILE command |  |  |  |  |  |  |  |
| … |  |  |  |  |  |  |  |  |  |
| 27.22.14.2 | 2.1 | SMS-PP Data Download after Steering of Roaming via DL NAS TRANSPORT message with REFRESH command [Steering of Roaming] | Rel-15 |  | C231 | E.1/24 AND E.1/2 | NG-SS only |  |  |
| 27.22.14.2 | 2.2 | Void | Rel-16 |  |  |  |  |  |  |
| 27.22.14.2 | 2.3 | Steering of Roaming via DL NAS TRANSPORT message with "Acknowledgement requested" and REFRESH command [Steering of Roaming] | Rel-15 |  | C231 | E.1/24 AND E.1/2 | NG-SS only |  |  |
| 27.22.14.2 | 2.4 | SMS-PP Data Download in several ENVELOPE commands after Steering of Roaming via DL NAS TRANSPORT long message with REFRESH command [Steering of Roaming] | Rel-16 |  | C231 | E.1/24 AND E.1/2 | NG-SS only |  |  |
| 27.22.14.3 | 3.1 | SMS-PP Data Download after Steering of Roaming via REGISTRATION ACCEPT message with REFRESH command [Steering of Roaming] | Rel-15 |  | C231 | E.1/24 AND E.1/2 | NG-SS only |  |  |
| 27.22.14.3 | 3.2 | Void | Rel-15 |  |  |  |  |  |  |
| 27.22.14.3 | 3.3 | SMS-PP Data Download after Steering of Roaming via REGISTRATION ACCEPT long message with REFRESH command [Steering of Roaming] | Rel-15 |  | C231 | E.1/24 AND E.1/2 | NG-SS only |  |  |
| NOTE: For Rel-13 if the UE supports NB-IoT, this test case shall be verified by accessing the NB System Simulator (NB-SS). NOTE 1: Blank entries indicate the latest valid release at the time of publication of this specification. |  |  |  |  |  |  |  |  |  |

***** second change *****

#### 27.22.14.2	Steering of Roaming via DL NAS TRANSPORT message

[…]

Expected Sequence 2.3: (Steering of Roaming via DL NAS TRANSPORT message with "Acknowledgement requested" and REFRESH command [Steering of Roaming])

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | NG-SS | NG-RAN Cell 1 and NG-RAN Cell 2 transmit BCCH. |  |
| 2 | USER ME | The ME is switched on | ME will perform Profile Download and USIM initialisation |
| 3 | UICC ME | PROACTIVE COMMAND PENDING: SET UP EVENT LIST 2.3.1 | If programmable non-removable UICC with a Test Applet is used (as defined in clause 27.0), the TERMINAL RESPONSE cannot be verified by the Test Applet and that the LOCATION STATUS Event has been successfully registered in the device after step 5 is implicitly verified at steps 8 and 19. |
| 4 | ME UICC | FETCH |  |
| 5 | UICC ME | PROACTIVE COMMAND: SET UP EVENT LIST 2.3.1 |  |
| 6a | ME UICC | TERMINAL RESPONSE: SET UP EVENT LIST 2.3.1 |  |
| 6b | ME UICC | ENVELOPE: EVENT DOWNLOAD - Location Status 2.3.0 | This step is optional and applies in case the ME did not successfully register to NG-RAN cell 1 yet. |
| 7 | ME NG-SS | The ME successfully registers to NG-RAN cell 1 |  |
| 8 | ME UICC | ENVELOPE: EVENT DOWNLOAD - Location Status 2.3.1 |  |
| 9 | NG-SS ME | DL NAS TRANSPORT message 2.3.1 | SOR header with: - ACK set to "acknowledgement requested" - List Type set to "secured packet" |
| 10 | ME UICC | ENVELOPE: SMS-PP DOWNLOAD 2.3.1 | the ME shall pass the message transparently to the UICC using the ENVELOPE (SMS-PP DOWNLOAD) command as specified in TS 31.111 [15] clause 7.1.1.1a |
| 11 | UICC ME | SW1/SW2 '91 XX' |  |
| 12 | ME NG-SS | ME sends to NG-SS an acknowledgement in the Payload container IE of an UL NAS TRANSPORT message with Payload type IE set to "Steering of Roaming transparent container". | Note: this step can be performed in parallel or any point after step 13 and before step 18. [SOR transparent container 2.3.1 with Acknowledgement] |
| 13 | ME UICC | FETCH |  |
| 14 | UICC ME | PROACTIVE COMMAND: REFRESH 2.3.1 [Steering of Roaming] |  |
| 15 | ME → UICC | Update of EFFPLMN | [Deletion of the entry with PLMN 254/003] |
| 16 | ME UICC | TERMINAL RESPONSE: REFRESH 2.3.1 |  |
| 17 | UICC ME | Proactive UICC session is terminated |  |
| 18 | ME NG-SS | The ME successfully registers to NG-RAN cell 2 within 6 minutes | Note: The ME might have registered to the Cell 2 before this step |
| 19 | ME → UICC | ENVELOPE: EVENT DOWNLOAD - Location Status 2.3.2 | PLMN MCC/MNC 254/003, Normal service |

DL NAS TRANSPORT message 2.3.1

Logically:

Message details (referring to TS 24.501, Figure 9.11.3.51.1)

Payload container type IE:	"0100" (SOR transparent container)

SOR header:

SOR data type:		"0" (SOR transparent container carries steering of roaming information)

List indication:		"1" (list of preferred PLMN/access technology combinations is provided)

List type:				"0" (The list type is a secured packet.)

ACK:					"1" (acknowledgment requested)

Secured packet:		as specified in TS 31.111 [15] clause 7.1.1.1a – TPDU Command Packet

Coding:				(Security payload with 254/003 and 254/004 included in the NG-RAN PLMN List)

Coding:

| 40 | 00 | 91 | 7F | F6 | 00 | 00 | 00 | 00 | 00 | 00 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4E | 02 | 70 | 00 | 00 | 49 | 15 | 02 | 00 | 10 | 10 | B0 |
| 01 | 40 | 00 | 00 | 00 | 00 | 00 | 00 | 64 | D8 | 5D | E1 |
| 35 | EB | A6 | BF | AA | 31 | 22 | 07 | 00 | A4 | 00 | 04 |
| 02 | 6F | 61 | 22 | 0F | 00 | D6 | 00 | 00 | 0A | 52 | 34 |
| 00 | 08 | 00 | 52 | 44 | 00 | 08 | 00 | 81 | 15 | 81 | 03 |
| 01 | 01 | 07 | 82 | 02 | 81 | 82 | 72 | 0A | 52 | 34 | 00 |
| 08 | 00 | 52 | 44 | 00 | 08 | 00 |

SOR Transparent container in REGISTRATION REQUEST (Acknowledgement) 2.3.1

Logically:

Payload container details (referring to TS 24.501 Figure 9.11.3.51.4 and 9.11.3.51.6)

Payload container type IE:	"0100" (SOR transparent container)

SOR header:

SOR data type:			"1" (The SOR transparent container carries acknowledgement of successful

reception of the steering of roaming information)

ENVELOPE: SMS-PP DOWNLOAD 2.3.1

Logically:

SMS-PP Download

Device identities:

Source device:		Network

Destination device:	UICC

SMS TPDU:				Contents of Secured Packet from DL NAS TRANSPORT message 2.3.1

Coding:

| BER-TLV: | D1 | 61 | 82 | 02 | 83 | 81 | 8B | 5B | 40 | 00 | 91 | 7F |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | F6 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 4E | 02 | 70 | 00 |
|  | 00 | 49 | 15 | 02 | 00 | 10 | 10 | B0 | 01 | 40 | 00 | 00 |
|  | 00 | 00 | 00 | 00 | 64 | D8 | 5D | E1 | 35 | EB | A6 | BF |
|  | AA | 31 | 22 | 07 | 00 | A4 | 00 | 04 | 02 | 6F | 61 | 22 |
|  | 0F | 00 | D6 | 00 | 00 | 0A | 52 | 34 | 00 | 08 | 00 | 52 |
|  | 44 | 00 | 08 | 00 | 81 | 15 | 81 | 03 | 01 | 01 | 07 | 82 |
|  | 02 | 81 | 82 | 72 | 0A | 52 | 34 | 00 | 08 | 00 | 52 | 44 |
|  | 00 | 08 | 00 |

PROACTIVE COMMAND: SET UP EVENT LIST 2.3.1

Logically:

Command details:

Command number:	1

Command type:		SET UP EVENT LIST

Command qualifier:	'00'

Device identities

Source device:		UICC

Destination device:	ME

Event List:

Event 1:				Location status

Coding:

| BER-TLV: | D0 | 0C | 81 | 03 | 01 | 05 | 00 | 82 | 02 | 81 | 82 | 99 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 01 | 03 |

TERMINAL RESPONSE: SET UP EVENT LIST 2.3.1

Logically:

Command details:

Command number:	1

Command type:		SET UP EVENT LIST

Command qualifier:	'00'

Device identities:

Source device:		ME

Destination device:	UICC

Result:

General Result:		Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 05 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

PROACTIVE COMMAND: REFRESH 2.3.1

Logically:

Command details:

Command number:	1

Command type:		REFRESH

Command qualifier:	Steering of Roaming

Device identities

Source device:		UICC

Destination device:	ME

PLMNwAcT List:

1st PLMN:				254/003

1st ACT:			NG-RAN

2nd PLMN:			254/004

2nd ACT:			NG-RAN

Coding:

| BER-TLV: | D0 | 15 | 81 | 03 | 01 | 01 | 07 | 82 | 02 | 81 | 82 | 72 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0A | 52 | 34 | 00 | 08 | 00 | 52 | 44 | 00 | 08 | 00 |  |

TERMINAL RESPONSE: REFRESH 2.3.1

Logically:

Command details:

Command number:	1

Command type:		REFRESH

Command qualifier:	Steering of Roaming

Device identities:

Source device:		ME

Destination device:	UICC

Result:

General Result:		Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 01 | 07 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

ENVELOPE: EVENT DOWNLOAD - LOCATION STATUS 2.3.0

Logically:

Event list:	Location status

Device identities

Source device:		ME

Destination device:	UICC

Location status:			no service

Coding:

| BER-TLV: | D6 | 0A | 19 | 01 | 03 | 82 | 02 | 82 | 81 | 9B | 01 | 02 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

ENVELOPE: EVENT DOWNLOAD - LOCATION STATUS 2.3.1

Logically:

Event list:	Location status

Device identities

Source device:		ME

Destination device:	UICC

Location status:			normal service

Location Information:

MCC & MNC:	the mobile country and network code (MCC = 254, MNC = 001)

TAC:				the tracking area code (000001)

NG-SS cell id:	the cell identity value (0001 (36 bits))

Coding:

| BER-TLV: | D6 | 17 | 19 | 01 | 03 | 82 | 02 | 82 | 81 | 1B | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 13 | 0B | 52 | 14 | 00 | 00 | 00 | 01 | 00 | 00 | 00 | 00 |
|  | 1F |

ENVELOPE: EVENT DOWNLOAD - LOCATION STATUS 2.3.2

Logically:

Event list:	Location status

Device identities

Source device:		ME

Destination device:	UICC

Location status:			normal service

Location Information:

MCC & MNC:	the mobile country and network code (MCC = 254, MNC = 003)

TAC:				the tracking area code (000001)

NG-SS cell id:	the cell identity value (0001 (36 bits))

Coding:

| BER-TLV: | D6 | 17 | 19 | 01 | 03 | 82 | 02 | 82 | 81 | 1B | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 13 | 0B | 52 | 34 | 00 | 00 | 00 | 01 | 00 | 00 | 00 | 00 |
|  | 1F |

Expected Sequence 2.4 (SMS-PP Data Download in several ENVELOPE commands after Steering of Roaming via DL NAS TRANSPORT long message with REFRESH command [Steering of Roaming])

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | USER ME | The ME is switched on | ME will perform Profile Download and USIM initialisation |
| 2 | ME NG-SS | ME successfully REGISTER with NG-RAN cell. |  |
| 3 | NG-SS ME | NG-SS send to ME DL NAS TRANSPORT long message 2.4.1 with acknowledgement not requested List Type is secured packet | SOR header with: ACK set to "acknowledgement not requested" List Type set to "secured packet" |
| 4 | ME UICC | ENVELOPE: SMS-PP DOWNLOAD 2.4.1 | the ME shall pass the message transparently to the UICC using the ENVELOPE (SMS-PP DOWNLOAD) command as specified in TS 31.111 [15] clause 7.1.1.1a Note: Message is too long for one ENVELOPE command then it is cut in several ENVELOPE commands. 1st part of message |
| 5 | UICC ME | SW1/SW2 90 00 |  |
| 6 | ME UICC | ENVELOPE: SMS-PP DOWNLOAD 2.4.2 | 2nd part of message |
| 7 | UICC ME | SW1/SW2 90 00 |  |
| 8 | ME UICC | ENVELOPE: SMS-PP DOWNLOAD 2.4.3 | 3rd and last part of message |
| 9 | UICC ME | SW1/SW2 91 XX |  |
| 10 | ME UICC | FETCH |  |
| 11 | UICC ME | PROACTIVE COMMAND: REFRESH 2.4.1 [Steering of Roaming] |  |
| 12 | ME UICC | TERMINAL RESPONSE: REFRESH 2.4.1 |  |
| 13 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 14 | ME | Steering of Roaming procedure | As specified in TS 23.122 [29] clause 4.4.6 Note: The SoR procedure cannot be verified completely in this step. A verification of the complete SoR procedure is done in Expected Sequence 2.3 |

DL NAS TRANSPORT message 2.4.1

Logically:

Message details (referring to TS 24.501 Table 9.11.3.51.1)

Payload container type IE:	"0100" (SOR transparent container)

SOR header:

SOR data type:	"0" (SOR transparent container carries steering of roaming information)

List indication:	"1" (list of preferred PLMN/access technology combinations is provided)

List type:	"0" (The list type is a secured packet.)

ACK:	"0" (acknowledgment not requested)

Secured packet	as specified in TS 31.111 [15] clause 7.1.1.1a:

| 40 | 00 | 91 | 7F | F6 | 00 | 00 | 00 | 00 | 00 | 00 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8C | 07 | 00 | 03 | 1C | 03 | 01 | 70 | 00 | 01 | 48 | 15 |
| 02 | 00 | 10 | 10 | B0 | 01 | 40 | 00 | 00 | 00 | 00 | 00 |
| 00 | 02 | 99 | 54 | A1 | DC | 40 | 46 | 7B | AA | 82 | 01 |
| 2E | 22 | 07 | 00 | A4 | 00 | 04 | 02 | 6F | 61 | 22 | 81 |
| 8C | 00 | D6 | 00 | 00 | 87 | 52 | 14 | 00 | 00 | 80 | 52 |
| 24 | 00 | 80 | 00 | 52 | 34 | 00 | 08 | 00 | 52 | 44 | 00 |
| 00 | 80 | 52 | 54 | 00 | 80 | 00 | 52 | 64 | 00 | 08 | 00 |
| 52 | 74 | 00 | 00 | 80 | 52 | 84 | 00 | 80 | 00 | 52 | 94 |
| 00 | 08 | 00 | 52 | 19 | 00 | 00 | 80 | 52 | 29 | 00 | 80 |
| 00 | 52 | 39 | 00 | 08 | 00 | 52 | 49 | 00 | 00 | 80 | 52 |
| 59 | 00 | 80 | 00 | 52 | 69 | 00 | 08 | 00 | 52 | 79 | 00 |
| 00 | 80 | 52 | 89 | 00 | 80 | 00 | 52 | 99 | 40 | 00 | 91 |
| 7F | F6 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 8C | 05 | 00 |
| 03 | 1C | 03 | 02 | 00 | 08 | 00 | 52 | 11 | 00 | 00 | 80 |
| 52 | 21 | 00 | 80 | 00 | 52 | 31 | 00 | 08 | 00 | 52 | 41 |
| 00 | 00 | 80 | 52 | 51 | 00 | 80 | 00 | 52 | 61 | 00 | 08 |
| 00 | 52 | 71 | 00 | 00 | 80 | 52 | 81 | 00 | 80 | 00 | 52 |
| 91 | 00 | 08 | 00 | 81 | 81 | 93 | 81 | 03 | 01 | 01 | 07 |
| 82 | 02 | 81 | 82 | 72 | 81 | 87 | 52 | 14 | 00 | 00 | 80 |
| 52 | 24 | 00 | 80 | 00 | 52 | 34 | 00 | 08 | 00 | 52 | 44 |
| 00 | 00 | 80 | 52 | 54 | 00 | 80 | 00 | 52 | 64 | 00 | 08 |
| 00 | 52 | 74 | 00 | 00 | 80 | 52 | 84 | 00 | 80 | 00 | 52 |
| 94 | 00 | 08 | 00 | 52 | 19 | 00 | 00 | 80 | 52 | 29 | 00 |
| 80 | 00 | 52 | 39 | 00 | 08 | 00 | 52 | 49 | 00 | 00 | 80 |
| 52 | 59 | 00 | 80 | 00 | 52 | 44 | 00 | 91 | 7F | F6 | 00 |
| 00 | 00 | 00 | 00 | 00 | 00 | 46 | 05 | 00 | 03 | 1C | 03 |
| 03 | 69 | 00 | 08 | 00 | 52 | 79 | 00 | 00 | 80 | 52 | 89 |
| 00 | 80 | 00 | 52 | 99 | 00 | 08 | 00 | 52 | 11 | 00 | 00 |
| 80 | 52 | 21 | 00 | 80 | 00 | 52 | 31 | 00 | 08 | 00 | 52 |
| 41 | 00 | 00 | 80 | 52 | 51 | 00 | 80 | 00 | 52 | 61 | 00 |
| 08 | 00 | 52 | 71 | 00 | 00 | 80 | 52 | 81 | 00 | 80 | 00 |
| 52 | 91 | 00 | 08 | 00 |  |  |  |  |  |  |  |

ENVELOPE: SMS-PP DOWNLOAD 2.4.1

Logically:

SMS-PP Download

Device identities

Source device:	Network

Destination device:	UICC

SMS TPDU		           1st part of Secured Packet from DL NAS TRANSPORT message 2.4.1

Coding:

| BER-TLV: | D1 | 81 | A0 | 02 | 02 | 83 | 81 | 0B | 81 | 99 | 40 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 91 | 7F | F6 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 8C | 07 |
|  | 00 | 03 | 1C | 03 | 01 | 70 | 00 | 01 | 48 | 15 | 02 | 00 |
|  | 10 | 10 | B0 | 01 | 40 | 00 | 00 | 00 | 00 | 00 | 00 | 02 |
|  | 99 | 54 | A1 | DC | 40 | 46 | 7B | AA | 82 | 01 | 2E | 22 |
|  | 07 | 00 | A4 | 00 | 04 | 02 | 6F | 61 | 22 | 81 | 8C | 00 |
|  | D6 | 00 | 00 | 87 | 52 | 14 | 00 | 00 | 80 | 52 | 24 | 00 |
|  | 80 | 00 | 52 | 34 | 00 | 08 | 00 | 52 | 44 | 00 | 00 | 80 |
|  | 52 | 54 | 00 | 80 | 00 | 52 | 64 | 00 | 08 | 00 | 52 | 74 |
|  | 00 | 00 | 80 | 52 | 84 | 00 | 80 | 00 | 52 | 94 | 00 | 08 |
|  | 00 | 52 | 19 | 00 | 00 | 80 | 52 | 29 | 00 | 80 | 00 | 52 |
|  | 39 | 00 | 08 | 00 | 52 | 49 | 00 | 00 | 80 | 52 | 59 | 00 |
|  | 80 | 00 | 52 | 69 | 00 | 08 | 00 | 52 | 79 | 00 | 00 | 80 |
|  | 52 | 89 | 00 | 80 | 00 | 52 | 99 |  |  |  |  |  |

ENVELOPE: SMS-PP DOWNLOAD 2.4.2

Logically:

SMS-PP Download

Device identities

Source device:	Network

Destination device:	UICC

SMS TPDU		           2nd part of Secured Packet from DL NAS TRANSPORT message 2.4.1

Coding:

| BER-TLV: | D1 | 81 | A0 | 02 | 02 | 83 | 81 | 0B | 81 | 99 | 40 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 91 | 7F | F6 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 8C | 05 |
|  | 00 | 03 | 1C | 03 | 02 | 00 | 08 | 00 | 52 | 11 | 00 | 00 |
|  | 80 | 52 | 21 | 00 | 80 | 00 | 52 | 31 | 00 | 08 | 00 | 52 |
|  | 41 | 00 | 00 | 80 | 52 | 51 | 00 | 80 | 00 | 52 | 61 | 00 |
|  | 08 | 00 | 52 | 71 | 00 | 00 | 80 | 52 | 81 | 00 | 80 | 00 |
|  | 52 | 91 | 00 | 08 | 00 | 81 | 81 | 93 | 81 | 03 | 01 | 01 |
|  | 07 | 82 | 02 | 81 | 82 | 72 | 81 | 87 | 52 | 14 | 00 | 00 |
|  | 80 | 52 | 24 | 00 | 80 | 00 | 52 | 34 | 00 | 08 | 00 | 52 |
|  | 44 | 00 | 00 | 80 | 52 | 54 | 00 | 80 | 00 | 52 | 64 | 00 |
|  | 08 | 00 | 52 | 74 | 00 | 00 | 80 | 52 | 84 | 00 | 80 | 00 |
|  | 52 | 94 | 00 | 08 | 00 | 52 | 19 | 00 | 00 | 80 | 52 | 29 |
|  | 00 | 80 | 00 | 52 | 39 | 00 | 08 | 00 | 52 | 49 | 00 | 00 |
|  | 80 | 52 | 59 | 00 | 80 | 00 | 52 |  |  |  |  |  |

ENVELOPE: SMS-PP DOWNLOAD 2.4.3

Logically:

SMS-PP Download

Device identities

Source device:	Network

Destination device:	UICC

SMS TPDU		           3rd part of Secured Packet from DL NAS TRANSPORT message 2.4.1

Coding:

| BER-TLV: | D1 | 59 | 02 | 02 | 83 | 81 | 0B | 53 | 44 | 00 | 91 | 7F |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | F6 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 46 | 05 | 00 | 03 |
|  | 1C | 03 | 03 | 69 | 00 | 08 | 00 | 52 | 79 | 00 | 00 | 80 |
|  | 52 | 89 | 00 | 80 | 00 | 52 | 99 | 00 | 08 | 00 | 52 | 11 |
|  | 00 | 00 | 80 | 52 | 21 | 00 | 80 | 00 | 52 | 31 | 00 | 08 |
|  | 00 | 52 | 41 | 00 | 00 | 80 | 52 | 51 | 00 | 80 | 00 | 52 |
|  | 61 | 00 | 08 | 00 | 52 | 71 | 00 | 00 | 80 | 52 | 81 | 00 |
|  | 80 | 00 | 52 | 91 | 00 | 08 | 00 |  |  |  |  |  |

PROACTIVE COMMAND: REFRESH 2.4.1

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	Steering of Roaming

Device identities

Source device:	UICC

Destination device:	ME

PLMNwAcT List (27 entries)

1st PLMN:		254/001			1st ACT:	GERAN

2nd PLMN:	254/002			2nd ACT:	UTRAN

3rd PLMN:	254/003			3rd ACT:	NG-RAN

4th PLMN:	254/004			4th ACT:	GERAN

5th PLMN:	254/005			5th ACT:	UTRAN

6th PLMN:	254/006			6th ACT:	NG-RAN

7th PLMN:	254/007			7th ACT:	GERAN

8th PLMN:	254/008			8th ACT:	UTRAN

9th PLMN:	254/009			9th ACT:	NG-RAN

10th PLMN:	259/001			10th ACT:	GERAN

11th PLMN:	259/002			11th ACT:	UTRAN

12th PLMN:	259/003			12th ACT:	NG-RAN

13th PLMN:	259/004			13th ACT:	GERAN

14th PLMN:	259/005			14th ACT:	UTRAN

15th PLMN:	259/006			15th ACT:	NG-RAN

16th PLMN:	259/007			16th ACT:	GERAN

17th PLMN:	259/008			17th ACT:	UTRAN

18th PLMN:	259/009			18th ACT:	NG-RAN

19th PLMN:	251/001			19th ACT:	GERAN

20th PLMN:	251/002			20th ACT:	UTRAN

21st PLMN:	251/003			21st ACT:	NG-RAN

22nd PLMN:	251/004			22nd ACT:	GERAN

23rd PLMN:	251/005			23rd ACT:	UTRAN

24th PLMN:	251/006			24th ACT:	NG-RAN

25th PLMN:	251/007			25th ACT:	GERAN

26th PLMN:	251/008			26th ACT:	UTRAN

27th PLMN:	251/009			27th ACT:	NG-RAN

Coding:

| BER-TLV: | D0 | 81 | 93 | 81 | 03 | 01 | 01 | 07 | 82 | 02 | 81 | 82 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 72 | 81 | 87 | 52 | 14 | 00 | 00 | 80 | 52 | 24 | 00 | 80 |
|  | 00 | 52 | 34 | 00 | 08 | 00 | 52 | 44 | 00 | 00 | 80 | 52 |
|  | 54 | 00 | 80 | 00 | 52 | 64 | 00 | 08 | 00 | 52 | 74 | 00 |
|  | 00 | 80 | 52 | 84 | 00 | 80 | 00 | 52 | 94 | 00 | 08 | 00 |
|  | 52 | 19 | 00 | 00 | 80 | 52 | 29 | 00 | 80 | 00 | 52 | 39 |
|  | 00 | 08 | 00 | 52 | 49 | 00 | 00 | 80 | 52 | 59 | 00 | 80 |
|  | 00 | 52 | 69 | 00 | 08 | 00 | 52 | 79 | 00 | 00 | 80 | 52 |
|  | 89 | 00 | 80 | 00 | 52 | 99 | 00 | 08 | 00 | 52 | 11 | 00 |
|  | 00 | 80 | 52 | 21 | 00 | 80 | 00 | 52 | 31 | 00 | 08 | 00 |
|  | 52 | 41 | 00 | 00 | 80 | 52 | 51 | 00 | 80 | 00 | 52 | 61 |
|  | 00 | 08 | 00 | 52 | 71 | 00 | 00 | 80 | 52 | 81 | 00 | 80 |
|  | 00 | 52 | 91 | 00 | 08 | 00 |  |  |  |  |  |  |

TERMINAL RESPONSE: REFRESH 2.4.1

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	Steering of Roaming

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 01 | 07 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

##### 27.22.14.2.5	Test requirement

The ME shall operate in the manner defined in expected sequence 2.1 to 2.4.

***** end of changes *****