3GPP TSG-CT Meeting #82	CP-183229

# Sorrento, Italy, 10 - 11 Decemebr 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.518 | CR | 0132 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to OpenAPI definition of UeContextTransferRspData |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-CT |  |  |  |  |  | Date: |  |  | 2018-12-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | CR 29.518 #0031 approved at CT#81 replaced the RegistrationCompleteNotify (originally defined as a callback operation) by a UERegStatusUpdate service operation. Accordingly, the CR deleted the regCompleteNotifyUri attribute from the UeContextTransferRspData type in subclause 6.1.6.2.24. But the corresponding OpenAPI change was missed to be specified, resulting in the following definition: UeContextTransferRspData: type: object properties: ueContext: $ref: '#/components/schemas/UeContext' regCompleteNotifyUri: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uri' supportedFeatures: $ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures' required: - ueContext - regCompleteNotifyUri The OpenAPI definition is inconsistent with the UeContextTransferRspData type specified in subclause 6.1.6.2.24. The schema validation would always fail if the regCompleteNotifyUri attribute was not included in the request. Correcting the OpenAPI definition after CT#82 would result in a backward incompatible change. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The regCompleteNofifyUri attribute is removed from the OpenAPI definition of the UeContextTransferRspData type to align with the type definition in subclause 6.1.6.2.24 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent definition of UeContextTransferRspData between the normative table and the OpenAPI document, causing interoperability issues during inter-AMF mobility scenarios. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## A.2	Namf_Communication API

openapi: 3.0.0

[…]

UeContextTransferRspData:

type: object

properties:

ueContext:

$ref: '#/components/schemas/UeContext'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- ueContext

[…]

* * * End of Changed Change * * * *