3GPP TSG CT WG4 Meeting #90	C4-191064

Xi'an, P.R.China; 08th – 12th April 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.272 | CR | 0797 | rev | - | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Service Gap Time Deletion |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15 |  |  |  |  |  |  | Date: |  | 2019-03-20 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | CR 0756r1 (C4-181301) introduced the new subscription parameter “Service Gap Time”, to be sent from HSS to MME, however, a deletion of this parameter is not possible. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new DSR-flag indicating that the Service-Gap-Time shall be deleted from the subscription data. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Deletion of Service Gap Time is not possible. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.25 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 7.3.25	DSR-Flags

The DSR-Flags AVP is of type Unsigned32 and it shall contain a bit mask. The meaning of the bits is defined in table 7.3.25/1:

Table 7.3.25/1: DSR-Flags

| Bit | Name | Description |
| --- | --- | --- |
| 0 | Regional Subscription Withdrawal | This bit, when set, indicates that Regional Subscription shall be deleted from the subscriber data. |
| 1 | Complete APN Configuration Profile Withdrawal | This bit, when set, indicates that all EPS APN configuration data for the subscriber shall be deleted from the subscriber data. This flag only applies to the S6d interface. |
| 2 | Subscribed Charging Characteristics Withdrawal | This bit, when set, indicates that the Subscribed Charging Characteristics have been deleted from the subscription data. |
| 3 | PDN subscription contexts Withdrawal | This bit, when set, indicates that the PDN subscription contexts whose identifier is included in the Context-Identifier AVP shall be deleted. (Note 1) |
| 4 | STN-SR | This bit, when set, indicates that the Session Transfer Number for SRVCC shall be deleted from the subscriber data. |
| 5 | Complete PDP context list Withdrawal | This bit, when set, indicates that all PDP contexts for the subscriber shall be deleted from the subscriber data. |
| 6 | PDP contexts Withdrawal | This bit, when set, indicates that the PDP contexts whose identifier is included in the Context-Identifier AVP shall be deleted. (Note 2) |
| 7 | Roaming Restricted due to unsupported feature | This bit, when set, indicates that the roaming restriction shall be deleted from the subscriber data in the MME or SGSN. |
| 8 | Trace Data Withdrawal | This bit, when set, indicates that the Trace Data shall be deleted from the subscriber data. |
| 9 | CSG Deleted | This bit, when set, indicates that - the "CSG-Subscription-Data from HSS" shall be deleted in the MME or SGSN when received over the S6a or S6d interface - the "CSG-Subscription-Data from CSS" shall be deleted in the MME or SGSN when received over the S7a or S7d interface. |
| 10 | APN-OI-Replacement | This bit, when set, indicates that the UE level APN-OI-Replacement shall be deleted from the subscriber data. |
| 11 | GMLC List Withdrawal | This bit, when set, indicates that the subscriber's LCS GMLC List shall be deleted from the MME or SGSN. |
| 12 | LCS Withdrawal | This bit, when set, indicates that the LCS service whose code is included in the SS-Code AVP shall be deleted from the MME or SGSN. |
| 13 | SMS Withdrawal | This bit, when set, indicates that the SMS service whose code is included in the SS-Code AVP or TS-Code AVP shall be deleted from the MME or SGSN. |
| 14 | Subscribed periodic RAU-TAU Timer Withdrawal | This bit, when set, indicates that the subscribed periodic RAU TAU Timer value shall be deleted from the subscriber data. |
| 15 | Subscribed VSRVCC Withdrawal | This bit, when set, indicates that the Subscribed VSRVCC shall be deleted from the subscriber data. |
| 16 | A-MSISDN Withdrawal | This bit, when set, indicates that the additional MSISDN, if present, shall be deleted from the subscriber data. |
| 17 | ProSe Withdrawal | This bit, when set, indicates that the ProSe subscription data shall be deleted from the MME or combined MME/SGSN. |
| 18 | Reset-IDs | This bit, when set, indicates that the set of Reset-IDs shall be deleted from the MME or SGSN. |
| 19 | DL-Buffering-Suggested-Packet-Count Withdrawal | This bit, when set, indicates that the DL-Buffering-Suggested-Packet-Count shall be deleted in the MME or SGSN. |
| 20 | Subscribed IMSI-Group-Id Withdrawal | This bit, when set, indicates that all subscribed IMSI-Group-Id(s) shall be deleted in the MME or SGSN. |
| 21 | Delete monitoring events | This bit when set indicates to the MME or SGSN to delete all the Monitoring events for the subscriber which are associated with the provided SCEF-ID. |
| 22 | User Plane Integrity Protection Withdrawal | This bit, when set, indicates to the SGSN that User Plane Integrity Protection may no longer be required when GERAN is used. The MME shall ignore it. |
| 23 | MSISDN Withdrawal | This bit, when set, indicates that the MSISDN shall be deleted from the subscriber data. |
| 24 | UE Usage Type Withdrawal | This bit, when set, indicates to the MME or SGSN that the UE Usage Type shall be deleted from the subscription data. |
| 25 | V2X Withdrawal | This bit, when set, indicates that the V2X subscription data shall be deleted from the MME or combined MME/SGSN. |
| 26 | External-Identifier-Withdrawal | This bit, when set, indicates that the External-Identifier shall be deleted from the subscriber data. |
| 27 | Aerial-UE-Subscription Withdrawal | This bit, when set, indicates that the Aerial UE subscription shall be deleted from the subscriber data. |
| 28 | Paging Time Window Subscription Withdrawal | This bit, when set, indicates that the Paging Time Window subscription shall be deleted from the subscriber data. |
| 29 | Active-Time-Withdrawal | This bit, when set, indicates that the Active Time used for PSM shall be deleted from the subscriber data. |
| 30 | eDRX-Cycle-Length -Withdrawal | This bit, when set, indicates that the eDRX-Cycle-Length shall be deleted from the subscriber data. |
| xx | Service-Gap-Time-Withdrawal | This bit, when set, indicates that the Service Gap Time shall be deleted from the subscriber data. |
| Note 1: If the Complete APN Configuration Profile Withdrawal bit is set, this bit should not be set. Note 2: If the Complete PDP context list Withdrawal bit is set, this bit should not be set. Note 3: Bits not defined in this table shall be cleared by the sending HSS and discarded by the receiving MME or SGSN. Note 4: Bits 3 and 6 are excluding alternatives and shall not both be set. Note 5: When this AVP is transferred over the S7a/S7d interface, only the bit 9 (CSG Deleted) is meaningful, other bits shall be cleared. |  |  |

* * * End Of Change * * * *