3GPP TSG-CT WG1 Meeting #126-e	C1-206453

# Electronic meeting, 15-23 October 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.501 | CR | 2614 | rev | 1 | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Alignment of User Plane Integrity Protection description |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Deutsche Telekom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16, UPIP_SEC |  |  |  |  |  | Date: |  |  | 2020-10-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 3GPP RAN2 informed CT1 and SA2 that 3GPP RAN2 agreed to apply the mandatory full rate UPIP for NE-DC with MN terminated bearers and advice CT1 and SA2 to check their related changes to their specifications for any cross-WG inconsistency - see LS C1-205854. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add clarification that the mandatory UPIP for UEs supporting N3 data transfer and supporting standalone NR connected to 5GCN includes also UEs supporting NR-E-UTRA dual connectivity - as described in RAN specification 3GPP TS 37.340. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment between 3GPP RAN2 and 3GPP CT1 description for full-rate user-plane integrity protection will cause cross-WG inconsistency. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.11.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR … CR… |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** Change Start *****

#### 9.11.4.7	Integrity protection maximum data rate

The purpose of the integrity protection maximum data rate information element is for the UE to indicate to the network the maximum data rate per UE for user-plane integrity protection for uplink and the maximum data rate per UE for user-plane integrity protection for downlink that are supported by the UE.

The integrity protection maximum data rate is coded as shown in figure 9.11.4.7.1 and table 9.11.4.7.2.

The integrity protection maximum data rate is a type 3 information element with a length of 3 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Integrity protection maximum data rate IEI |  |  |  |  |  |  |  | octet 1 |
| Maximum data rate per UE for user-plane integrity protection for uplink |  |  |  |  |  |  |  | octet 2 |
| Maximum data rate per UE for user-plane integrity protection for downlink |  |  |  |  |  |  |  | octet 3 |

Figure 9.11.4.7.1: Integrity protection maximum data rate information element

Table 9.11.4.7.2: Integrity protection maximum data rate information element

| Maximum data rate per UE for user-plane integrity protection for uplink (octet 2) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bits |  |  |  |  |  |  |  |  |  |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | 64 kbps (NOTE 3) |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |  | NULL (NOTE 1) |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |  | Full data rate (NOTE 2) |
| All other values are spare and shall not be used by a UE compliant to the present version of this specification. If received they shall be interpreted as "64 kbps". |  |  |  |  |  |  |  |  |  |
| Maximum data rate per UE for user-plane integrity protection for downlink (octet 3) |  |  |  |  |  |  |  |  |  |
| Bits |  |  |  |  |  |  |  |  |  |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | 64 kbps (NOTE 3) |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |  | NULL (NOTE 1) |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |  | Full data rate (NOTE 2) |
| All other values are spare and shall not be used by a UE compliant to the present version of this specification. If received they shall be interpreted as "64 kbps". |  |  |  |  |  |  |  |  |  |
| NOTE 1: This value shall be used when N3 data transfer is not supported by the UE. |  |  |  |  |  |  |  |  |  |
| NOTE 2: If the UE supports N3 data transfer and supports standalone NR connected to 5GCN (this includes UEs supporting NR-NR dual connectivity, NR-E-UTRA dual connectivity with MN terminated bearers or both of them as described in 3GPP TS 37.340 [51]), then the UE shall use this value. |  |  |  |  |  |  |  |  |  |
| NOTE 3: The network can receive this value from a UE compliant to an earlier version of this specification or when the UE supports N3 data transfer and does not support standalone NR connected to 5GCN. |  |  |  |  |  |  |  |  |  |

***** Change End *****