3GPP TSG-CT WG4 Meeting #129	C4-252264

Bratislava, Slovakia, 19th – 23nd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.338 | CR | 0063 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MPS for Messaging IP-SM-GW behaviour |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Peraton Labs, CISA ECD, AT&T |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | MPS4msg |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The behaviour for the IP-SM-GW is missing from the Send Routing Info for SM procedures. In TS 23.204, clause 6.14, item 3c: The IP SM GW returns the indication whether MPS for Messaging is set (enabled) for the UE in the HLR/HSS (see TS 23.228 [9]). If MPS for Messaging is enabled for the UE, the IP-SM-GW sets the transport priority of the message to a value appropriate for MPS and handles the message with priority. The behaviour for the IP-SM-GW is missing for MO SMS over IP. From TS 23.204 clause 6.7, step 4: If the Resource-Priority information on the Instant Message has a value appropriate for MPS, the IP-SM-GW sets the transport priority of the outgoing Short Message to a value appropriate for MPS and handles the Short Message with priority. |  |  |  |  |  |  |  |  |
| Summary of change: |  | First change: When the HSS forwards a Send Routing Info for SM request, it sets the transport priority if the MPS for Messaging indication in the HSS is set. Second change: When the IP-SM-GW sends a Send Routing Info for SM answer, it adds the MPS for Messaging indication and sets the transport priority if the MPS for Messaging indication in the HSS is set. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The MO SMS will not be treated with priority after it passes the IP-SM-GW. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1.x (new), 5.2.1.y (new). |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# ***** First change *****

#### 5.2.1.x	Detailed bevahiour of the HSS when forwarding

This clause describes the HSS behaviour when the HSS receives a Send Routing Info for SM request that is forwarded to an IP-SM-GW.

If the HSS receives a Send Routing Info for SM request from the SMS-GMSC, the HSS has an MPS for Messaging indication indicating that MPS for Messaging for the UE has been enabled, and only if the UE has an MPS subscription in the HSS, if allowed by local policy, the HSS shall handle the Send Routing Info for SM request with priority, should include a DSCP value appropriate for MPS and shall include the DRMP AVP with a value appropriate for MPS priority in the Send Routing Info for SM request forwarded to the IP-SM-GW.

# ***** Second change *****

#### 5.2.1.y	Detailed behaviour of the IP-SM-GW

This clause describes the IP-SM-GW behaviour when the IP-SM-GW receives a Send Routing Info for SM request from the SMS-GMSC via the HSS.

After the IP-SM-GW receives a Send Routing Info for SM request from the HSS, and after sending a Send Routing Info for SM request to the HSS, if the IP-SM-GW receives a Send Routing Info for SM answer from the HSS with the MPS-Priority AVP indicating MPS priority, the IP-SM-GW shall handle the Send Routing Info for SM answer from the HSS with priority, shall include the MPS-Priority AVP, should include a DSCP value appropriate for MPS and shall include the DRMP AVP with a value appropriate for MPS priority in the Send Routing Info for SM answer to the SMS-GMSC.

# ***** End of changes ****