3GPP TSG-CT WG4 Meeting #129	C4-252301

Bratislava, Slovakia; 19th – 23rd May 2025

# Title:	Reply LS on Format of FQDN for PRINS

# Response to:	LS (5GMRR Doc 024_203r1) on Format of FQDN for PRINS from 5GMRR

# Release:	Rel-19

# Work Item:

Source:	3GPP CT4

To:	5GMRR

Cc:

Contact Person:

Name:	Mamdoh Shahin

Tel. Number:

E-mail Address:	mamdoh.shahin@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	C4-252107 [3GPP TS 29.573 CR on Format of IEs with FQDN data type].

1. Overall Description:

CT4 would like to thank GSMA 5GMRR for the LS on the usage of FQDNs within TS 29.573 and for raising the question regarding whether node-level or domain-level FQDNs are expected in specific Information Elements.

CT4 has agreed the attached 3GPP TS 29.573 CR to define the format of IEs with FQDN data type. The CR provides clarification on the appropriate usage and expected format for each relevant IE.

The table below summarizes the expected FQDN format (domain-level or node-level) per IE in 3GPP TS 29.573:

| Clause | IE Name | Expected FQDN Format | Description |
| --- | --- | --- | --- |
| 6.1.5.2.4 / 6.1.5.2.5 | ipxProviderSecInfoList.ipxProviderId | Node-level | Used to associate specific keying material (e.g., certificates, raw public keys) with the exact RI node; required for per-node integrity verification. |
| 6.1.5.2.8 | isModifiableByIpx (key: ipxProviderId) | Domain-level | Defines modifiability rules per RI operator, not individual nodes. |
| 6.2.5.2.9 | authorizedIpxId | Domain-level | Identifies the RI operator authorized to insert modifications; authorization applies at operator level. |
| 6.2.5.2.9 | riFqdn | Node-level | Identifies the originating RI node for an N32-f request message; required for source traceability. |
| 6.2.5.2.10 | identity | Node-level | Identifies the RI node that inserted a modification block. |
| 6.2.5.2.10 | riOperatorId | Domain-level | Explicitly signals the operator responsible for a modification. |
| 6.1.5.2.12 | FailedModificationInfo.ipxId | Node-level | Identifies the RI node whose modification failed. |

2. Actions:

To GSMA NG 5GMRR group.

ACTION: 	CT4 kindly asks 5GMRR group to take the above answer into account.

3. Date of Next CT4 Meetings:

CT4 Meeting calendar can be found at:

https://www.3gpp.org/dynareport?code=Meetings-C4.htm