3GPP TSG-CT WG4 Meeting #129	C4-252380

# Bratislava, Slovakia; 19th – 23rd May 2025												was C4-252076

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.175 | CR | 0064 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Removal of Bootstrap Data Channel |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The bootstrap data channel might be removed before the IMS session termination, during the interaction with supplementary service, as described in TS24.186 Annex A. (also see S2-2504204/4205) However, in current stage 3 specification, it doesn't support the bootstrap data channel removal before the IMS session termination. To support this, updates to TS29.175 is required so that to allow the IMS AS to notify the DCSF that the bootstrap data channel is required to be removed. It also applies in other cases when the removal of application data channel is required. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Following changes are made: - in clause 6.1.6.2.5, add "mediaRemovalInd" to indicate the media (e.g. DC media) is to be removed; - update OpenAPI in A.2 accordingly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not possible to notify the DCSF that the bootstrap data channel is to be removed during the interaction with supplementary service. It results in waste of data channel resource in the MF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.6.2.5, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible correction to the following APIs: TS29175_Nimsas_SessionEventControl.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev#1: Change to clause 5.2.2.2 is reverted. Update the description of "mediaRemovalInd" attribute to clear the presence condition, e.g. to the BDC removal during the interaction with supplementary service or in other cases if ADC is to be removed. |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 6.1.6.2.5	Type: MediaInfo

Table 6.1.6.2.5-1: Definition of type MediaInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| mediaId | MediaId | M | 1 | The media ID uniquely identifies this media item within the list. The identity is allocated by IMS AS. |
| mediaType | MediaType | M | 1 | The media type of the media for which the notification is generated. |
| dcMediaSpec | DcMediaSpec | C | 0..1 | The data channel media specification includes the relevant media attributes of interest to the consumer. It shall be contained if the mediaType is set to "DC". |
| mediaSuspended | boolean | C | 0..1 | This IE indicates whether the media is suspended or not. It shall be contained if the media is suspended or resumed as specified in 3GPP TS 24.610 [18] and 3GPP TS 24.186 [19]. - true: the media is suspended. - false: the media is resumed. |
| mediaRemovalInd | boolean | C | 0..1 | This IE indicates whether the media is to be removed or not. It shall be only present with true value if the bootstrap data channel media is to be removed during the interaction with supplementary service, or if the application data channel is to be removed in other cases. Presence with false value shall be prohibited. |

* * * Next Change * * * *

# A.2	Nimsas_SessionEventControl API

********TEXT SKIPPED********

MediaInfo:

description: The media information related to the notification event.

type: object

required:

- mediaId

- mediaType

properties:

mediaId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MediaId'

mediaType:

$ref: '#/components/schemas/MediaType'

dcMediaSpec:

$ref: '#/components/schemas/DcMediaSpec'

mediaSuspended:

type: boolean

mediaRemovalInd:

type: boolean

enum:

- true

********TEXT SKIPPED********

* * * End of Changes * * * *