3GPP TSG-CT WG4 Meeting #129	C4-252430

# Bratislava, Slovakia; 19th – 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.380 | CR | 0128 | rev | 2 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add the PCRF/PCF-based restoration solution for EPC/5GC NF failure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | IMS_RES-CT |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The SID of IMS_RES has completed and a new WID of IMS_RES-CT has been agreed/endorsed in CT4 #128. The following solutions for KI #3 "Speed up the restoration procedure of the terminating session when EPC/5GC NF fails" should be specified in TS 23.380: According to the evaluation for Key Issue #3, the following conclusions are concluded: - For the failure check, the health check triggered by the P-CSCF described in Solution #6 and #9 or by the PCRF/PCF described in Solution #8 are proposed to be specified in normative work. - For the failure restoration, Solution #9 is proposed to be specified in normative work for it is an enhancement to Solution #6. - Solution #7 is proposed as an example to detect the P-CSCF failure to be added in 3GPP TS 23.380 [2]. Thus it is proposed to add the PCRF/PCF-based restoration solution for EPC/5GC NF failure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Add a reference to 3GPP TS 29.513. 2. Add the PCRF/PCF-based restoration solution for EPC/5GC NF failure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | How to detect the EPC/5GC NF failure by the P-CSCF is unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, X.2.3(new), X.2.3.1(new), X.2.3.2(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Clause X, subclause X.1, X.2, and X.2.1 are introduced in CR 0127 and Clause X.2.2 is introduced in CR 0129 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: Reword the title of X.2.3 to reflect the solution. Use the visio form for the figure. Rev2: 1. Reword the title of the X.2.3 and figure. 2. Correct some typos in the steps and figure. 3. Reword the step 4c to distinguish two cases. |  |  |  |  |  |  |  |  |

* * * First Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.007: "Restoration procedures".

[3]	3GPP TS 29.228: "IP Multimedia (IM) Subsystem Cx and Dx interfaces; Signalling flows and message contents".

[4]	3GPP TS 24.008: "Mobile radio interface Layer 3 specification".

[5]	3GPP TS 29.213: "Policy and charging control signalling flows and Quality of Service (QoS) parameter mapping".

[6]	3GPP TS 29.212: "Policy and Charging Control (PCC); Reference points".

[7]	3GPP TS 29.214: "Policy and Charging Control over Rx reference point".

[8]	3GPP TS 29.060: "General Packet Radio Service (GPRS); GPRS Tunnelling Protocol (GTP) across the Gn and Gp interface".

[9]	3GPP TS 29.061: "Interworking between the Public Land Mobile Network (PLMN) supporting packet based services and Packet Data Networks (PDN)".

[10]	3GPP TS 29.274: "3GPP Evolved Packet System. Evolved GPRS Tunnelling Protocol for EPS (GTPv2)".

[11]	IETF RFC 3361: "Dynamic Host Configuration Protocol (DHCP-for-IPv4) Option for Session Initiation Protocol (SIP) Servers".

[12]	IETF RFC 1034: "Session Initiation Protocol (SIP): Locating SIP Servers".

[13]	IETF RFC 3319: "Dynamic Host Configuration Protocol (DHCPv6) Options for Session Initiation Protocol (SIP) Servers".

[14]	IETF RFC 6223: "Indication of Support for Keep-Alive".

[15]	3GPP TS 29.275: "Proxy Mobile IPv6 (PMIPv6) based Mobility and Tunnelling protocols; Stage 3".

[16]	IETF RFC 7077: "Update Notifications for Proxy Mobile IPv6".

[17]	3GPP TS 23.401: "GPRS Enhancements for E-UTRAN Access".

[18]	IETF RFC 3261: "SIP: Session Initiation Protocol".

[19]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[20]	3GPP TS 23.060: "General Packet Radio Service (GPRS); Service description; Stage 2".

[21]	GSMA PRD IR.65: "IMS Roaming and Interworking Guidelines, version 14.0".

[22]	3GPP TS 24.244: "Wireless LAN control plane protocol for trusted WLAN access to EPC; Stage 3".

[23]	IETF RFC 7296: "Internet Key Exchange Protocol Version 2 (IKEv2)".

[24]	3GPP TS 29.273: "Evolved Packet System (EPS); 3GPP EPS AAA interfaces".

[25]	IETF RFC 3588: "Diameter Base Protocol)".

[26]	IETF RFC 5996 (September 2010): "Internet Key Exchange Protocol Version 2 (IKEv2)".

[27]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[28]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[29]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[30]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System; Stage 2".

[31]	3GPP TS 29.503: "5G System; Unified Data Management Services; Stage 3".

[32]	3GPP TS 29.512: "5G System; Session Management Policy Control Service; Stage 3".

[33]	3GPP TS 23.632: "User Data Interworking, Coexistence and Migration; Stage 2".

[34]	3GPP TS 24.237: "IP Multimedia Subsystem (IMS) Service Continuity; Stage 3".

[xx]	3GPP TS 29.513: "5G System; Policy and Charging Control signalling flows and QoS parameter mapping; Stage 3"

* * * Next Change * * * *

X.2.3	P-CSCF Receiving EPC/5GC NF failure notification from PCRF/PCF

X.2.3.1	Introduction

The PCRF-based restoration is optionally extended by reusing part of the "HSS-based P-CSCF restoration for 3GPP access" mechanism described in clause 5.4 when the P-CSCF receives EPC/5GC NF failure notification from PCRF/PCF.

The extension is based on the possibility for the PCRF/PCF detecting whether the PGW-C/SMF fails.

X.2.3.2	Description

Figure X.2.3.2-1: P-CSCF Receiving EPC/5GC NF failure notification from PCRF/PCF

1.	The terminating I/S-CSCF receives a SIP message for a destination UE and forwards the SIP message to this called UE's terminating P-CSCF.

2.	When the P-CSCF (acting as the AF) determines that session related media requires PCC supervision, or the access network information (i.e. user location and/or user timezone information) is required, the P-CSCF in terminating network sends a Rx Authorization-Authentication-Request (AAR) to the PCRF as specified in 3GPP TS 29.214 [7] or invokes the Npcf_PolicyAuthorization_Create service operation to the PCF as specified in 3GPP TS 29.513 [xx].

3.	The PCRF/PCF returns the Rx Authorization-Authentication-Answer (AAA) or the Npcf_PolicyAuthorization_Create response to the P-CSCF.

4a.	The PCRF/PCF decides to monitor the status of CN nodes. If the PCRF/PCF has previously stored the failure status of the CN failure for the same PGW-C/SMF and the status is still valid, step 4a-4d are skipped.

NOTE:	The PCRF/PCF decides to monitor the status of CN nodes based on the operator's configuration. Frequent CN node failure detection need be avoided. For example, the PCRF/PCF is configured to count the number of Rx sessions associated with the same PGW-C/SMF or the PCRF/PCF triggers the detection periodically.

4b.	PCRF/PCF sends a Gx Re-Auth-Request (RAR) or invokes the Npcf_SMPolicyControl_UpdateNotify service operation to the PGW-C/SMF.

4c.	If the PCRF/PCF receives no response from the PGW-C/SMF, it may determine the service unavailability of the PGW-C/SMF.

If the PGW-C/SMF detects some errors in the user plane, it may send a response to the PCRF/PCF with service unavailability indication which is considered as the case of EPC/5GC NF failure.

4d.	If the EPC/5GC NF failure is detected in 4a, 4b or 4c, the PCRF/PCF may store the failure status of the CN failure of the same PGW-C/SMF for future usage e.g. to avoid triggering Gx RAR message frequently. If any request or response with success code is received from the same PGW-C/SMF, the PCRF/PCF will update the status of the PGW-C/SMF.

5.	If the EPC/5GC NF failure is detected in steps 4a-4d, the PCRF/PCF sends Rx Abort-Session-Request (ASR) or invokes the Npcf_PolicyAuthorization_Notify service operation to the P-CSCF including the service unavailability indication.

6.	Based on this indication, P-CSCF decides to send a response with an error code (i.e. the UE registration data is not present) to the S-CSCF or not to send any response.

When the S-CSCF receives the error code (i.e. the UE registration data is not present) or no response, it considers the case as the called UE's terminating P-CSCF is not able to process this request. Then the steps 4-8 of the HSS based P-CSCF restoration procedure as described in clause 5.4.2.1 are executed.

* * * End of Change * * * *