3GPP TSG-CT WG4 Meeting #129	C4-252439

# Bratislava, Slovakia; 19th – 23rd May 2025									revision of C4-252408

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.518 | CR | 1213 | rev | 2 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of UE access behavior trend and UE location trend events reports |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Nokia, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There are two major problems in the current implementation of the UE access behaviour trends and UE location trends event exposure in TS 29.518. See the discussion paper in C4-252216 for more detailed analysis. Issue 1: In Table 6.2.6.2.29-1 and Table 6.2.6.2.30-1 of TS 29.518, the average and variance are represented by a single mandatory attribute of type "DurationSec". Issue 2: Mobility disrupts the calculation of the statistical data. When the UE moves from source AMF to the target AMF, the source AMF removes the UE information, while the target AMF does not receive information of the statistics of the event and re-starts the calculation of average and variance from the beginning. Considering the long periods for the UE trends reports, handover and change of AMF may easily happen. The target AMF receives the subscription information from the source AMF as part of the UE context but can only start calculating the statistical data from the beginning. Meanwhile, the UE may leave the serving area of the new AMF, before end of the reporting period. Even if at some point in time, an AMF finally generates the event report and notifies the subscriber (e.g. NWDAF), the reported values for the statistical data are different from the average and variance values for the reporting interval. It is proposed to apply the following changes to fix the problems: - Add a separate attribute for the variance with the data type Float (similar to the approach in TS 29.520). - Include UE Access Behavior Trends and UE Location Trends in the eventSubscriptionList in the UE context that is transferred from source AMF to target AMF. - Add the number of occurrences of the event so far to the UE Access Behavior Trends and UE Location Trends information in order to enable the target AMF to update the average and variance to new values when the event occurs again. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the description of the IE eventSubscriptionList in the UE context. - Add ueAccessBehaviorTrends and ueLocationTrends to AmfEventSubscriptionAddInfo. - Add separate attributes for the variances with the data type Float in Type: UeAccessBehaviorReportItem and Type: UeLocationTrendsReportItem. - Add the number of occurrences of the event so far to in Type: UeAccessBehaviorReportItem and Type: UeLocationTrendsReportItem. Udate the OpenAPI accordingly. Difference with Rel-17 CAT F CR (CR#1211): some of the corrections in clause 6.1.6.1 are not applicable to Rel-19 CR, because they are either already corrected in Rel-19 specification or are included in the agreed CR#1193 from CT4#128. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The event report for the UE access behaviour trends and UE location trends event exposure is not working. Issue 1: It is incorrect to include both average and variance in one attribute with single value, and furthermore this attribute is mandatory and cannot be left unprovided. Issue 2: Mobility disrupts the calculation of the statistical data. Even if at some point in time, an AMF finally generates the event report and notifies the subscriber (e.g. NWDAF), the reported values for the statistical data are different from the actual average and variance values for the reporting interval. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.2.1.1, 5.2.2.2.3.1, 6.1.6.1, 6.1.6.2.25, 6.1.6.2.66, 6.2.6.1, 6.2.6.2.29, 6.2.6.2.30, A.2, A.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible correction to the following APIs: TS29518_Namf_Communication.yaml TS29518_Namf_EventExposure.yaml TS29520_Nnwdaf_AnalyticsInfo.yaml TS29520_Nnwdaf_DataManagement.yaml TS29520_Nnwdaf_RoamingData.yaml TS29574_Ndccf_DataManagement.yaml TS29575_Nadrf_DataManagement.yaml TS29576_Nmfaf_3caDataManagement.yaml TS29576_Nmfaf_ContextManagement.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: - Nokia and Ericsson added as co-source. - Discriptions of the IEs updated. - Variance attributes are changed from optional to conditional and a text is added to description for more clarification. Rev2: - Description of the attribute "occurrences" in Table 6.2.6.2.29-1 and Table 6.2.6.2.30-1 is updated. |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the Namf_Communication service based interface protocol.

Table 6.1.6.1-1: Namf_Communication specific Data Types

| Data type | Clause defined | Description |
| --- | --- | --- |
| SubscriptionData | 6.1.6.2.2 | Data within an AMF Status Change Subscription request and response. |
| AmfStatusChangeNotification | 6.1.6.2.3 | Data within an AMF Status Change Notification request. |
| AmfStatusInfo | 6.1.6.2.4 | AMF Status Information |
| AssignEbiData | 6.1.6.2.5 | Data within an EBI assignment request. |
| AssignedEbiData | 6.1.6.2.6 | Data within a successful response to an EBI assignment request. |
| AssignEbiFailed | 6.1.6.2.7 | Represents failed assignment of EBI(s) |
| UEContextRelease | 6.1.6.2.8 | Data within a Release UE Context request. |
| N2InformationTransferReqData | 6.1.6.2.9 | Data within a N2 Information Transfer request containing the N2 information requested to be transferred to 5G AN. |
| NonUeN2InfoSubscriptionCreateData | 6.1.6.2.10 | Data within a create subscription requestfor non-UE specific N2 information notification. |
| NonUeN2InfoSubscriptionCreatedData | 6.1.6.2.11 | Data for the created subscription for non-UE specific N2 information notification. |
| UeN1N2InfoSubscriptionCreateData | 6.1.6.2.12 | Data within a create subscription request for UE specific N1 and/or N2 information notification. |
| UeN1N2InfoSubscriptionCreatedData | 6.1.6.2.13 | Data for the created subscription for UE specific N1 and/or N2 information notification. |
| N2InformationNotification | 6.1.6.2.14 | Data within a N2 information notification request. |
| N2InfoContainer | 6.1.6.2.15 | N2 information container. |
| N1MessageNotification | 6.1.6.2.16 | Data within a N1 message notification request. |
| N1MessageContainer | 6.1.6.2.17 | N1 Message Container. |
| N1N2MessageTransferReqData | 6.1.6.2.18 | Data within a N1/N2 message transfer request. |
| N1N2MessageTransferRspData | 6.1.6.2.19 | Data within a N1/N2 message transfer response. |
| RegistrationContextContainer | 6.1.6.2.20 | Registration Context Container used to send the UE context information, N1 message from UE, AN address etc during Registration with AMF re-allocation procedure. |
| AreaOfValidity | 6.1.6.2.21 | Area of validity information for N2 information transfer |
| UeContextTransferReqData | 6.1.6.2.23 | Data within a UE Context Transfer Request to start transferring of an individual ueContext resource from old AMF to new AMF. |
| UeContextTransferRspData | 6.1.6.2.24 | Data within a successful response to the UE Context Transfer request. |
| UeContext | 6.1.6.2.25 | Represents an individual ueContext resource |
| N2SmInformation | 6.1.6.2.26 | Represents the session management SMF related N2 information data part. |
| N2InfoContent | 6.1.6.2.27 | Represents a transparent N2 information content to be relayed by AMF. |
| NrppaInformation | 6.1.6.2.28 | Represents a NRPPa related N2 information data part. |
| PwsInformation | 6.1.6.2.29 | Represents a PWS related information data part. |
| N1N2MsgTxfrFailureNotification | 6.1.6.2.30 | Data within a N1/N2 Message Transfer Failure Notification request |
| N1N2MessageTransferError | 6.1.6.2.31 | Data within a N1/N2 Message Transfer Error response. |
| N1N2MsgTxfrErrDetail | 6.1.6.2.32 | N1/N2 Message Transfer Error Details |
| N2InformationTransferRspData | 6.1.6.2.33 | Data within a successful response to the N2 Information Transfer request to transfer N2 Information to the AN. |
| MmContext | 6.1.6.2.34 | Represents a Mobility Management Context in UE Context |
| SeafData | 6.1.6.2.35 | Represents SEAF data derived from data received from AUSF |
| NasSecurityMode | 6.1.6.2.36 | Indicates the NAS Security Mode |
| PduSessionContext | 6.1.6.2.37 | Represents a PDU Session Context in UE Context |
| NssaiMapping | 6.1.6.2.38 | Represents the mapping between a S-NSSAI in serving PLMN to a S-NSSAI in home PLMN. |
| UeRegStatusUpdateReqData | 6.1.6.2.39 | Data within a UE registration status update request to indicate a completion of transferring at a target AMF. |
| AssignEbiError | 6.1.6.2.40 | Data within a failure response to the EBI assignment request. |
| UeContextCreateData | 6.1.6.2.41 | Data within a request to create an individual ueContext resource |
| UeContextCreatedData | 6.1.6.2.42 | Data within a successful response for creating an individual ueContext resource |
| UeContextCreateError | 6.1.6.2.43 | Data within a failure response for creating a UE context |
| NgRanTargetId | 6.1.6.2.44 | Indicates a NG RAN as target of the handover |
| N2InformationTransferError | 6.1.6.2.45 | Data within a failure response for a non-UE related N2 Information Transfer. |
| PWSResponseData | 6.1.6.2.46 | Data related PWS included in a N2 Information Transfer response. |
| PWSErrorData | 6.1.6.2.47 | Data related to PWS error included in a N2 Information Transfer failure response. |
| NgKsi | 6.1.6.2.49 | Represents the ngKSI (see 3GPP TS 33.501 [27]) |
| KeyAmf | 6.1.6.2.50 | Represents the Kamf or K'amf. (see 3GPP TS 33.501 [27]). |
| ExpectedUeBehavior | 6.1.6.2.51 | Represents the expected UE behavior (e.g. UE moving trajectory) and its validity period. |
| UeRegStatusUpdateRspData | 6.1.6.2.52 | Data within a UE registration status update response to provide the status of UE context transfer status update at a source AMF. |
| N2RanInformation | 6.1.6.2.53 | Represents the RAN related N2 information data part. |
| N2InfoNotificationRspData | 6.1.6.2.54 | Data within a N2 information notification response. |
| SmallDataRateStatusInfo | 6.1.6.2.55 | Represents the small data rate status |
| SmfChangeInfo | 6.1.6.2.56 | SMF change information for PDU session(s) |
| V2xContext | 6.1.6.2.57 | Represents the V2X services related parameters |
| ImmediateMdtConf | 6.1.6.2.58 | Immediate MDT Configuration |
| V2xInformation | 6.1.6.2.59 | V2X related N2 information |
| EpsNasSecurityMode | 6.1.6.2.60 | Indicates the EPS NAS Security Mode |
| UeContextRelocateData | 6.1.6.2.61 | Data within a Relocate UE Context request |
| UeContextRelocatedData | 6.1.6.2.62 | Data within a Relocate UE Context |
| EcRestrictionDataWb | 6.1.6.2.64 | Enhanced Coverage Restriction Data for WB-N1 mode. |
| ExtAmfEventSubscription | 6.1.6.2.65 | AMF event subscription extended with additional information received for the subscription |
| AmfEventSubscriptionAddInfo | 6.1.6.2.66 | Additional information received for an AMF event subscription, e.g. binding indications. |
| UeContextCancelRelocateData | 6.1.6.2.67 | Data structure used for cancellation of UE Context Relocation. |
| UeDifferentiationInfo | 6.1.6.2.68 | Represents the UE Differentiation Information and its validity time. |
| CeModeBInd | 6.1.6.2.69 | CE-mode-B Support Indicator |
| LteMInd | 6.1.6.2.70 | LTE-M Indication |
| NpnAccessInfo | 6.1.6.2.71 | NPN Access Information |
| ProseContext | 6.1.6.2.72 | Represents the ProSE services related parameters |
| AnalyticsSubscription | 6.1.6.2.73 | Analytics subscriptions created in the NWDAF. |
| NwdafSubscription | 6.1.6.2.74 | Individual NWDAF subscription identified by the subscription Id. |
| UpdpSubscriptionData | 6.1.6.2.75 | UE policy delivery related N1 message notification subscription data |
| ProSeInformation | 6.1.6.2.76 | 5G ProSe related N2 information. |
| ReleaseSessionInfo | 6.1.6.2.77 | PDU session Id(s) and the cause for triggering the release |
| AreaOfInterestEventState | 6.1.6.2.78 | Area Of Interest Event State in old AMF |
| TssInformation | 6.1.6.2.79 | TSS related N2 information |
| AmPolicyInfoContainer | 6.1.6.2.80 | AM Policy Information Container |
| RslpInformation | 6.1.6.2.81 | Ranging/SL positioning related N2 information |
| A2xContext | 6.1.6.2.82 | Represents the A2X services related parameters |
| A2xInformation | 6.1.6.2.83 | A2X related N2 information |
| LcsUpContext | 6.1.6.2.84 | Represents the LCS UP related parameters |
| DeregInactTimerInfo | 6.1.6.2.85 | Network Slice Deregistration Inactivity Timer Information |
| TssRspPerNgran | 6.1.6.2.86 | TSS Information from NG-RAN |
| SliceReplacementMapping | 6.1.6.2.87 | Represents the mapping between a replaced S-NSSAI in serving PLMN to its alternative S-NSSAI. |
| SliceDeregInactConfig | 6.1.6.2.88 | Deregistration inactivity configuration |
| NrppaRspPerNgran | 6.1.6.2.89 | NRPPa Information related to failed NG-RAN |
| EpsBearerId | 6.1.6.3.2 | EPS Bearer Identifier |
| Ppi | 6.1.6.3.2 | Paging Policy Indicator |
| NasCount | 6.1.6.3.2 | Represents a NAS COUNT |
| 5GMmCapability | 6.1.6.3.2 | Represents a 5GMM capability |
| UeSecurityCapability | 6.1.6.3.2 | Represents a UE Security Capability |
| S1UeNetworkCapability | 6.1.6.3.2 | Represents a S1 UE Network Capability |
| DrxParameter | 6.1.6.3.2 | Indicates the UE DRX Parameters |
| OmcIdentifier | 6.1.6.3.2 | Represents the OMC Identifier |
| MSClassmark2 | 6.1.6.3.2 | Indicates the MS Classmark 2 of a 5G SRVCC UE |
| SupportedCodec | 6.1.6.3.2 | Indicates the supported codec of a 5G SRVCC UE |
| UePolicyContainer | 6.1.6.3.2 | Information about the UE policy in NAS |
| StatusChange | 6.1.6.3.3 | Enumeration for AMF status |
| N2InformationClass | 6.1.6.3.4 | Enumeration for N2 Information Class |
| N1MessageClass | 6.1.6.3.5 | Enumeration for N1 Message Class |
| N1N2MessageTransferCause | 6.1.6.3.6 | Enumeration for N1N2Message Transfer Cause |
| UeContextTransferStatus | 6.1.6.3.7 | Describes the status of an individual ueContext resource in UE Context Transfer procedures |
| N2InformationTransferResult | 6.1.6.3.8 | Describes the result of N2 information transfer by AMF to the AN. |
| CipheringAlgorithm | 6.1.6.3.9 | Indicates the supported Ciphering Algorithm |
| IntegrityAlgorithm | 6.1.6.3.10 | Indicates the supported Integrity Algorithm |
| ScType | 6.1.6.3.12 | Indicates the security context type. |
| KeyAmfType | 6.1.6.3.13 | Indicates the Kamf type. |
| TransferReason | 6.1.6.3.14 | Indicates UE Context Transfer Reason |
| PolicyReqTrigger | 6.1.6.3.15 | Policy Request Triggers |
| RatSelector | 6.1.6.3.16 | Indicates the RAT type for the transfer of N2 information |
| NgapIeType | 6.1.6.3.17 | Indicates the supported NGAP IE types |
| N2InfoNotifyReason | 6.1.6.3.18 | N2 Information Notify Reason |
| SmfChangeIndication | 6.1.6.3.19 | Indicates the I-SMF or V-SMF change or removal |
| SbiBindingLevel | 6.1.6.3.20 | SBI Binding Level |
| EpsNasCipheringAlgorithm | 6.1.6.3.21 | Indicates the supported EPS NAS Ciphering Algorithm |
| EpsNasIntegrityAlgorithm | 6.1.6.3.22 | Indicates the supported EPS NAS Integrity Algorithm |
| PeriodicCommunicationIndicator | 6.1.6.3.23 | Indicates the Periodic Communication Indicator |
| UuaaMmStatus | 6.1.6.3.24 | Indicates UUAA-MM status |
| ReleaseCause | 6.1.6.3.25 | The cause for triggering the release |
| NgranFailureInfo | 6.1.6.3.26 | Information related to a NG-RAN failure |
| XrDeviceWith2Rx | 6.1.6.3.27 | XR Device with 2Rx information |
| NrppaTransFailure | 6.1.6.3.28 | Cause for NRPPa transfer failure |

Table 6.1.6.1-2 specifies data types re-used by the Namf_Communication service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Namf_Communication service based interface.

Table 6.1.6.1-2: Namf_Communication re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| Snssai | 3GPP TS 29.571 [6] |  |
| Arp | 3GPP TS 29.571 [6] |  |
| PduSesisonId | 3GPP TS 29.571 [6] |  |
| Guami | 3GPP TS 29.571 [6] | Globally Unique AMF Identifier |
| AmfName | 3GPP TS 29.571 [6] | The name of the AMF |
| Supi | 3GPP TS 29.571 [6] | Subscription Permanent Identifier |
| Cause | 3GPP TS 29.571 [6] | 5G-AN Cause |
| ProblemDetails | 3GPP TS 29.571 [6] | Detailed problems in failure case |
| supportedFeatures | 3GPP TS 29.571 [6] | Supported Features |
| TimeZone | 3GPP TS 29.571 [6] |  |
| UserLocation | 3GPP TS 29.571 [6] |  |
| AccessType | 3GPP TS 29.571 [6] |  |
| AllowedNssai | 3GPP TS 29.531 [18] |  |
| NfInstanceId | 3GPP TS 29.571 [6] |  |
| Uri | 3GPP TS 29.571 [6] |  |
| Ecgi | 3GPP TS 29.571 [6] | EUTRA Cell Identifier |
| Ncgi | 3GPP TS 29.571 [6] | NR Cell Identifier |
| Uint16 | 3GPP TS 29.571 [6] |  |
| 5Qi | 3GPP TS 29.571 [6] | 5G QoS Identifier |
| CorrelationID | 3GPP TS 29.572 [25] | LCS Correlation ID |
| Pei | 3GPP TS 29.571 [6] |  |
| Dnn | 3GPP TS 29.571 [6] |  |
| Gpsi | 3GPP TS 29.571 [6] |  |
| GroupId | 3GPP TS 29.571 [6] |  |
| PlmnId | 3GPP TS 29.571 [6] |  |
| RfspIndex | 3GPP TS 29.571 [6] |  |
| EbiArpMapping | 3GPP TS 29.502 [16] | EBI - ARP mapping |
| NsiId | 3GPP TS 29.531 [18] |  |
| TraceData | 3GPP TS 29.571 [6] | Trace control and configuration parameters |
| ConfiguredSnssai | 3GPP TS 29.531 [18] |  |
| NgApCause | 3GPP TS 29.571 [6] | Represents the NG AP cause IE |
| Area | 3GPP TS 29.571 [6] |  |
| ServiceAreaRestriction | 3GPP TS 29.571 [6] |  |
| CoreNetworkType | 3GPP TS 29.571 [6] |  |
| Ambr | 3GPP TS 29.571 [6] |  |
| SliceMbr | 3GPP TS 29.571 [6] |  |
| GlobalRanNodeId | 3GPP TS 29.571 [6] |  |
| NfGroupId | 3GPP TS 29.571 [6] | Network Function Group Id |
| DurationSec | 3GPP TS 29.571 [6] |  |
| StnSr | 3GPP TS 29.571 [6] | Session Transfer Number for SRVCC |
| CMsisdn | 3GPP TS 29.571 [6] | Correlation MSISDN |
| DateTime | 3GPP TS 29.571 [6] |  |
| SmallDataRateStatus | 3GPP TS 29.571 [6] |  |
| NfSetId | 3GPP TS 29.571 [6] | NF Set ID |
| NfServiceSetId | 3GPP TS 29.571 [6] | NF Service Set ID |
| LMFIdentification | 3GPP TS 29.572 [25] | LMF Identification |
| PlmnAssiUeRadioCapId | 3GPP TS 29.571 [6] |  |
| ManAssiUeRadioCapId | 3GPP TS 29.571 [6] |  |
| NrV2xAuth | 3GPP TS 29.571 [6] | NR V2X services authorized |
| LteV2xAuth | 3GPP TS 29.571 [6] | LTE V2X services authorized |
| BitRate | 3GPP TS 29.571 [6] | Bit Rate |
| Pc5QoSPara | 3GPP TS 29.571 [6] | PC5 QoS parameters |
| PduSessionInfo | 3GPP TS 29.571 [6] | The Slice and DNN combination of a PDU session. |
| PcfUeCallbackInfo | 3GPP TS 29.571 [6] | The callback information of the PCF for the UE to allow the PCF for the PDU session to send SM Policy Association Establishment and Termination events notification. |
| CnAssistedRanPara | 3GPP TS 29.502 [16] | SMF derived CN assisted RAN Parameters Tuning |
| MoExpDataCounter | 3GPP TS 29.571 [6] | MO Exception Data Counter |
| CagData | 3GPP TS 29.503 [35] | Closed Access Group Data |
| NssaaStatus | 3GPP TS 29.571 [6] | Subscribed S-NSSAI subject to NSSAA procedure and the status |
| JobType | 3GPP TS 29.571 [6] | Job Type in the trace |
| MeasurementLteForMdt | 3GPP TS 29.571 [6] | Measurements used for MDT in LTE in the trace |
| MeasurementNrForMdt | 3GPP TS 29.571 [6] | Measurements used for MDT in NR in the trace |
| ReportingTrigger | 3GPP TS 29.571 [6] | Reporting Triggers for MDT in the trace |
| ReportIntervalMdt | 3GPP TS 29.571 [6] | Report Interval for MDT in LTE in the trace |
| ReportAmountMdt | 3GPP TS 29.571 [6] | Report Amount for MDT in the trace |
| CollectionPeriodRmmLteMdt | 3GPP TS 29.571 [6] | Collection period for RRM measurements LTE for MDT in the trace |
| MeasurementPeriodLteMdt | 3GPP TS 29.571 [6] | Measurement period LTE for MDT in the trace in |
| AreaScope | 3GPP TS 29.571 [6] | Area Scope |
| PositioningMethodMdt | 3GPP TS 29.571 [6] | Positioning Method for MDT in the trace in LTE |
| ReportIntervalNrMdt | 3GPP TS 29.571 [6] | Report Interval for MDT in NR in the trace |
| CollectionPeriodRmmNrMdt | 3GPP TS 29.571 [6] | Collection period for RRM measurements NR for MDT in the trace |
| SensorMeasurement | 3GPP TS 29.571 [6] | Sensor information for MDT in the trace |
| ScheduledCommunicationTime | 3GPP TS 29.571 [6] | Scheduled Communication Time |
| StationaryIndication | 3GPP TS 29.571 [6] | Stationary Indication |
| TrafficProfile | 3GPP TS 29.571 [6] | Traffic Profile |
| BatteryIndication | 3GPP TS 29.571 [6] | Battery Indication |
| NFType | 3GPP TS 29.510 [29] | NF type |
| UeAuth | 3GPP TS 29.571 [6] | UE authorisation for PC5 service |
| PartitioningCriteria | 3GPP TS 29.571 [6] | Partitioning Criteria |
| RedirectResponse | 3GPP TS 29.571 [6] | Response body of the redirect response message. |
| CagId | 3GPP TS 29.571 [6] | CAG ID |
| NnwdafEventsSubscription | 3GPP TS 29.520 [52] | Represents an Individual NWDAF Event Subscription resource |
| PresenceInfo | 3GPP TS 29.571 [6] |  |
| UePositioningCapabilities | 3GPP TS 29.572 [25] | Indicates the positioning capabilities supported by the UE. |
| SmfSelectionData | 3GPP TS 29.507 [32] |  |
| EpsInterworkingInfo | 3GPP TS 29.503 [35] |  |
| IpAddress | 3GPP TS 29.503 [35] |  |
| Fqdn | 3GPP TS 29.571 [6] |  |
| PresenceState | 3GPP TS 29.571 [6] | Presence State |
| SatelliteBackhaulCategory | 3GPP TS 29.571 [6] | Satellite Backhaul Category |
| WirelineServiceAreaRestriction | 3GPP TS 29.571 [6] |  |
| PartiallyAllowedSnssai | 3GPP TS 29.571 [6] | S-NSSAI with TAIs information |
| VarRepPeriod | 3GPP TS 29.571 [6] | Variable Reporting Periodicity |
| AsTimeDistributionParam | 3GPP TS 29.507 [32] |  |
| PruInd | 3GPP TS 29.503 [35] | PRU Indicator |
| SliceUsageControlInfo | 3GPP TS 29.571 [6] | Slice Usage Control Information |
| MbsrOperationAllowed | 3GPP TS 29.503 [35] | MBSR Operation Information |
| UpConnectionStatus | 3GPP TS 29.572 [25] | User Plane Connection Status |
| LMFIdentification | 3GPP TS 29.572 [25] |  |
| TaiRange | 3GPP TS 29.510 [29] | TAI range |
| BluetoothMeasurement | 3GPP TS 29.571 [6] | Bluetooth Measurement Configuration |
| WlanMeasurement | 3GPP TS 29.571 [6] | WLAN Measurement Configuration |
| PduSessionPriority | 3GPP TS 29.502 [16] | PDU Session Priority |
| UeAccessBehaviorReportItem | Clause 6.2.6.2.29 | Report Item for UE Access Behavior Trends event. |
| UeLocationTrendsReportItem | Clause 6.2.6.2.30 | Report Item for UE Location Trends event. |

* * * Next Change * * * *

##### 6.1.6.2.25	Type: UeContext

Table 6.1.6.2.25-1: Definition of type UeContext

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| supi | Supi | C | 0..1 | This IE shall be present if available. When present, this IE contains SUPI of the UE. |  |
| supiUnauthInd | boolean | C | 0..1 | This IE shall be present if SUPI is present. When present, it shall indicate whether the SUPI is unauthenticated. |  |
| gpsiList | array(Gpsi) | C | 1..N | This IE shall be present if available and if it is not case b) specified in clause 5.2.2.2.1.1 step 2a. When present, this IE shall contain the GPSI(s) of the UE. |  |
| pei | Pei | C | 0..1 | This IE shall be present if available and if it is not case b) specified in clause 5.2.2.2.1.1 step 2a. When present, this IE shall contain Mobile Equipment Identity of the UE. |  |
| ... [non-changing rows skipped for clarity] ... |  |  |  |  |  |
| restrictedCnList | array(CoreNetworkType) | O | 1..N | When present, this IE shall indicate the list of Core Network Types that are restricted for the UE. |  |
| eventSubscriptionList | array(ExtAmfEventSubscription) | C | 1..N | This IE shall be present if available and if it is not case b) specified in clause 5.2.2.2.1.1 step 2a. When present, it shall indicate the event subscription(s) targeting the UE or the group the UE is part of. If the source AMF supports binding procedures and if it received binding indications for event notifications (i.e. with "callback" scope) or for subscription change event notifications (i.e. with "subscription-events" scope) for certain subscriptions, these binding indications should also be included. If the source AMF knows the NF type of the NF that created the subscription, this information should also be indicated. The UE access behavior trends report and/or UE location trends report may be included, if the NF service consumer is subscribed to the UE_ACCESS_BEHAVIOR_TRENDS event and/or UE_LOCATION_TRENDS event respectively. This information is required for calculating the statistical data of trends. |  |
| ... [non-changing rows skipped for clarity] ... |  |  |  |  |  |
| uePolicyInd | boolean | C | 0..1 | This IE shall be present when the UE policy indicator is received from UDM. When present, this IE shall be set to the value received from UDM: - true: indicates that the UE policy association is enabled. - false: indicates that the UE policy association is disabled. |  |
| NOTE 1: If the restrictedPrimaryRatList and restrictedSecondaryRatList attributes are supported by the sender, the sender shall include the list of RAT Types that are restricted, if any, in the restrictedRatList attribute, shall include the list of RAT Types that are restricted for use as primary RAT, if any, in the restrictedPrimaryRatList attribute and shall include the list of RAT Types that are restricted for use as secondary RAT, if any, in the restrictedSsecondaryRatList attribute. If the restrictedPrimaryRatList and restrictedSecondaryRatList attributes are supported by the receiver, the receiver shall use the data in the restrictedPrimaryRatList attribute, if received, as the list of RAT Types that are restricted for use as primary RAT for the UE, and shall use the data in the restrictedSecondaryRatList attribute, if received, as the list of RAT Types that are restricted for use as secondary RAT for the UE, otherwise the receiver shall use the data in the restrictedRatList attribute, if received, as the list of RAT Types that are restricted for the UE. NOTE 2: A particular PDU session not supported by the target AMF shall not be transferred, e.g. MA-PDU session context shall not be transferred if target AMF does not support ATSSS. NOTE 3: After ecRestrictionDataWb and/or ecRestrictionDataNb attributes are sent from source AMF to target AMF to build the UeContext in the target AMF, the target AMF shall re-determine the EC restriction information based on the received subscription data from UDM and UE 5GMM capability because EC restriction information may change (e.g. due to that subscription data in UDM is changed but not notified the old AMF yet) and then compare the re-determined EC restriction information with the one received in the UeContext. If the target AMF finds EC restriction information has changed after comparing, the target AMF shall proceed as described in clause 5.31.12, 3GPP TS 23.501 [2]. NOTE 4: If present, this attribute shall be used together with routingIndicator. This attribute is only used by the HPLMN in roaming scenarios. NOTE 5: If the information as indicated in both IEs were received from the old AMF in UE Context, during PDU session establishment the AMF shall identify whether a non-roaming or local breakout PDU session is applicable for SM Policy Association events, i.e, whethe the slice and DNN combination of the PDU session is listed in the smPolicyNotifyPduList IE or not. If the PDU session is applicable for notification of SM Policy Association events , the AMF shall provide the callback information for the PCF of the UE contained in the pcfUeCallbackInfo IE to the SMF of a new PDU session via Create SM Context service operation together with the indication for notification of SM Policy Association events. See clause 4.3.2.2.1 of 3GPP TS 23.502 [3]. NOTE 6: This IE is deprecated. An AMF complying with this version of specification shall use the pcfAmpBindingInfo IE to carry the Binding indication of the AM Policy Association resource and use the pcfUepBindingInfo IE to carry the binding indication of the UE Policy Association resource. |  |  |  |  |  |

* * * Next Change * * * *

##### 6.1.6.2.66	Type: AmfEventSubscriptionAddInfo

Table 6.1.6.2.66-1: Definition of type AmfEventSubscriptionAddInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| bindingInfo | array(string) | O | 1..2 | Binding indications received for event notifications (i.e. with "callback" scope) or for subscription change event notifications (i.e. with "subscription-events" scope) for an AMF event subscription. When present, entries of the array shall be set to the value of the 3gpp-Sbi-Binding header defined in clause 5.2.3.2.6 of 3GPP TS 29.500 [4], without the header name. Example of an array entry: "bl= nf-set; nfset=set1.udmset.5gc.mnc012.mcc345; servname=nudm-ee;scope=subscription-events" |
| subscribingNfType | NFType | C | 0..1 | This IE should be present if the information is available. When present, it shall contain the NF type of the NF that created the subscription. (NOTE 1) |
| eventSyncInd | boolean | C | 0..1 | This IE should be present with value "true" when the event subscription shall be synchronized with UDM during EPS to 5GS mobility registration procedure, as specified in clause 5.3.2.4.2. When present, this IE shall be set as following: - true: the event subscription shall be synchronized with UDM. - false: the event subscription shall not be synchronized with UDM. |
| nfConsumerInfo | array(string) | C | 1..N | When present, this IE shall contain the NF Service Consumer information received together with the AMF event subscription and entries of the array shall be set to the value of the 3gpp-Sbi-Consumer-Info header defined in clause 5.2.3.3.7 of 3GPP TS 29.500 [4], without the header name. |
| aoiStateList | map(AreaOfInterestEventState) | O | 1..N | Map of subscribed Area of Interest (AoI) Event State in the old AMF. For the subscribed AoI Event(s), the JSON pointer to an AmfEventArea element in the areaList IE (or a PresenceInfo element in presenceInfoList IE) of the AmfEvent data type (see clause 6.2.6.2.3) shall be the key of the map. (NOTE 2) |
| accessToken | string | O | 0..1 | See clause 13.4.1.4 of 3GPP TS 33.501 [27], and clause 5.2.2.2.1.1 and 5.2.2.2.3.1 for the use of this IE. When present, this IE shall contain a JWS Compact Serialized representation of the JWS signed JSON object containing AccessTokenClaims (see clause 6.3.5.2.4 of 3GPP TS 29.510 [29]) that was received by the source AMF for authorizing the subscription. |
| amfSetLevelBulkSubs | boolean | C | 0..1 | This IE shall be present and set to true if the source AMF supports the AMF Set level Bulk Subscription feature (see Table 6.2.8-1), the subscription is a subscription for a group of UEs and this is an AMF Set level bulk subscription. The presence of this IE with the false value shall be prohibited. |
| ueAccessBehaviorTrends | array(UeAccessBehaviorReportItem) | O | 1..N | When present, this IE shall contain the UE access behavior trend information collected and calculated by the AMF. This IE may be present when there is a subscription for the event "UE_ACCESS_BEHAVIOR_TRENDS" and the event subscription is transferred from source AMF to a target AMF and at least one occurrence of the event is already available in the source AMF. |
| ueLocationTrends | array(UeLocationTrendsReportItem) | O | 1..N | When present, this IE shall contain the UE location trend information collected and calculated by the AMF. This IE may be present when there is a subscription for the event "UE_LOCATION_TRENDS" and the event subscription is transferred from source AMF to a target AMF and at least one occurrence of the event is already available in the source AMF. |
| NOTE 1: In scenarios where an "intermediate NF" (e.g. UDM) creates a subscription on behalf of a "source NF" (e.g. NEF), this IE shall contain the NF type of the "intermediate NF". The NF type of the "source NF" may be available in the AmfEventSubscription. NOTE 2: The new AMF may use the information to determine whether the UE presence state in the AOI(s) has changed, thus should be reported to the NF consumer. |  |  |  |  |

* * * Next Change * * * *

#### 6.2.6.1	General

This clause specifies the application data model supported by the API.

Table 6.2.6.1-1 specifies the data types defined for the Namf_EventExposure service based interface protocol.

Table 6.2.6.1-1: Namf_EventExposure specific Data Types

| Data type | Clause defined | Description |
| --- | --- | --- |
| AmfEventSubscription | 6.2.6.2.2 | Represents an individual event subscription resource on AMF |
| AmfEvent | 6.2.6.2.3 | Describes an event to be subscribed |
| AmfEventNotification | 6.2.6.2.4 | Data within an AMF Event Notification request. |
| AmfEventReport | 6.2.6.2.5 | Represents a report triggered by a subscribed event type |
| AmfEventMode | 6.2.6.2.6 | Describes how the reports shall be generated by a subscribed event |
| AmfEventState | 6.2.6.2.7 | Represents the state of a subscribed event |
| RmInfo | 6.2.6.2.8 | Represents the registration state of a UE for an access type |
| CmInfo | 6.2.6.2.9 | Represents the connection management state of a UE for an access type |
| CommunicationFailure | 6.2.6.2.11 | Describes a communication failure detected by AMF |
| AmfCreateEventSubscription | 6.2.6.2.12 | Data within a create AMF event subscription request |
| AmfCreatedEventSubscription | 6.2.6.2.13 | Data within a create AMF event subscription response |
| AmfUpdateEventSubscriptionItem | 6.2.6.2.14 | Document describing the modification(s) to an AMF Event Subscription |
| AmfUpdatedEventSubscription | 6.2.6.2.15 | Represents a successful update on an AMF Event Subscription |
| AmfEventArea | 6.2.6.2.16 | Represents an area to be monitored by an AMF event. |
| LadnInfo | 6.2.6.2.17 | LADN Information |
| AmfUpdateEventOptionItem | 6.2.6.2.18 | Document describing the modifications to AMF event subscription options. |
| 5GsUserStateInfo | 6.2.6.2.19 | Represents the 5GS User state of the UE for an access type |
| TrafficDescriptor | 6.2.6.2.20 | Represents the Traffic Descriptor |
| UEIdExt | 6.2.6.2.21 | UE Identity |
| AmfEventSubsSyncInfo | 6.2.6.2.22 | AMF Event Subscriptions Information for synchronization |
| AmfEventSubscriptionInfo | 6.2.6.2.23 | Individual AMF Event Subscription Information |
| TargetArea | 6.2.6.2.24 | TA list or TAI range list or any TA |
| SnssaiTaiMapping | 6.2.6.2.25 | List of restricted or unrestricted S-NSSAIs per TAI(s) |
| SupportedSnssai | 6.2.6.2.26 | Supported S-NSSAIs |
| UeInAreaFilter | 6.2.6.2.27 | Describe the filter related to UEs In Area Report event. |
| IdleStatusIndication | 6.2.6.2.28 | Represents the idle status indication. |
| UeAccessBehaviorReportItem | 6.2.6.2.29 | Report Item for UE Access Behavior Trends event. |
| UeLocationTrendsReportItem | 6.2.6.2.30 | Report Item for UE Location Trends event. |
| DispersionArea | 6.2.6.2.31 | Dispersion Area |
| MmTransactionLocationReportItem | 6.2.6.2.32 | UE MM Transaction Report Item per Location |
| MmTransactionSliceReportItem | 6.2.6.2.33 | UE MM Transaction Report Item per Slice |
| SliceAreaRestrictionInfo | 6.2.6.2.34 | Information of a slice subject to area restriction |
| ScheduledArea | 6.2.6.2.35 | Scheduled Area |
| TrajectorySegment | 6.2.6.2.36 | Trajectory Segment |
| Trajectory | 6.2.6.2.37 | Trajectory |
| SmCommFailureFilter | 6.2.6.2.38 | Filter for Session Management Communication Failure |
| SignallingMeasurementInfo | 6.2.6.2.39 | AMF Signalling Measurement Information |
| AmfEventType | 6.2.6.3.3 | Describes the supported event types of Namf_EventExposure Service |
| AmfEventTrigger | 6.2.6.3.4 | Describes how AMF should generate the report for the event |
| LocationFilter | 6.2.6.3.5 | Describes the supported filters of LOCATION_REPORT or UE_LOCATION_TRENDS event type |
| UeReachability | 6.2.6.3.7 | Describes the reachability of the UE |
| RmState | 6.2.6.3.9 | Describes the registration management state of a UE |
| CmState | 6.2.6.3.10 | Describes the connection management state of a UE |
| 5GsUserState | 6.2.6.3.11 | Describes the 5GS User State of a UE |
| LossOfConnectivityReason | 6.2.6.3.12 | Describes the reason for loss of connectivity |
| ReachabilityFilter | 6.2.6.3.13 | Event filter for REACHABILITY_REPORT event type. |
| UeType | 6.2.6.3.14 | Describe UE type |
| AccessStateTransitionType | 6.2.6.3.15 | Access State Transition Type |
| SubTerminationReason | 6.2.6.3.16 | Subscription Termination Reason |
| TrajectoryReportReason | 6.2.6.3.17 | Trajectory Tracking Event Report Reason |

Table 6.2.6.1-2 specifies data types re-used by the Namf_EventExposure service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Namf_EventExposure service based interface.

Table 6.2.6.1-2: Namf_EventExposure re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| Supi | 3GPP TS 29.571 [6] |  |
| GroupId | 3GPP TS 29.571 [6] |  |
| DurationSec | 3GPP TS 29.571 [6] |  |
| Gpsi | 3GPP TS 29.571 [6] |  |
| Uri | 3GPP TS 29.571 [6] |  |
| Pei | 3GPP TS 29.571 [6] |  |
| UserLocation | 3GPP TS 29.571 [6] |  |
| Tai | 3GPP TS 29.571 [6] |  |
| TimeZone | 3GPP TS 29.571 [6] |  |
| AccessType | 3GPP TS 29.571 [6] |  |
| Ecgi | 3GPP TS 29.571 [6] | EUTRA Cell Identifier |
| Ncgi | 3GPP TS 29.571 [6] | NR Cell Identifier |
| NfInstanceId | 3GPP TS 29.571 [6] |  |
| ProblemDetails | 3GPP TS 29.571 [6] | Problem Details |
| SupportedFeatures | 3GPP TS 29.571 [6] | Supported Features |
| DateTime | 3GPP TS 29.571 [6] |  |
| NgApCause | 3GPP TS 29.571 [6] |  |
| PresenceInfo | 3GPP TS 29.571 [6] | Presence Reporting Area Information |
| PresenceState | 3GPP TS 29.571 [6] | Describes the presence state of the UE to a specified area of interest |
| Dnn | 3GPP TS 29.571 [6] |  |
| Snssai | 3GPP TS 29.571 [6] |  |
| DddTrafficDescriptor | 3GPP TS 29.571 [6] | Downlink Data Delivery Traffic Descriptor |
| SamplingRatio | 3GPP TS 29.571 [6] | Sampling Ratio. |
| RedirectResponse | 3GPP TS 29.571 [6] | Response body of the redirect response message. |
| NotificationFlag | 3GPP TS 29.571 [6] | Notification flag |
| ExtSnssai | 3GPP TS 29.571 [6] |  |
| N3gaLocation | 3GPP TS 29.571 [6] | Non-3GPP Location |
| SnssaiDnnItem | 3GPP TS 29.571 [6] | Combination of S-NSSAIs and DNNs |
| NfSignallingInfo | 3GPP TS 29.571 [6] | Network Function Signalling Information |
| ReferenceId | 3GPP TS 29.503 [35] |  |
| NsiId | 3GPP TS 29.531 [18] | NSI ID |
| NFType | 3GPP TS 29.510 [29] | NF type |
| TaiRange | 3GPP TS 29.510 [29] |  |
| MutingExceptionInstructions | 3GPP TS 29.571 [6] | Muting exception instructions. |
| MutingNotificationsSettings | 3GPP TS 29.571 [6] | Muting notifications settings. |
| LcsBroadcastAssistanceTypesData | 3GPP TS 29.503 [35] | LCS broadcasting assistance data type which presence a bitmap. |
| Altitude | 3GPP TS 29.572 [25] | Altitude |
| UePositioningCapabilities | 3GPP TS 29.572 [25] | Indicates the positioning capabilities supported by the UE. |
| UeUpPositioningCapabilities | 3GPP TS 29.572 [25] | Indicates the user plane positioning capabilities supported by the UE. |
| TimeWindow | 3GPP TS 29.122 [62] | Time Window |
| Float | 3GPP TS 29.571 [6] | Number with format "float" |

* * * Next Change * * * *

##### 6.2.6.2.29	Type: UeAccessBehaviorReportItem

Table 6.2.6.2.29-1: Definition of type UeAccessBehaviorReportItem

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| stateTransitionType | AccessStateTransitionType | M | 1 | Indicate the type the state transition behavior. |  |
| spacing | DurationSec | M | 1 | Indicates the average of the time interval separating two consecutive occurrences of the state transition as indicated by the stateTransitionType IE. |  |
| spacingVar | Float | C | 0..1 | Indicates the variance of the time interval separating two consecutive occurrences of the state transition as indicated by the stateTransitionType IE. An implementation complying with this version of the specification shall include this IE. |  |
| duration | DurationSec | M | 1 | Indicates the average of duration in the resulting state as indicated by stateTransitionType IE. |  |
| durationVar | Float | C | 0..1 | Indicates the variance of duration in the resulting state as indicated by stateTransitionType IE. An implementation complying with this version of the specification shall include this IE. |  |
| occurrences | integer | C | 0..1 | Represents the number of occurrences of the state transition of type stateTransitionType in the current reporting interval. This information is required for calculating spacing, spacingVar, duration and durationVar after handover to a target AMF. An implementation complying with this version of the specification shall include this IE if the event subscription is transferred from source AMF to a target AMF and at least one occurrence of the event is already available in the source AMF. |  |
| timestamp | DateTime | C | 0..1 | Indicates the date and time of the last occurrences of the indicated state transition. This information is required for calculating spacing, spacingVar, duration and durationVar after handover to a target AMF. An implementation complying with this version of the specification shall include this IE. |  |

* * * Next Change * * * *

##### 6.2.6.2.30	Type: UeLocationTrendsReportItem

Table 6.2.6.2.30-1: Definition of type UeLocationTrendsReportItem

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| tai | Tai | C | 0..1 | Indicates the TAI where the UE arrived. (NOTE) |  |
| ranNodeId | GlobalRanNodeId | C | 0..1 | Indicates the global identifier of the RAN node where the UE arrived. (NOTE) |  |
| ncgi | Ncgi | C | 0..1 | Indicates the NR cell where the UE arrived. (NOTE) |  |
| ecgi | Ecgi | C | 0..1 | Indicates the EUTRAN cell where the UE arrived. (NOTE) |  |
| n3gaLocation | N3gaLocation | C | 0..1 | Indicates the Non-3GPP location where the UE arrived. (NOTE) |  |
| spacing | DurationSec | M | 1 | Indicates the average of the time interval separating two consecutive arrivals at the indicated location. |  |
| spacingVar | Float | C | 0..1 | Indicates the variance of the time interval separating two consecutive arrivals at the indicated location. An implementation complying with this version of the specification shall include this IE. |  |
| duration | DurationSec | M | 1 | Indicates the average of duration of stay in the indicated location. |  |
| durationVar | Float | C | 0..1 | Indicates the variances of duration of stay in the indicated location. An implementation complying with this version of the specification shall include this IE. |  |
| occurrences | integer | C | 0..1 | Represents the number of arrivals at the indicated location in the current reporting interval. This information is required for calculating spacing, spacingVar, duration and durationVar after handover to a target AMF. An implementation complying with this version of the specification shall include this IE if the event subscription is transferred from source AMF to a target AMF and at least one occurrence of the event is already available in the source AMF. |  |
| timestamp | DateTime | M | 1 | Indicates the date and time (in UTC) of UE last arrival to the indicated location. |  |
| NOTE: At least one of the "tai", "ranNodeId", "ncgi", "ecgi" and "n3gaLocation" shall be present. |  |  |  |  |  |

* * * Next Change * * * *

# A.2	Namf_Communication API

openapi: 3.0.0

info:

version: 1.4.0-alpha.3

title: Namf_Communication

description: |

AMF Communication Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

security:

- {}

- oAuth2ClientCredentials:

- namf-comm

externalDocs:

description: 3GPP TS 29.518 V19.2.0; 5G System; Access and Mobility Management Services

url: 'https://www.3gpp.org/ftp/Specs/archive/29_series/29.518/'

[non-changing parts skipped for clarity]

AmfEventSubscriptionAddInfo:

description: >

Additional information received for an AMF event subscription, e.g. binding indications,

statistical information for UE access behavior trends report or UE location trends report.

type: object

properties:

bindingInfo:

type: array

items:

type: string

minItems: 1

maxItems: 2

subscribingNfType:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/NFType'

eventSyncInd:

type: boolean

nfConsumerInfo:

type: array

items:

type: string

minItems: 1

aoiStateList:

type: object

description: >

Map of subscribed Area of Interest (AoI) Event State in the old AMF. The JSON pointer to

an AmfEventArea element in the areaList IE (or a PresenceInfo element in

presenceInfoList IE) of the AmfEvent data type shall be the key of the map.

additionalProperties:

$ref: '#/components/schemas/AreaOfInterestEventState'

accessToken:

type: string

description: >

JWS Compact Serialized representation of JWS signed JSON object (AccessTokenClaims

defined in 3GPP TS 29.510)

amfSetLevelBulkSubs:

type: boolean

enum:

- true

ueAccessBehaviorTrends:

type: array

items:

$ref: 'TS29518_Namf_EventExposure.yaml#/components/schemas/UeAccessBehaviorReportItem'

minItems: 1

ueLocationTrends:

type: array

items:

$ref: 'TS29518_Namf_EventExposure.yaml#/components/schemas/UeLocationTrendsReportItem'

minItems: 1

[non-changing parts skipped for clarity]

* * * Next Change * * * *

# A.3	Namf_EventExposure API

openapi: 3.0.0

info:

version: 1.4.0-alpha.2

title: Namf_EventExposure

description: |

AMF Event Exposure Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

security:

- {}

- oAuth2ClientCredentials:

- namf-evts

externalDocs:

description: 3GPP TS 29.518 V19.2.0; 5G System; Access and Mobility Management Services

url: 'https://www.3gpp.org/ftp/Specs/archive/29_series/29.518/'

[non-changing parts skipped for clarity]

UeAccessBehaviorReportItem:

description: Report Item for UE Access Behavior Trends event.

type: object

properties:

stateTransitionType:

$ref: '#/components/schemas/AccessStateTransitionType'

spacing:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

spacingVar:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Float'

duration:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

durationVar:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Float'

occurrences:

type: integer

timestamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

required:

- stateTransitionType

- spacing

- duration

[non-changing parts skipped for clarity]

UeLocationTrendsReportItem:

description: Report Item for UE Location Trends event.

type: object

properties:

tai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Tai'

ranNodeId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/GlobalRanNodeId'

ncgi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ncgi'

ecgi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ecgi'

n3gaLocation:

$ref: 'TS29571_CommonData.yaml#/components/schemas/N3gaLocation'

spacing:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

spacingVar:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Float'

duration:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

durationVar:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Float'

occurrences:

type: integer

timestamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

required:

- spacing

- duration

- timestamp

[non-changing parts skipped for clarity]

* * * End of Changes * * * *