3GPP TSG-CT WG4 Meeting #101e	C4-205508

# E-Meeting, 3rd – 13th November 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.632 | CR | 0026 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE Reachability for IP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | UDICOM |  |  |  |  |  | Date: |  |  | 2020-11-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | It needs to be clarified that the HSS subscribes at the UDM to a notification for UE Reachability For SMS over IP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that the HSS subscribes to UE Reachability notification with event type UE_REACHABILITY_FOR_SMS and reachability for SMS configuration of REACHABILITYLE_FOR_SMS_OVER_IP |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UDM cannot distinguish between UE Reachability for SMS over NAS and UE Reachability for SMS over IP |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backwards compatible corrections to the Nudm_EE API |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 5.4.5	UE Reachability

Figure 5.4.5-1 shows the scenario where the HSS receives a Subscription to notification request for UE-reachability from the IMS-AS  for a subscriber who has a 5GC subscription.

Figure 5.4.5-1: Subscription to UE reachability for 5G subscriber

1.	The HSS receives a Subscribe request for UE reachability for IP from the IMS-AS.

NOTE:	An SBA capable IMS-AS makes use of Nhss_ImsSDM service to interact with the HSS for this operation as defined in 3GPP TS 23.228 [7]. Otherwise the IMS-AS makes use of Diameter Sh-SNR/SNA command.

2.	The HSS reads subscription data from the EPS-UDR.

3.	The HSS sets the UE Reachability flags for EPC and if the UE is registered in EPC contacts the registered MME and SGSN to get notified when the UE becomes reachable.

4.	If the HSS detects in step 2 that the user has a 5GC subscription, the HSS uses the Nudm_EventExposure_Subscribe service operation (one time immediate report requested) to get notified when the UDM detects UE reachability for SMS over IP. Otherwise, continue with step 10.

5.	The UDM sets the URRP-AMF flag in the 5GS-UDR and reads data from the 5GS-UDR to get the AMF for 3GPP Access Registration Information and the AMF for non-3GPP Access Registration Information if any.

6-7.	[Conditional] If an AMF is registered in UDM for the target UE and the UDM has not already subscribed in AMF due to a previous subscription from a different NF, the UDM subscribes to UE reachability notifications at the registered AMFs by means of the Namf_EventExposure_Subscribe service operation (see 3GPP TS 23.502 [5]).

8.	The UDM updates the 5GS-UDR with the EE-Subscription for the HSS.

9.	The UDM acknowledges the EE Subscription to the HSS.

10.	The HSS acknowledges the Sh subscription to the IMS-AS.

11.	The HSS updates the EPS-UDR with the Sh subscription for the IMS-AS.

Figure 5.4.5-2 shows the scenario where the UDM detects UE reachability for SMS over IP and notifies the HSS that has previously subscribed.

Figure 5.4.5-2: UE reachability notification for 5G subscriber

1.	The UDM receives a Notification or Registration from the AMF.

2.	The UDM reads subscription data from the 5GS-UDR.

3.	The UDM acknowledges step 1 towards the AMF. If an old AMF is registered in the UDM, the UDM sends a Nudm_UECM_DeregistrationNotification service operation to the old AMF.

4.	The UDM notifies the HSS (and any other NF that has subscribed) about the reachability of the UE.

5.	The HSS reads data from the EPS-UDR to see whether an IMS-AS has subscribed do reachability notification.

6.	The HSS acknowledges step 4.

7.	The UDM updates the 5GS-UDR to delete the EE-Subscription(s).

8.	The HSS notifies the IMS-AS about UE reachability for IP.

NOTE:	An SBA capable IMS-AS receives the notification from HSS using the Nhss_ImsSDM service as defined in 3GPP TS 23.228 [7]. Otherwise the IMS-AS receives the notification via a Diameter Sh-PNR/PNA command.

9.	The IMS-AS acknowledges step 8.

10.	The HSS updates the EPS-UDR to delete the IMS-AS's subscription.

* * * End Of Change * * * *