3GPP TSG-CT WG4 Meeting #105-e	C4-214xyz

# E-Meeting, 17th – 27th August 2021	(was C4-214367)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.505 | CR | 0367 | rev | 1 | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Retrieval of odbData as ProvisionedDataSet |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | SBIProtoc17 |  |  |  |  |  | Date: |  |  | 2021-08-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | It is not possible to retrieve odb data together with other datasets from UDR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | odbData added to ProvisionedDataSets container and DataSetName. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UDM cannot use a single operator to UDR to retrieve e.g. am-data and odb data required to apply access retrictions at UE registration. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.2.8, 5.4.3.4, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backwards-compatible new features with impacts on the following APIs: - TS29504_Nudr_DR.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 5.4.2.8	Type: ProvisionedDataSets

# Table 5.4.2.8-1: ProvisionedDataSets

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| amData | AccessAndMobilitySubscriptionData | O | 0..1 | Access and Mobility Subscription Data |
| smfSelData | SmfSelectionSubscriptionData | O | 0..1 | SMF Selection Subscription Data |
| smsSubsData | SmsSubscriptionData | O | 0..1 | SMS Subscription Data |
| smData | array(SessionManagementSubscriptionData) | O | 1..N | Session Management Subscription Data |
| traceData | TraceData | O | 0..1 | Trace Data. The Null value indicates that trace is not active. |
| smsMngData | SmsManagementSubscriptionData | O | 0..1 | SMS Management Subscription Data |
| lcsPrivacyData | LcsPrivacyData | O | 0..1 | LCS Privacy Subscription Data |
| lcsMoData | LcsMoData | O | 0..1 | LCS Mobile Originated Subscription Data |
| lcsBcaData | LcsBroadcastAssistanceTypesData | O | 0..1 | LCS Broadcast Assistance Subscription Data |
| v2xData | V2xSubscriptionData | O | 0..1 | V2X Subscription Data |
| proseData | ProseSubscriptionData | O | 0..1 | ProSe service Subscription Data |
| odbData | OdbData | O | 0..1 | Operator Determined Barring Subscription Data |

* * * Next Change * * * *

#### 5.4.3.4	Enumeration: DataSetName

# Table 5.4.3.4-1: Enumeration DataSetName

| Enumeration value | Description |
| --- | --- |
| "AM" | Access and Mobility Subscription Data |
| "SMF_SEL" | SMF Selection Subscription Data |
| "SMS_SUB" | SMS Subscription Data |
| "SM" | Session Management Subscription Data |
| "TRACE" | Trace Data |
| "SMS_MNG" | SMS Management Subscription Data |
| "LCS_PRIVACY" | LCS Privacy Subscription Data |
| "LCS_MO" | LCS Mobile Originated Subscription Data |
| "LCS_BCA" | LCS Broadcast Assistance Subscription Data |
| "V2X" | V2X Subscription Data |
| "PROSE" | ProSe service Subscription Data |
| "ODB" | Operator Determined Barring Subscription Data |

* * * Next Change * * * *

## A.2	Nudm_ DataRepository API for Subscription Data

openapi: 3.0.0

(... text not shown for clarity ...)

DataSetName:

description: The name of data set.

anyOf:

- type: string

enum:

- AM

- SMF_SEL

- SMS_SUB

- SM

- TRACE

- SMS_MNG

- LCS_PRIVACY

- LCS_MO

- LCS_BCA

- V2X

- PROSE

- ODB

- type: string

ProvisionedDataSets:

description: Contains the provisioned data sets.

type: object

properties:

amData:

$ref: '#/components/schemas/AccessAndMobilitySubscriptionData'

smfSelData:

$ref: '#/components/schemas/SmfSelectionSubscriptionData'

smsSubsData:

$ref: '#/components/schemas/SmsSubscriptionData'

smData:

type: array

items:

$ref: '#/components/schemas/SessionManagementSubscriptionData'

traceData:

# $ref: 'TS29571_CommonData.yaml#/components/schemas/TraceData'

smsMngData:

# $ref: '#/components/schemas/SmsManagementSubscriptionData'

lcsPrivacyData:

# $ref: '#/components/schemas/LcsPrivacyData'

lcsMoData:

# $ref: '#/components/schemas/LcsMoData'

lcsBcaData:

# $ref: '#/components/schemas/LcsBroadcastAssistanceTypesData'

v2xData:

# $ref: '#/components/schemas/V2xSubscriptionData'

proseData:

# $ref: '#/components/schemas/ProseSubscriptionData'

# odbData:

$ref: 'TS29571_CommonData.yaml#/components/schemas/OdbData'

(... text not shown for clarity ...)

* * * End of Changes * * * *