3GPP TSG-CT WG4 Meeting #105-e	C4-214xyz

# E-Meeting, 17th – 27th August 2021	(was C4-214371)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.505 | CR | 0369 | rev | 1 | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Authorization of Group Identifier Translation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | SBIProtoc17 |  |  |  |  |  | Date: |  |  | 2021-08-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Currently the Nudr_DR API does not support the possibility to indicate to the UDM which AFs are authorized to perform group identifier translation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the list of allowed AFs to be optionally included as part of the Nudr response message when GroupIdentifier resource is requested. This information will be used by UDM to authorize the Group Id translation query. Besides this change, some specific ProblemDetails have been included to specify the cases where the group does not exist, or the translated group identifier is not found in UDR. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Authorization of Group Identifier translation is not supported on the Nudr interface. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.33.3.1, 5.4.1, 5.4.2.x (new), A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backwards-compatible new features with impacts on the following APIs: - TS29504_Nudr_DR.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 5.2.33.3.1	GET

This method shall support the URI query parameters specified in table 5.2.33.3.1-1.

# Table 5.2.33.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [8] clause 6.6 |
| ext-groud-id | ExtGroupId | C | 0..1 | External Group ID |
| int-group-id | GroupId | C | 0..1 | Internal Group ID |
| ue-id-ind | boolean | C | 0..1 | Indication whether UE identifiers are required or not. When present, it shall be set as following: - true: UE identifiers are required - false (default): UE identifiers are not required |
| NOTE: Either ext-group-id or int-group-id shall be present in the request. |  |  |  |  |

Either the ext-group-id or the int-group-id shall be present in the request.

This method shall support the request data structures specified in table 5.2.33.3.1-2 and the response data structures and response codes specified in table 5.2.33.3.1-3.

# Table 5.2.33.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

# Table 5.2.33.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| GroupIdentifiers | M | 1 | 200 OK | Upon success, a response body containing the group identifier(s) shall be returned. |
| ProblemDetails | O | 0..1 | 404 Not Found | If the ueId does not exist, a response code of 404 Not Found shall be returned. The "cause" attribute may be used to indicate one of the following application errors: - GROUP_IDENTIFIER_NOT_FOUND - DATA_NOT_FOUND |
| NOTE: In addition, common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

* * * Next Change * * * *

### 5.4.1	General

This clause specifies the application data model supported by the API.

Table 5.4.1-1 specifies the data types defined for the Nudr service based interface protocol.

# Table 5.4.1-1: Nudr Subscriber Data specific Data Types

| Data type | Clause defined | Description |
| --- | --- | --- |
| AuthenticationSubscription | 5.4.2.2 | A UE's authentication data |
| OperatorSpecificDataContainer | 5.4.2.3 | Container for operator specific data |
| SmfRegList | 5.4.2.4 | The list of all the SMF registrations of a UE |
| SubscriptionDataSubscriptions | 5.4.2.5 | A subscription to notifications. |
| DataChangeNotify | 5.4.2.6 | Container for data which have changed and notification was requested when changed. |
| IdentityData | 5.4.2.7 | Identity data corresponds to the provided ueId |
| ProvisionedDataSets | 5.4.2.8 | Contains the provisioned data sets |
| SorData | 5.4.2.9 | Used to store the status of the latest SOR data update |
| UpuData | 5.4.2.9A | Used to store the status of the latest UPU data update |
| NssaiAckData | 5.4.2.9B | Used to store the status of the latest NSSAI data update |
| CagAckData | 5.4.2.9C | Used to store the status of the latest CAG data update |
| AmfSubscriptionInfo | 5.4.2.19 | Information the UDR stores and retrieves related to active subscriptions at the AMF(s) |
| EeProfileData | 5.4.2.20 | Event Exposure Profile Data |
| ContextDataSets | 5.4.2.22 | Contains the context data sets |
| SequenceNumber | 5.4.2.23 | Contains the SQN |
| MessageWaitingData | 5.4.2.24 | Message Waiting Data list |
| SmscData | 5.4.2.25 | Addresses of SM-Service Center entities with SMS wating to be delivered to the UE |
| SmfSubscriptionInfo | 5.4.2.26 | Information the UDR stores and retrieves related to active subscriptions at the SMF(s) |
| SmfSubscriptionItem | 5.4.2.27 | Contains info about a single SMF event subscription |
| MtcProvider | 5.4.2.28 | MTC provider information |
| HssSubscriptionInfo | 5.4.2.29 | Information the UDR stores related to active subscriptions at the HSS(s) |
| HssSubscriptionItem | 5.4.2.30 | Contains info about a single HSS event subscription |
| EeGroupProfileData | 5.4.2.31 | The Event Exposure Profile Data for a group of UEs |
| Pp5gVnGroupProfileData | 5.4.2.32 | The Paramter Provision Profile Data for 5G VN groups |
| PpProfileData | 5.4.2.33 | The Paramter Provision Profile Data |
| AllowedMtcProviderInfo | 5.4.2.34 | Allowed MTC Providers or AFs Information to provision parameters for UE. |
| GroupIdentifiers | 5.4.2.x | External or Internal Group Identifier with a list of group members |
| AuthMethod | 5.4.3.3 | Contains the Authentication Method |
| DataSetName | 5.4.3.4 | The name of data set |
| ContextDataSetName | 5.4.3.6 | The name of context data set |
| SqnScheme | 5.4.3.7 | Scheme for generation of Sequence Numbers |
| Sign | 5.4.3.8 | Sign of the DIF value |
| UeUpdateStatus | 5.4.3.9 | Status of the procedure |
| PpDataType | 5.4.3.10 |  |

Table 5.4.1-2 specifies data types re-used by the Nudr service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nudr service based interface.

# Table 5.4.1-2: Nudr re-used Data Types

| Data type |  | Reference |  | Comments |  |
| --- | --- | --- | --- | --- | --- |
| AccessAndMobilitySubscriptionData |  | 3GPP TS 29.503 [6] |  | Access and Mobility Subscription Data |  |
| SmfSelectionSubscriptionData |  | 3GPP TS 29.503 [6] |  | SMF Selection Subscription Data |  |
| SessionManagementSubscriptionData |  | 3GPP TS 29.503 [6] |  | Session Management Subscription Data |  |
| Amf3GppAccessRegistration |  | 3GPP TS 29.503 [6] |  |  |  |
| AmfNon3GppAccessRegistration |  | 3GPP TS 29.503 [6] |  | The complete set of information relevant to the AMF where the UE has registered via non 3GPP access. |  |
| SmfRegistration |  | 3GPP TS 29.503 [6] |  |  |  |
| SmsfRegistration |  | 3GPP TS 29.503 [6] |  |  |  |
| SmsManagementSubscriptionData |  | 3GPP TS 29.503 [6] |  |  |  |
| SupportedFeatures |  | 3GPP TS 29.571 [3] |  | Used to negotiate the applicability of the optional features |  |
| ProblemDetails |  | 3GPP TS 29.571 [3] |  | Detailed information about the status code. |  |
| PpData |  | 3GPP TS 29.503 [6] |  |  |  |
| PpDataEntry |  | 3GPP TS 29.503 [6] |  | Provisioned Parameter Data Entry |  |
| SmsSubscriptionData |  | 3GPP TS 29.503 [6] |  |  |  |
| PatchItem |  | 3GPP TS 29.571 [3] |  | Data structure used for JSON patch. |  |
| SdmSubscription |  | 3GPP TS 29.503 [6] |  |  |  |
| EeSubscription |  | 3GPP TS 29.503 [6] |  |  |  |
| Uri |  | 3GPP TS 29.571 [3] |  |  |  |
| TraceData |  | 3GPP TS 29.571 [3] |  |  |  |
| Dnn |  | 3GPP TS 29.571 [3] |  | Data Network Name |  |
| Snssai |  | 3GPP TS 29.571 [3] |  | Single NSSAI |  |
| VarUeId |  | 3GPP TS 29.571 [3] |  | String represents the SUPI or GPSI. |  |
| NfInstanceId |  | 3GPP TS 29.571 [3] |  |  |  |
| NotifyItem |  | 3GPP TS 29.571 [3] |  | Common data type used for data change notification. |  |
| OdbData |  | 3GPP TS 29.571 [3] |  | Operator Determined Barring Data |  |
| EventType |  | 3GPP TS 29.503 [6] |  |  |  |
| ExtGroupId |  | 3GPP TS 29.503 [6] |  | External Group Identifier |  |
| PduSessionId |  | 3GPP TS 29.571 [3] |  |  |  |
| DateTime |  | 3GPP TS 29.571 [3] |  |  |  |
| UpuMac |  | 3GPP TS 29.509 [15] |  |  |  |
| 5GVnGroupConfiguration |  | 3GPP TS 29.503 [6] |  |  |  |
| PatchResult |  | 3GPP TS 29.571 [3] |  |  |  |
| SupportedFeatures |  | 3GPP TS 29.571 [3] |  |  |  |
| AppPortId |  | 3GPP TS 29.503 [6] |  |  |  |
| LcsPrivacyData |  | 3GPP TS 29.503 [6] |  | LCS Privacy Subscription Data |  |
| LcsMoData |  | 3GPP TS 29.503 [6] |  | LCS Mobile Originated Subscription Data |  |
| MtcProviderInformation |  | 3GPP TS 29.571 [3] |  |  |  |
| AuthorizationData |  | 3GPP TS 29.503 [6] |  |  |  |
| EnhancedCoverageRestrictionData |  | 3GPP TS 29.503 [6] |  |  |  |
| LocationInfo |  | 3GPP TS 29.503 [6] |  |  |  |
| V2xSubscriptionData |  | 3GPP TS 29.503 [6] |  | V2X Subscription Data |  |
| E164Number |  | 3GPP TS 29.503 [6] |  |  |  |
| NetworkNodeDiameterAddress |  | 3GPP TS 29.503 [6] |  |  |  |
| IpSmGwRegistration |  | 3GPP TS 29.503 [6] |  |  |  |
| LcsBroadcastAssistanceTypesData |  | 3GPP TS 29.503 [6] |  | LCS Broadcast Assistance Data Types |  |
| ContextInfo |  | 3GPP TS 29.503 [6] |  |  |  |
| NfGroupId |  | 3GPP TS 29.571 [3] |  |  |  |
| ProseSubscriptionData |  | 3GPP TS 29.503 [6] |  | ProSe Service Subscription Data |  |
| PlmnId |  | 3GPP TS 29.571 [3] |  | PLMN ID |  |

* * * Next Change * * * *

##### 5.4.2.X	Type: GroupIdentifiers

Table 5.4.2.X-1: Definition of type GroupIdentifiers

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| extGroupId | ExtGroupId | C | 0..1 | This IE shall contain the External Group ID associated to the provided Internal Group ID. |
| intGroupId | GroupId | C | 0..1 | This IE shall contain the Internal Group ID associated to the provided External Group ID.. |
| ueIdList | array(UeId) | C | 1..N | This IE shall contain a list of the UE identifiers that belong to the provided Internal/External Group ID if they are required. |
| allowedAfIds | array(string) | O | 1..N | A list of Application Function Identifiers authorized to retrieve this Identities lists. The absence of this IE indicates that any AF is allowed to retrieve them. |

* * * Next Change * * * *

## A.2	Nudr_DataRepository API for Subscription Data

(... text not shown for clarity ...)

/subscription-data/group-data/group-identifiers:

get:

summary: Mapping of Group Identifiers

operationId: GetGroupIdentifiers

tags:

- Group Identifiers

security:

- {}

- oAuth2ClientCredentials:

- nudr-dr

- oAuth2ClientCredentials:

- nudr-dr

- nudr-dr:subscription-data

parameters:

- name: ext-group-id

in: query

description: External Group Identifier

required: false

schema:

$ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/ExtGroupId'

- name: int-group-id

in: query

description: Internal Group Identifier

required: false

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/GroupId'

- name: ue-id-ind

in: query

description: Indication whether UE identifiers are required or not

required: false

schema:

type: boolean

default: false

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/GroupIdentifiers'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

(... text not shown for clarity ...)

PpDataType:

anyOf:

- type: string

enum:

- COMMUNICATION_CHARACTERISTICS

- EXPECTED_UE_BEHAVIOUR

- EC_RESTRICTION

- ACS_INFO

- TRACE

- STN_SR

- LCS_PRIVACY

- SOR_INFO

- type: string

GroupIdentifiers:

description: External or Internal Group Identifier with a list of group members

type: object

properties:

extGroupId:

$ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/ExtGroupId'

intGroupId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/GroupId'

ueIdList:

type: array

items:

$ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/UeId'

minItems: 1

allowedAfIds:

type: array

items:

type: string

minItems: 1

* * * End of Changes * * * *