3GPP TSG-CT WG4 Meeting #107-e	C4-216035v2

# E-Meeting, 15th – 23rd November 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.168 | CR | 0077 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of Test Flag |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | one2many, KPN |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2021-11-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Any test aimed at checking that resources needed for the delivery of a PWS message are correctly operating (e.g. correct operation of nodes, interfaces, cells, availability of resources over the air) can only be done end to end, i.e. involving broadcasting over the air and decoding at the UE of test messages with specific Message Identifiers. The addition of a Test Flag has been specified in stage 2 (published as TS 23.041 V17.2.0). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of Test Flag in the message request which is processed as a normal message by the RAN nodes, but doesn't result in broadcast over the air and doesn't allocate any resources. The Test Flag is only applicable if the Write-Replace-Warning Request is sent via a PWS-IWF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | When using the existing test techniques involving the air interface and UE, are: • waste of radio resources due to potentially frequent broadcast over the air • impact on UE battery consumption due to UEs having to decode updated SIBs. An inconsistency will occur between stage 2 and stage 3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.4.2.1, 4.3.4.3.x (new), 4.4.4, 4.4.5, 4.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | The assigned criticality for the Test Flag was changed from "ignore" to "reject". When the PWS-IWF does not recognize the Test Flag it shall reject the message because a message wihout Test Flag shal not result in any broadcast. |  |  |  |  |  |  |  |  |

*******

* * * First Change * * * *

##### 4.3.4.2.1	WRITE-REPLACE WARNING REQUEST

This message is sent by the CBC to request start or overwrite of a warning message broadcast.

If the message is sent to the MME, then:

-	the List of TAIs IE, the Warning Area List IE and the Global eNB ID IE may be used; and

-	the List of 5GS TAIs IE, the Warning Area List 5GS IE, the Global RAN Node ID IE and the RAT Selector 5GS IE shall not be used.

If the message is sent to the PWS-IWF, then:

-	the List of 5GS TAIs IE, the Warning Area List 5GS IE, the Global RAN Node ID IE may be used;

-	the List of TAIs IE, the Warning Area List IE and the Global eNB ID IE shall not be used; and-	the message shall address a Warning Area in E-UTRA or in NR, but not both simultaneously; the RAT Selector 5GS IE shall be used if the message is for the NR RAT.

Direction: CBC  MME, PWS-IWF

# Table 4.3.4.2.1-1: WRITE-REPLACE WARNING REQUEST message contents

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 4.3.4.3.1 |  | YES | reject |
| Message Identifier | M |  | [7] |  | YES | reject |
| Serial Number | M |  | [7] |  | YES | reject |
| List of TAIs | O |  |  |  | YES | reject |
| >TAI List Item |  | 1 to <maxnoofTAI> |  |  |  |  |
| >>TAI | M |  | [7] |  |  |  |
| Warning Area List | O |  | [7] |  | YES | ignore |
| Repetition Period | M |  | [7] |  | YES | reject |
| Extended Repetition Period | O |  | [7] |  | YES | reject |
| Number of Broadcast Requested | M |  | [7] |  | YES | reject |
| Warning Type | O |  | [7] |  | YES | ignore |
| Warning Security Information | O |  | [7] |  | YES | ignore |
| Data Coding Scheme | O |  | [7] |  | YES | ignore |
| Warning Message Contents | O |  | [7] |  | YES | ignore |
| OMC ID | O |  | 4.3.4.3.4 |  | YES | ignore |
| Concurrent Warning Message Indicator | O |  | [7] |  | YES | reject |
| Send Write-Replace-Warning-Indication | O |  | 4.3.4.3.5 |  | YES | ignore |
| Global eNB ID | O |  | [7] |  | YES | ignore |
| List of 5GS TAIs | O |  |  |  | YES | ignore |
| >5GS TAI List Item |  | 1 to <maxnoof5GSTAIs> |  |  |  |  |
| >>5GS TAI | M |  | [17] |  |  |  |
| Warning Area List 5GS | O |  | [17] |  | YES | ignore |
| Global RAN Node ID | O |  | [17] |  | YES | Ignore |
| RAT Selector 5GS | C |  | 4.3.4.3.10 |  | YES | ignore |
| Warning Area Coordinates | O |  | [7] |  | YES | Ignore |
| Test Flag 5GS | O |  | 4.3.4.3.x |  | YES | reject |

# Table 4.3.4.2.1-2: RANGE explanation

| Range bound | Explanation |
| --- | --- |
| maxnoofTAI | Maximum no. of TAI subject for warning message broadcast in E-UTRAN. Value is 65535. |
| Maxnoof5GSTAIs | Maximum no. of TAI subject for warning message broadcast in 5GS. |

Editor's Note: the Maxnoof5GSTAIs value is FFS in RAN3 (assumption for ASN1 is 65535).

* * * Next Change * * * *

##### 4.3.4.3.x	Test Flag 5GS

The Test Flag 5GS IE may be included to indicate to the NG-RAN node that the request is to be processed and responded to normally, but shall not result in broadcast over the air and shall not allocate any resources (i.e. the request cannot be cancelled).

This IE may only be included in the WRITE-REPLACE WARNING-REQUEST if the CBC sends the request to a PWS-IWF.

# Table 4.3.4.3.x-1: Test Flag 5GS information element

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Test Flag 5GS | O |  | ENUMERATED(true) |  |

* * * Next Change * * * *

### 4.4.4	PDU definitions

-- **************************************************************

--

-- PDU definitions for SBC-AP.

--

-- **************************************************************

SBC-AP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) sbc-AP (3) version1 (1) sbc-AP-PDU-Contents (1)}

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Cause,

Concurrent-Warning-Message-Indicator,

Criticality-Diagnostics,

Data-Coding-Scheme,

Message-Identifier,

Serial-Number,

List-of-TAIs,

Warning-Area-List,

Omc-Id,

Repetition-Period,

Extended-Repetition-Period,

Number-of-Broadcasts-Requested,

Warning-Type,

Warning-Security-Information,

Warning-Message-Content,

Warning-Area-Coordinates,

Send-Write-Replace-Warning-Indication,

Broadcast-Scheduled-Area-List,

Unknown-Tracking-Area-List,

Send-Stop-Warning-Indication,

Broadcast-Cancelled-Area-List,

Stop-All-Indicator,

Broadcast-Empty-Area-List,

Restarted-Cell-List,

Global-ENB-ID,

List-of-TAIs-Restart,

List-of-EAIs-Restart,

Failed-Cell-List,

List-of-5GS-TAIs,

Warning-Area-List-5GS,

Global-RAN-Node-ID,

Global-GNB-ID,

RAT-Selector-5GS,

Unknown-5GS-Tracking-Area-List,

Broadcast-Scheduled-Area-List-5GS,

Broadcast-Cancelled-Area-List-5GS,

Broadcast-Empty-Area-List-5GS,

Restarted-Cell-List-NR,

Failed-Cell-List-NR,

List-of-5GS-TAI-for-Restart,

Test-Flag-5GS

FROM SBC-AP-IEs

ProtocolExtensionContainer{},

ProtocolIE-Container{},

SBC-AP-PROTOCOL-EXTENSION,

SBC-AP-PROTOCOL-IES

FROM SBC-AP-Containers

id-Concurrent-Warning-Message-Indicator,

id-Criticality-Diagnostics,

id-Cause,

id-Data-Coding-Scheme,

id-List-of-TAIs,

id-Message-Identifier,

id-Serial-Number,

id-Number-of-Broadcasts-Requested,

id-Omc-Id,

id-Radio-Resource-Loading-List,

id-Recovery-Indication,

id-Repetition-Period,

id-Extended-Repetition-Period,

id-Warning-Area-List,

id-Warning-Message-Content,

id-Warning-Area-Coordinates,

id-Warning-Security-Information,

id-Warning-Type,

id-Send-Write-Replace-Warning-Indication,

id-Broadcast-Scheduled-Area-List,

id-Unknown-Tracking-Area-List,

id-Send-Stop-Warning-Indication,

id-Broadcast-Cancelled-Area-List,

id-Stop-All-Indicator,

id-Broadcast-Empty-Area-List,

id-Global-ENB-ID,

id-Restarted-Cell-List,

id-List-of-TAIs-Restart,

id-List-of-EAIs-Restart,

id-Failed-Cell-List,

id-List-of-5GS-TAIs,

id-Warning-Area-List-5GS,

id-Global-RAN-Node-ID,

id-Global-GNB-ID,

id-RAT-Selector-5GS,

id-Unknown-5GS-Tracking-Area-List,

id-Broadcast-Scheduled-Area-List-5GS,

id-Broadcast-Cancelled-Area-List-5GS,

id-Broadcast-Empty-Area-List-5GS,

id-Restarted-Cell-List-NR,

id-Failed-Cell-List-NR,

id-List-of-5GS-TAI-for-Restart,

id-Test-Flag-5GS

FROM SBC-AP-Constants;

-- **************************************************************

--

-- Write-Replace-Warning-Request

--

-- **************************************************************

Write-Replace-Warning-Request ::= SEQUENCE {

protocolIEs	ProtocolIE-Container      { {Write-Replace-Warning-Request-IEs} },

protocolExtensions	ProtocolExtensionContainer { {Write-Replace-Warning-Request-Extensions} } OPTIONAL,

...

}

Write-Replace-Warning-Request-IEs SBC-AP-PROTOCOL-IES ::= {

{ ID id-Message-Identifier	CRITICALITY reject	TYPE Message-Identifier	PRESENCE mandatory } |

{ ID id-Serial-Number	CRITICALITY reject	TYPE Serial-Number	PRESENCE mandatory } |

{ ID id-List-of-TAIs	CRITICALITY reject	TYPE List-of-TAIs	PRESENCE optional } |

{ ID id-Warning-Area-List	CRITICALITY ignore	TYPE Warning-Area-List	PRESENCE optional } |

{ ID id-Repetition-Period	CRITICALITY reject	TYPE Repetition-Period	PRESENCE mandatory  } |

{ ID id-Extended-Repetition-Period	CRITICALITY reject	TYPE Extended-Repetition-Period	PRESENCE optional  } |

{ ID id-Number-of-Broadcasts-Requested

CRITICALITY reject	TYPE Number-of-Broadcasts-Requested	PRESENCE mandatory } |

{ ID id-Warning-Type	CRITICALITY ignore	TYPE Warning-Type	PRESENCE optional } |

{ ID id-Warning-Security-Information	CRITICALITY ignore	TYPE Warning-Security-Information	PRESENCE optional } |

{ ID id-Data-Coding-Scheme	CRITICALITY ignore	TYPE Data-Coding-Scheme	PRESENCE optional } |

{ ID id-Warning-Message-Content

CRITICALITY ignore	TYPE Warning-Message-Content	PRESENCE optional } |

{ ID id-Omc-Id  CRITICALITY ignore TYPE Omc-Id PRESENCE optional } |

{ ID id-Concurrent-Warning-Message-Indicator  CRITICALITY reject TYPE Concurrent-Warning-Message-Indicator PRESENCE optional }	|

{ ID id-Send-Write-Replace-Warning-Indication	CRITICALITY ignore	TYPE Send-Write-Replace-Warning-Indication	PRESENCE optional }	|

{ ID id-Global-ENB-ID	CRITICALITY ignore	TYPE Global-ENB-ID	PRESENCE optional },

... ,

{ID id-Warning-Area-Coordinates	CRITICALITY ignore TYPE Warning-Area-Coordinates PRESENCE optional}

}

Write-Replace-Warning-Request-Extensions SBC-AP-PROTOCOL-EXTENSION ::= {

{ ID id-List-of-5GS-TAIs	CRITICALITY ignore	EXTENSION List-of-5GS-TAIs	PRESENCE optional }|

{ ID id-Warning-Area-List-5GS	CRITICALITY ignore	EXTENSION Warning-Area-List-5GS	PRESENCE optional }|

{ ID id-Global-RAN-Node-ID	CRITICALITY ignore	EXTENSION Global-RAN-Node-ID	PRESENCE optional }|

{ ID id-RAT-Selector-5GS	CRITICALITY ignore	EXTENSION RAT-Selector-5GS	PRESENCE optional }|

{ ID id-Test-Flag-5GS		CRITICALITY reject	EXTENSION Test-Flag-5GS		PRESENCE optional },

...

}

-- **************************************************************

--

-- Write-Replace-Warning-Response

--

-- **************************************************************

Write-Replace-Warning-Response ::= SEQUENCE {

protocolIEs	ProtocolIE-Container       { {Write-Replace-Warning-Response-IEs} },

protocolExtensions	ProtocolExtensionContainer { {Write-Replace-Warning-Response-Extensions} } OPTIONAL,

...

}

Write-Replace-Warning-Response-IEs SBC-AP-PROTOCOL-IES ::= {

{ ID id-Message-Identifier	CRITICALITY reject	TYPE Message-Identifier	PRESENCE mandatory } |

{ ID id-Serial-Number	CRITICALITY reject	TYPE Serial-Number	PRESENCE mandatory } |

{ ID id-Cause	CRITICALITY reject	TYPE Cause	PRESENCE mandatory }	|

{ ID id-Criticality-Diagnostics	CRITICALITY ignore	TYPE Criticality-Diagnostics	PRESENCE optional } |

{ ID id-Unknown-Tracking-Area-List CRITICALITY ignore	TYPE List-of-TAIs	PRESENCE optional },

...

}

Write-Replace-Warning-Response-Extensions SBC-AP-PROTOCOL-EXTENSION ::= {

{ ID id-Unknown-5GS-Tracking-Area-List	CRITICALITY ignore	EXTENSION Unknown-5GS-Tracking-Area-List	PRESENCE optional },

...

}

== Part of code left out for brevity ===

# ErrorIndicationIEs SBC-AP-PROTOCOL-IES ::= {

{ ID id-Cause	CRITICALITY ignore	TYPE Cause	PRESENCE optional	} |

{ ID id-Criticality-Diagnostics	CRITICALITY ignore	TYPE Criticality-Diagnostics	PRESENCE optional	} ,

...

}

END

### 4.4.5	Information element definitions

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

== Part of code left out for brevity ===

-- T

TAC ::= OCTET STRING (SIZE (2))

TAC-5GS ::= OCTET STRING (SIZE (3))

TAI-Broadcast-List ::= SEQUENCE (SIZE(1..maxnoofTAIforWarning)) OF TAI-Broadcast-List-Item

TAI-Broadcast-List-Item ::= SEQUENCE {

tAI	TAI,

scheduledCellinTAI	ScheduledCellinTAI,

iE-Extensions	ProtocolExtensionContainer { {TAI-Broadcast-List-Item-ExtIEs} } OPTIONAL,

...

}

# TAI-Broadcast-List-Item-ExtIEs SBC-AP-PROTOCOL-EXTENSION ::= {

...

}

TAI-Broadcast-List-5GS ::= SEQUENCE (SIZE(1..maxnoof5GSTAIs)) OF

SEQUENCE {

tAI-5GS	TAI-5GS,

scheduledCellinTAI-5GS	ScheduledCellinTAI-5GS,

iE-Extensions	ProtocolExtensionContainer { {TAI-Broadcast-List-5GS-ExtIEs} } OPTIONAL,

...

}

# TAI-Broadcast-List-5GS-ExtIEs SBC-AP-PROTOCOL-EXTENSION ::= {

...

}

TAI-Cancelled-List ::= SEQUENCE (SIZE(1..maxnoofTAIforWarning)) OF TAI-Cancelled-List-Item

TAI-Cancelled-List-Item ::= SEQUENCE {

tAI	TAI,

cancelledCellinTAI	CancelledCellinTAI,

iE-Extensions	ProtocolExtensionContainer { {TAI-Cancelled-List-Item-ExtIEs} } OPTIONAL,

...

}

# TAI-Cancelled-List-Item-ExtIEs SBC-AP-PROTOCOL-EXTENSION ::= {

...

}

TAI-Cancelled-List-5GS ::= SEQUENCE (SIZE(1..maxnoof5GSTAIs)) OF

SEQUENCE {

tAI-5GS	TAI-5GS,

cancelledCellinTAI-5GS	CancelledCellinTAI-5GS,

iE-Extensions	ProtocolExtensionContainer { {TAI-Cancelled-List-5GS-ExtIEs} } OPTIONAL,

...

}

# TAI-Cancelled-List-5GS-ExtIEs SBC-AP-PROTOCOL-EXTENSION ::= {

...

}

TAI-List-for-Warning	::= SEQUENCE (SIZE(1.. maxnoofTAIforWarning)) OF TAI

# TAI ::= SEQUENCE {

pLMNidentity	PLMNidentity,

tAC	TAC,

iE-Extensions	ProtocolExtensionContainer { {TAI-ExtIEs} } OPTIONAL

}

# TAI-ExtIEs SBC-AP-PROTOCOL-EXTENSION ::= {

...

}

TBCD-STRING ::= OCTET STRING (SIZE (3))

TypeOfError ::= ENUMERATED {

not-understood,

missing,

...

}

# TAI-5GS ::= SEQUENCE {

pLMNidentity	PLMNidentity,

tAC-5GS	TAC-5GS,

iE-Extensions	ProtocolExtensionContainer { {TAI-5GS-ExtIEs} } OPTIONAL

}

# TAI-5GS-ExtIEs SBC-AP-PROTOCOL-EXTENSION ::= {

...

}

Test-Flag-5GS ::= ENUMERATED {true}

-- U

Unknown-Tracking-Area-List	::= SEQUENCE (SIZE(1.. maxNrOfTAIs)) OF TAI

Unknown-5GS-Tracking-Area-List	::= SEQUENCE (SIZE(1.. maxnoof5GSTAIs)) OF TAI-5GS

-- V

-- W

Warning-Area-List	::= CHOICE {

cell-ID-List	ECGIList,

tracking-Area-List-for-Warning	TAI-List-for-Warning,

emergency-Area-ID-List	Emergency-Area-ID-List,

...

}

Warning-Message-Content	::= OCTET STRING (SIZE (1..9600))

Warning-Area-Coordinates	::= OCTET STRING (SIZE (1..1024))

Warning-Security-Information	::= OCTET STRING (SIZE (50))

Warning-Type	::=  OCTET STRING (SIZE (2))

Warning-Area-List-5GS	::= CHOICE {

cell-ID-List	ECGIList,

nR-CGIList	NR-CGIList,

tAIList-5GS	TAI-5GS,

emergencyAreaIDList	Emergency-Area-ID-List,

...

}

-- X

-- Y

END

* * * Next Change * * * *

### 4.4.7	Constant definitions

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

SBC-AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) sbc-AP (3) version1 (1) sbc-AP-Constants (4)}

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- Elementary Procedures

--

-- **************************************************************

id-Write-Replace-Warning 	INTEGER ::= 0

id-Stop-Warning 	INTEGER ::= 1

id-Error-Indication 	INTEGER ::= 2

id-Write-Replace-Warning-Indication	INTEGER ::= 3

id-Stop-Warning-Indication	INTEGER ::= 4

id-PWS-Restart-Indication	INTEGER ::= 5

id-PWS-Failure-Indication	INTEGER ::= 6

-- **************************************************************

--

-- IEs

--

-- **************************************************************

id-Broadcast-Message-Content	INTEGER ::= 0

id-Cause 	INTEGER ::= 1

id-Criticality-Diagnostics	INTEGER ::=2

id-Data-Coding-Scheme 	INTEGER ::= 3

id-Failure-List 	INTEGER ::= 4

id-Message-Identifier 	INTEGER ::= 5

id-Number-of-Broadcasts-Completed-List 	INTEGER ::= 6

id-Number-of-Broadcasts-Requested 	INTEGER ::= 7

id-Radio-Resource-Loading-List 	INTEGER ::= 8

id-Recovery-Indication 	INTEGER ::= 9

id-Repetition-Period 	INTEGER ::= 10

id-Serial-Number 	INTEGER ::= 11

id-Service-Areas-List 	INTEGER ::= 12

id-TypeOfError	INTEGER ::= 13

id-List-of-TAIs	INTEGER ::= 14

id-Warning-Area-List 	INTEGER ::= 15

id-Warning-Message-Content	INTEGER ::= 16

id-Warning-Security-Information	INTEGER ::= 17

id-Warning-Type	INTEGER ::= 18

id-Omc-Id 	INTEGER ::= 19

id-Concurrent-Warning-Message-Indicator INTEGER ::= 20

id-Extended-Repetition-Period	INTEGER ::= 21

id-Unknown-Tracking-Area-List	INTEGER ::= 22

id-Broadcast-Scheduled-Area-List	INTEGER ::= 23

id-Send-Write-Replace-Warning-Indication INTEGER ::= 24

id-Broadcast-Cancelled-Area-List	INTEGER ::= 25

id-Send-Stop-Warning-Indication	INTEGER ::= 26

id-Stop-All-Indicator	INTEGER ::= 27

id-Global-ENB-ID	INTEGER ::= 28

id-Broadcast-Empty-Area-List	INTEGER ::= 29

id-Restarted-Cell-List	INTEGER ::= 30

id-List-of-TAIs-Restart	INTEGER ::= 31

id-List-of-EAIs-Restart	INTEGER ::= 32

id-Failed-Cell-List	INTEGER ::= 33

id-List-of-5GS-TAIs	INTEGER ::= 34

id-Warning-Area-List-5GS	INTEGER ::= 35

id-Global-RAN-Node-ID	INTEGER ::= 36

id-Global-GNB-ID	INTEGER ::= 37

id-RAT-Selector-5GS	INTEGER ::= 38

id-Unknown-5GS-Tracking-Area-List	INTEGER ::= 39

id-Broadcast-Scheduled-Area-List-5GS	INTEGER ::= 40

id-Broadcast-Cancelled-Area-List-5GS	INTEGER ::= 41

id-Broadcast-Empty-Area-List-5GS	INTEGER ::= 42

id-Restarted-Cell-List-NR	INTEGER ::= 43

id-Failed-Cell-List-NR	INTEGER ::= 44

id-List-of-5GS-TAI-for-Restart	INTEGER ::= 45

id-Warning-Area-Coordinates	INTEGER ::= 46

id-Test-Flag-5GS INTEGER ::= 47

-- **************************************************************

--

-- Extension constants

--

-- **************************************************************

-- **************************************************************

--

-- Lists

--

-- **************************************************************

maxNrOfErrors	INTEGER ::= 256

maxnoofCellID	INTEGER	::= 65535

maxnoofCellinEAI	INTEGER ::= 65535

maxnoofCellinTAI	INTEGER ::= 65535

maxNrOfTAIs	INTEGER ::= 65535

maxnoofEmergencyAreaID	INTEGER	::= 65535

maxnoofTAIforWarning	INTEGER ::= 65535

maxProtocolExtensions 	INTEGER ::= 65535

maxProtocolIEs	INTEGER ::= 65535

maxnoofeNBIds	INTEGER ::= 256

maxnoofRestartedCells	INTEGER ::= 256

maxnoofRestartTAIs	INTEGER ::= 2048

maxnoofRestartEAIs	INTEGER ::= 256

maxnoofFailedCells	INTEGER ::= 256

maxnoof5GSTAIs	INTEGER ::= 2048

maxnoofCellsingNB	INTEGER ::= 16384

maxnoofCellsin5GS	INTEGER ::= 16776960

maxnoofCellsin5GSTAI	INTEGER ::= 65535

maxnoofRANNodes	INTEGER ::= 65535

maxnoofRestart5GSTAIs	INTEGER ::= 2048maxnoofCellsforRestartNR 	INTEGER ::= 16384

END

* * * End of Changes * * * *