3GPP TSG-CT WG4 Meeting #108-e	C4-221xyz

# E-Meeting, 17th – 25th February 2022	(was C4-221179)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.510 | CR | 0677 | rev | 1 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | P-CSCF Restoration Enhancements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | SBIProtoc17 |  |  |  |  |  | Date: |  |  | 2022-02-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | At CT4#107-bis-e, CT4 agreed on enhancements to the P-CSCF restoration procedures, so the SMF could be notified (by UDM or PCF) about the address(es) of a failed P-CSCF (Mw interface addresses), after S-CSCF has detected it. Then, the SMF may use this information to build an updated list of P-CSCF address (Gm interface addresses) to be sent to the UE over NAS/PCO. The mapping between Mw and Gm address(es) can be locally configured at the SMF, but they could also be provided during P-CSCF discovery done by SMF, if they are available in the NF Profile registered by the P-CSCF in the NRF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the Mw addresses of the P-CSCF instance to the NF Profile of the P-CSCF in NRF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The P-CSCF restoration procedures are not optimal. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 6.1.6.2.53, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backwards-compatible new features with impact on OpenAPI specification files: - TS29510_Nnrf_NFManagement.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[5]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[6]	3GPP TS 29.518: "5G System; Access and Mobility Management Services; Stage 3".

[7]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[8]	ECMA-262: "ECMAScript® Language Specification", https://www.ecma-international.org/ecma-262/5.1/.

[9]	IETF RFC 7540: "Hypertext Transfer Protocol Version 2 (HTTP/2)".

[10]	OpenAPI Initiative, "OpenAPI Specification Version 3.0.0", https://spec.openapis.org/oas/v3.0.0.

[11]	IETF RFC 7807: "Problem Details for HTTP APIs".

[12]	3GPP TS 23.003: "Numbering, Addressing and Identification".

[13]	IETF RFC 6902: "JavaScript Object Notation (JSON) Patch".

[14]	IETF RFC 6901: "JavaScript Object Notation (JSON) Pointer".

[15]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[16]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[17]	IETF RFC 3986: "Uniform Resource Identifier (URI): Generic Syntax".

[18]	IETF RFC 4122: "A Universally Unique IDentifier (UUID) URN Namespace".

[19]	IETF RFC 7232: "Hypertext Transfer Protocol (HTTP/1.1): Conditional Requests".

[20]	IETF RFC 7234: "Hypertext Transfer Protocol (HTTP/1.1): Caching".

[21]	3GPP TS 29.244: "Interface between the Control Plane and the User Plane Nodes; Stage 3".

[22]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

[23]	IETF RFC 2782: "A DNS RR for specifying the location of services (DNS SRV)".

[24]	IETF RFC 7515: "JSON Web Signature (JWS)".

[25]	IETF RFC 7519: "JSON Web Token (JWT)".

[26]	W3C HTML 4.01 Specification, https://www.w3.org/TR/2018/SPSD-html401-20180327/.

[27]	3GPP TS 23.527: "5G System; Restoration Procedures; Stage 2".

[28]	3GPP TS 29.513: "5G System; Policy and Charging Control signalling flows and QoS parameter mapping; Stage 3".

[29]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[30]	IETF RFC 1952: "GZIP file format specification version 4.3".

[31]	3GPP TR 21.900: "Technical Specification Group working methods".

[32]	3GPP TS 29.520: "5G System; Network Data Analytics Services; Stage 3".

[33]	3GPP TS 29.572: "5G System; Location Management Services; Stage 3".

[34]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[35]	3GPP TS 29.517: "Application Function Event Exposure Service".

[36]	3GPP TS 29.503: "Unified Data Management Services".

[37]	3GPP TS 29.336: "Home Subscriber Server (HSS) diameter interfaces for interworking with packet data networks and applications".

[38]	IANA: "SMI Network Management Private Enterprise Codes", http://www.iana.org/assignments/enterprise-numbers.

[39]	Semantic Versioning Specification: https://semver.org.

[40]	IETF RFC 7231: "Hypertext Transfer Protocol (HTTP/1.1): Semantics and Content".

[41]	IETF RFC 7694: "Hypertext Transfer Protocol (HTTP) Client-Initiated Content-Encoding".

[42]	3GPP TS 29.531: "5G System; Network Slice Selection Services; Stage 3".

[43]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services".

[44]	ITU-T Recommendation E.164: "The international public telecommunication numbering plan".

[xx]	3GPP TS 23.380: "IMS Restoration Procedures".

* * * Next Change * * * *

##### 6.1.6.2.53	Type: PcscfInfo

Table 6.1.6.2.53-1: Definition of type PcscfInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| accessType | array(AccessType) | C | 1..N | If included, this IE shall contain the access type (3GPP_ACCESS and/or NON_3GPP_ACCESS) supported by the P-CSCF. If not included, it shall be assumed that all access types are supported. |
| dnnList | array(Dnn) | O | 1..N | DNNs supported by the P-CSCF. The DNN shall contain the Network Identifier and it may additionally contain an Operator Identifier. If the Operator Identifier is not included, the DNN is supported for all the PLMNs in the plmnList of the NF Profile. If not provided, the P-CSCF can serve any DNN. |
| gmFqdn | Fqdn | O | 0..1 | FQDN of the P-CSCF for the Gm interface |
| gmIpv4Addresses | array(Ipv4Addr) | O | 1..N | IPv4 address(es) of the P-CSCF for the Gm interface |
| gmIpv6Addresses | array(Ipv6Addr) | O | 1..N | IPv6 address(es) of the P-CSCF for the Gm interface |
| mwFqdn | Fqdn | O | 0..1 | FQDN of the P-CSCF for the Mw interface (NOTE) |
| mwIpv4Addresses | array(Ipv4Addr) | O | 1..N | IPv4 address(es) of the P-CSCF for the Mw interface (NOTE) |
| mwIpv6Addresses | array(Ipv6Addr) | O | 1..N | IPv6 address(es) of the P-CSCF for the Mw interface (NOTE) |
| servedIpv4AddressRanges | array(Ipv4AddressRange) | O | 1..N | List of ranges of UE IPv4 addresses used on the Gm interface, served by P-CSCF. The absence of this attribute does not mean the P-CSCF can serve any IPv4 address. |
| servedIpv6PrefixRanges | array(Ipv6PrefixRange) | O | 1..N | List of ranges of UE IPv6 prefixes used on the Gm interface, served by P-CSCF. The absence of this attribute does not mean the P-CSCF can serve any IPv6 prefix. |
| NOTE: The Mw addressing information of the P-CSCF may be used by other NFs (e.g., SMF) in P-CSCF restoration scenarios (see 3GPP TS 23.380 [xx], clause 5.8.4.2 and clause 5.8.5.2), where a mapping between Gm and Mw addresses may be used to determine the updated list of P-CSCFs to be sent to the UE, after excluding those P-CSCF instances that have been deemed as failed by the S-CSCF. |  |  |  |  |

* * * Next Change * * * *

## A.2	Nnrf_NFManagement API

(... text not shown for clarity ...)

PcscfInfo:

description: Information of a P-CSCF NF Instance

type: object

properties:

accessType:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AccessType'

minItems: 1

dnnList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

minItems: 1

gmFqdn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Fqdn'

gmIpv4Addresses:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

minItems: 1

gmIpv6Addresses:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Addr'

minItems: 1

mwFqdn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Fqdn'

mwIpv4Addresses:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

minItems: 1

mwIpv6Addresses:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Addr'

minItems: 1

servedIpv4AddressRanges:

type: array

items:

$ref: '#/components/schemas/Ipv4AddressRange'

minItems: 1

servedIpv6PrefixRanges:

type: array

items:

$ref: '#/components/schemas/Ipv6PrefixRange'

minItems: 1

(... text not shown for clarity ...)

* * * End of Changes * * * *