3GPP TSG-CT WG4 Meeting #110-e	C4-223xyz

# E-Meeting, 12th – 20th May 2022	(was C4-223202)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.510 | CR | 0722 | rev | 1 | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Encoding of Query Parameters |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | SBIProtoc17 |  |  |  |  |  | Date: |  |  | 2022-04-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Query parameters defined as an array of simple types (e.g. array of strings) must be formatted using OpenAPI keyword "style" set to "form" and keyword "explode" set to false. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the wrong encoding of query parameters: - ml-analytics-id-list - preferences-precedence (Given that these query parameters were introduced during Rel-17, and therefore the affected API is still in development "alpha" status, such correction is considered as backwards-compatible) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | API specification not conformant to the general guidelines described in TS 29.501. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backwards-compatible corrections with impacts on the following APIs: - TS29510_Nnrf_NFDiscovery.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## A.3	Nnrf_NFDiscovery API

(... text not shown for clarity ...)

- name: ml-analytics-id-list

in: query

description: Analytics Id(s) of Nnwdaf_MLModelProvision service

schema:

type: array

items:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/NwdafEvent'

minItems: 1

style: form

explode: false

(... text not shown for clarity ...)

- name: preferences-precedence

in: query

description: Indicating the precedence of the preference query parameters (from higher to lower).

schema:

type: array

items:

type: string

minItems: 2

style: form

explode: false

(... text not shown for clarity ...)

* * * End of Changes * * * *