3GPP TSG-CT WG4 Meeting #97e	C4-202345

# E-Meeting, 15th – 24th April 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.524 | CR | 0021 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New mapping between Application Error Received from PCF by SMF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Orange |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2020-04-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | CT3 has introduced defined a new Application Error sent by PCF to SMF. This Application Error has to be mapped to a corresponding Cause Code. The CT3 LS (C4-200367) proposes to map this Application Error to Cause #29. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the proposed mapping in clause 5.2.2.2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The mapping is not documented in Rel-16. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 5.2.2	Mapping between Npcf_SMPolicyControl Service causes on N7 and 5GSM causes

#### 5.2.2.1	General

This clause defines the mapping for the Npcf_SMPolicyControl service (see 3GPPTS 29.512 [6]).

#### 5.2.2.2	Mapping from HTTP to 5GSM cause values – Request rejected by PCF

Table 5.2.2.2-1: Mapping from HTTP to 5GSM cause values – Request rejected by PCF

| HTTP status code on N7 | Protocol or Application Error | 5GSM cause to UE |
| --- | --- | --- |
| 400 Bad Request | USER_UNKNOWN | Cause #29 - User authentication or authorization failed |
|  | ERROR_INITIAL_PARAMETERS | Cause #31 - Request rejected, unspecified |
|  | ERROR_TRIGGER_EVENT | Cause #31 - Request rejected, unspecified |
| 403 Forbidden | TRAFFIC_MAPPING_INFO_REJECTED | Cause #29 - User authentication or authorization failed |
|  | ERROR_CONFLICTING_REQUEST | Cause #67 - insufficient resources for specific slice and DNN |
|  | POLICY_CONTEXT_DENIED | Cause #29 - User authentication or authorization failed (NOTE) |
|  | VALIDATION_CONDITION_NOT_MET | Cause #29 - User authentication or authorization failed |
| NOTE: Upon receiving the cause POLICY CONTEXT DENIED, the SMF may reject the request, or it may accept the request based on the local policy. |  |  |

* * * End of Change * * * *