3GPP TSG-CT WG4 Meeting #98e	C4-203348

# E-Meeting, 2nd – 12th June 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.562 | CR | 0018 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Cleanup of TS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | eIMS5G_SBA |  |  |  |  |  | Date: |  |  | 2020-05-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There are some misalignments and small fixes still pending in TS |  |  |  |  |  |  |  |  |
| Summary of change: |  | Most relevant changes are listed below: - Editor’s note is removed to align with SRVCC capability agreed in Sh (see C4-2002356) - Editor’s note about Oauth is removed, since SA3 has already defined to procedures and TS is already aligned with them (see C4-200118) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TS misalignments between some sections |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.2.1, 5.3.2.2.1, 5.3.2.6.2, 5.3.2.7.4, 6.1.3.2.2, 6.1.4.2.2, 6.2.3.1, Annex A.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backwards-compatible corrections with impact on the following APIs: - TS29562_Nhss_imsSDM.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 5.3.2.1	Introduction

For the Nhss_imsSubscriberDataManagement service the following service operations are defined:

-	Get

-	Subscribe

-	Unsubscribe

-	Notification

-	Update

The Nhss_imsSubscriberDataManagement Service is used by Consumer NFs (S-CSCF, IMS-AS) to retrieve the UE's individual subscription data relevant to the consumer NF, or by I-CSCF to retrieve S-CSCF capabilities or the assigned S-CSCF, from the SBI capable HSS by means of the Get service operation. If the consumer NF supports the feature "sharedData", the retrieved individual subscription data for the UE may contain shared data identifier identifying additional parts of subscription data shared by several UEs. If so, the Nhss_imsSubscriberDataManagement Service is also used by Consumer NFs to:

-	retrieve shared subscription data from the UDM by means of the Get service operation.

The Nhss_imsSubscriberDataManagement Service is also used by Consumer NFs to:

-	subscribe to notifications of data change by means of the Subscribe service operation. If the consumer NF supports the feature "sharedData", the consumer NF may also subscribe to notifications of shared data change by means of the Subscribe service operation.

-	to unsubscribe from notifications of data changes by means of the Unsubscribe service operation. If the feature "sharedData" is supported, it may also be used to unsubscribe from notifications of shared data changes by means of the Unsubscribe service operation.

-	get notified by means of the Notification service operation when HSS decides to modify the subscribed data. If the feature "sharedData" is supported by the consumer NF and the consumer NF has previously subscribed to notifications of shared data change, it is also used by the consumer NF to get notified by means of the Notification service operation when the HSS decides to modify the subscribed shared data.

It is also used by Consumer NFs to:

-	provide the information about the status of the subscription data management procedures.

-	update information (e.g. transparent data) associated to the subscription data management.

* * * Next Change * * * *

##### 5.3.2.2.1	General

The following procedures using the Get service operation are supported:

-	Repository Data Retrieval

-	IMS Identity Data (e.g. IMS associated identities including Implicit Registration Set and Alias Group) Retrieval

-	IMS Profile Data (e.g. IFCs) Retrieval

-	IMS Location Data Retrieval

-	IMS Registration Status Data Retrieval

-	Access Data (e.g. T-ADS) Retrieval

-	SRVCC Data (e.g. STN-SR) Retrieval

-	Service Data (e.g. DSAI) Retrieval

-	Retrieval Of Multiple Data Sets

-	Shared Subscription Data Retrieval

-	Trace Data Retrieval

-	PSI Activation State Retrieval

When the feature SharedData is supported and the retrieved UE-individual data (i.e. data other than Shared Subscription Data) contain SharedDataIds, the NF service consumer shall also retrieve the shared data identified by the received shared data Ids unless the identified shared data are already available at the NF service consumer. The order of sequence of sharedDataIds within UE-individual data is significant: Individual data take precedence over shared data; shared data "SharedDataX" identified by a sharedDataId X takes precedence over shared data "SharedDataY" identified by a sharedDataId Y if X appears before Y within the list of SharedDataIds in the UE-individual data.

* * * Next Change * * * *

##### 5.3.2.6.2	Data Change Notification To NF

Figure 5.3.2.6.2-1 shows a scenario where the HSS notifies the NF service consumer (that has subscribed to receive such notification) about subscription data change. The request contains the callbackReference URI as previously received in the ImsSdmSubscription.

Figure 5.3.2.6.2-1: Subscription Data Change Notification

1.	The HSS sends a POST request to the callbackReference as provided by the NF service consumer during the subscription.

2.	The NF service consumer responds with "204 No Content".

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the POST response body.

* * * Next Change * * * *

##### 5.3.2.7.4	STN-SR update

Figure 5.3.2.7.4-1 shows a scenario where the IMS-AS sends a request to the HSS to update the STN-SR. The request contains the UE's identity (/{imsUeId}) which shall be an IMS Public Identity or a Private Identity and an instruction to modify the STN-SR.

Figure 5.3.2.7.4-1: IMS AS updating STN-SR

1.	The IMS AS sends a PATCH request to the resource representing the UE's SRVCC Data.

2a.	On success, the HSS responds with "204 No Content".

2b.	If the resource does not exist (i.e. the user is not subscribed to SRVCC), HTTP status code "404 Not Found" should be returned including additional error information in the response body (in the "ProblemDetails" element).

2c.	If the operation is not authorized due to e.g IMS AS does not have the required permissions for the operation requested, HTTP status code "403 Forbidden" should be returned including additional error information in the response body (in "ProblemDetails" element).

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the PATCH response body.

* * * Next Change * * * *

##### 6.1.3.2.2	Resource Definition

Resource URI: {apiRoot}/nhss-ims-uecm/{apiVersion}/{imsUeId}/scscf-registration

This resource shall support the resource URI variables defined in table 6.1.3.2.2-1.

Table 6.1.3.2.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 6.1.1 |
| apiVersion | See clause 6.1.1 |
| imsUeId | Represents the IMS Subscription Identifier, either the IMS Public Identity (i.e. IMPU or Public PSI) or the Private Identity (i.e. IMPI or PSI). See 3GPP TS 23.228 [6] clause 4.3). IMS Public Identity is used with the PUT method; IMS Public Identity or IMS Private Identity is used with the PATCH methods. pattern: "^(impu-sip\:([a-zA-Z0-9_\-.!~*()&=+$,;?\/]+)\@([A-Za-z0-9]+([-A-Za-z0-9]+)\.)+[a-z]{2,}¦impu-tel\:\+[0-9]{5,15}¦impi-.+¦.+)$" |

* * * Next Change * * * *

##### 6.1.4.2.2	Operation Definition

This resource shall support the resource URI variables defined in table 6.1.4.2.2-1.

Table 6.1.4.2.2-1: URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 6.1.1 |
| apiVersion | See clause 6.1.1 |
| imsUeId | Represents the IMS Subscription Identifier, either the IMS Public Identity (i.e. IMPU or Public PSI) or the Private Identity (i.e. IMPI or PSI). See 3GPP TS 23.228 [6] clause 4.3). IMS Public Identity is used with the PUT method; IMS Public Identity or IMS Private Identity is used with the PATCH methods. pattern: "^(impu-sip\:([a-zA-Z0-9_\-.!~*()&=+$,;?\/]+)\@([A-Za-z0-9]+([-A-Za-z0-9]+)\.)+[a-z]{2,}¦impu-tel\:\+[0-9]{5,15}¦impi-.+¦.+)$" |

This operation shall support the request data structures specified in table 6.1.4.2.2-1 and the response data structure and response codes specified in table 6.1.4.2.2-2.

Table 6.1.4.2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AuthorizationInformation | M | 1 | Contains input for authorization, such as the authorization type or the visited network identifier. |

Table 6.1.4.2.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AuthorizationInfoResult | M | 1 | 200 OK | A response body containing result information or the registered S-CSCF shall be returned. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate one of the following application errors: - USER_NOT_FOUND - IDENTITY_NOT_REGISTERED |
| ProblemDetails | O | 0..1 | 403 Forbidden | The "cause" attribute may be used to indicate one of the following application errors: - AUTHORIZATION_REJECTED - IDENTITIES_DONT_MATCH |
| NOTE: In addition, common data structures as listed in table 6.1.7-1 are supported. |  |  |  |  |

* * * Next Change * * * *

#### 6.2.3.1	Overview

Figure 6.2.3.1-1: Resource URI structure of the Nhss_imsSDM API

Figure 6.2.3.1-2: Resource URI structure of the Nhss_imsSDM API

Table 6.2.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.2.3.1-1: Resources and methods overview

| Resource name (Archetype) | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| RepositoryData (Document) | /{imsUeId}/repository-data/ {serviceIndication} | GET | Retrieve repository data for a service indication |
|  |  | DELETE | Delete repository data for a service indication |
|  |  | PUT | Update repository data for a service indication |
| ImsAssociatedIdentities (Document) | /{imsUeId}/identities/ims-associated-identities | GET | Retrieve the UE's IMS associated identities (implicit and alias) |
| Msisdns (Document) | /{imsUeId}/identities/msisdns | GET | Retrieve the UE's MSISDNs (basic and additional) |
| PrivateIdentities (Document) | /{imsUeId}/identities/private-identities | GET | Retrieve the UE's Private Identities (IMPI and IMSI) |
| ImeiSv (Document) | /{imsUeId}/identities/imeisv | GET | Retrieve the UE's IMEISV |
| ImsProfile (Document) | /{imsUeId}/ims-data/profile-data | GET | Retrieve the UE's subscribed Profile |
| Ifcs (Document) | /{imsUeId}/ims-data/ profile-data/ifcs | GET | Retrieve the UE's subscribed Initial Filter Criteria |
| ServiceLevelTraceInformation (Document) | /{imsUeId}/ims-data/profile-data/service-level-trace-info | GET | Retrieve the UE's subscribed Service Level Trace Information |
| PriorityLevels (Document) | /{imsUeId}/ims-data/ profile-data/priority-levels | GET | Retrieve the UE's subscribed Service Priority Levels |
| ImsServerNameAndCaps (Document) | /{imsUeId}/ims-data/location-data | GET | Retrieve the UE's S-CSCF name and the server capabilities in IMS |
| ImsLocationData (Document) | /{imsUeId}/ims-data/location-data/server-name | GET | Retrieve the UE's S-CSCF name |
| ScscfCapabilities (Document) | /{imsUeId}/ims-data/location-data/scscf-capabilities | GET | Retrieve the UE's S-CSCF capabilities (mandatory and/or optional) |
| ImsRegistrationStatus (Document) | /{imsUeId}/ims-data/ registration-status | GET | Retrieve the UE's IMS registration status. |
| Subscriptions (Collection) | /{imsUeId}/subscriptions | POST | Create a subscription |
| Individual subscription (Document) | /{imsUeId}/subscriptions/ {subscriptionId} | DELETE | Delete the subscription identified by {subscriptionId}, i.e. unsubscribe |
|  |  | PATCH | Modify the sdm-subscription identified by {subscriptionId} |
| PsLocation (Document) | /{imsUeId}/access-data/ ps-domain/location-data | GET | Retrieve the UE's location in PS domain |
| CsLocation (Document) | /{imsUeId}/access-data/ cs-domain/location-data | GET | Retrieve the UE's location in CS domain |
| PsUserState (Document) | /{imsUeId}/access-data/ ps-domain/user-state | GET | Retrieve the UE's state in PS domain |
| CsUserState (Document) | /{imsUeId}/access-data/ cs-domain/user-state | GET | Retrieve the UE's state in CS domain |
| TadsInfo (Document) | /{imsUeId}/access-data/ ps-domain/tads-info | GET | Retrieve the UE's T-ADS information |
| UeRechabilityIpSubscriptions (Collection) | /{imsUeId}/access-data/ ps-domain/ue-reach-subs | POST | Create a subscription to UE Reachability for IP |
| UeRechabilityIpIndividual subscription (Document) | /{imsUeId}/access-data/ ps-domain/ue-reach-subs/ {subscriptionId} | DELETE | Delete the subscription identified by {subscriptionId}, i.e. unsubscribe |
| IpAddress (Document) | /{imsUeId}/access-data/ ps-domain/ip-address | GET | Retrieve the UE's stored IP Address Secure Binding Information |
| Csrn (Document) | /{imsUeId}/access-data/ cs-domain/csrn | GET | Retrieve a CSRN for the UE |
| ReferenceAccessLocation (Document) | /{imsUeId}/access-data/wireline-domain/reference-location | GET | Retrieve the UE's subscribed Reference Access Location |
| SrvccData (Document) | /{imsUeId}/srvcc-data | GET | Retrieve the UE's STN-SR and SRVCC capability |
| PsiActivationState (Document) | /{imsUeId}/service-data/ psi-state | GET | Retrieve a PSI activation status |
|  |  | PATCH | Update a PSI activation status |
| Dsai (Document) | /{imsUeId}/service-data/dsai | GET | Retrieve DSAI data |
|  |  | PATCH | Update DSAI data |
| SMSRegistrationInfo (Document) | /{imsUeId}/service-data/ sms-registration-info | GET | Retrieve the SMS Registration Information (e.g. IP-SM-GW address) |
|  |  | PUT | Creates or updates the SMS Registration Information (e.g. IP-SM-GW address) |
|  |  | DELETE | Deletes the SMS Registration Information (e.g. IP-SM-GW address) |
| SharedData (Collection) | /shared-data | GET | Retrieve shared data |
| SharedDataSubscriptions (Collection) | /shared-data-subscriptions | POST | Create a subscription |
| SharedDataIndividual subscription (Document) | /shared-data-subscriptions/{subscriptionId} | DELETE | Delete the subscription identified by {subscriptionId}, i.e. unsubscribe |

* * * Next Change * * * *

###### 6.2.3.19.3.2	PUT

This method shall support the URI query parameters specified in table 6.2.3.19.3.2-1.

Table 6.2.3.19.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.19.3.2-2 and the response data structures and response codes specified in table 6.2.3.19.3.2-3.

Table 6.2.3.19.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| RepositoryData | M | 1 | The Repository Data to be created or replaced. |

Table 6.2.3.19.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| RepositoryData | M | 1 | 201 Created | Upon success, a response body containing a representation of the created Individual RepositoryData resource shall be returned. |
| RepositoryData | M | 1 | 200 OK | Upon success, a response body containing a representation of the updated Individual RepositoryData resource shall be returned. |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate the following application error: - USER_NOT_FOUND |
| ProblemDetails | O | 0..1 | 403 Forbidden | The "cause" attribute may be used to indicate one of the following application errors: - OPERATION_NOT_ALLOWED |
| ProblemDetails | O | 0..1 | 409 Conflict | The "cause" attribute may be used to indicate one of the following application errors: - OUT_OF_SYNC |
| ProblemDetails | O | 0..1 | 413 Payload Too Large | The "cause" attribute may be used to indicate one of the following application errors: - TOO_MUCH_DATA |
| NOTE: In addition, common data structures as listed in table 6.1.7-1 are supported. |  |  |  |  |

* * * Next Change * * * *

##### 6.2.6.2.30	Type: SrvccData

Table 6.2.6.2.30-1: Definition of type SrvccData

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| stnSr | StnSr | M | 1 | Iindicates the STN-SR (Session Transfer Number for SRVCC) of the UE. |
| ueSrvccCapabilities | array(SrvccCapabilty) | O | 1..N | List of accesses supported by UE SRVCC capability (e.g. 4G, 5G). Absence of this attribute indicates that UE is not SRVCC capable. |

* * * Next Change * * * *

## A.3	Nhss_imsSDM API

***************** not shown for clarity ***************

***************** not shown for clarity ***************

ServiceInformation:

anyOf:

- type: string

enum:

- INCLUDE_REGISTER_REQUEST

- INCLUDE_REGISTER_RESPONSE

- type: string

***************** not shown for clarity ***************

* * * End of Change * * * *