3GPP TSG-CT WG4 Meeting #98e	C4-203437

# E-Meeting, 02nd – 12th June 2020

Title:	LS on Bulk operation of LCS-service

Response to:

Release:	Release-16

Work Item:	5G_eLCS

Source:	CT WG4

To:	SA WG2

Cc:	CT WG3

Contact Person:

Name:	Qingfen Liu

Tel. Number:

E-mail Address:	liuqingfen@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

CT4 has analysed the procedure of Bulk Operation of LCS Service Request targeting Multiple UEs, as specified in clause 6.8 of TS 23.273, and identified some confusions. CT4 would like to ask SA2 to clarify the following questions:

- When using Service Based Interface (SBI), which node is expected to be the aggregation point of Bulk Operation of LCS Service, i.e. the node who resolves the group identifier to member UEs and aggregate the location requests and responses for each UE in the group, NEF or GMLC or both?

- If the GMLC is expected to be the aggregation point of Bulk Operation of LCS Service, what should the GMLC respond for a request of bulk operation, especially for 5GC_MT_LR procedure?

NOTE:	CT4 observed that if NEF sends a 5GC-MT-LR request towards GMLC for group of UEs, there is no way for GMLC to relay the multiple responses (one per UE) to the NEF, using current service operations specified in TS 23.273 (Only one service response is allowed per service request in SBI and location for one UE is allowed in Ngmlc_Location_ProvideLocation response).

- If used as aggregation point, when does the GMLC / NEF acknowledge the request for Bulk Operation, i.e. does it respond the LCS client / AF after it received all positioning responses from network side for all UEs in the group or respond immediately?

- If GMLC / NEF responds the LCS client / AF immediately, based on what criterion does the GMLC / NEF decide to respond success or failure or partial success? And How should GMLC / NEF to notify the LCS Client / AF if the requests for positioning for some UEs fail because of some different reasons (e.g. The UE is not online) after GMLC / NEF responded success to LCS client / AF?

2. Actions:

To SA WG2 group.

ACTION: 	CT4 kindly requests SA2 to clarify above questions.

3. Date of Next CT4 Meetings:

3GPP TSG CT4#99e	08/2020	E-Meeting