3GPP TSG-CT WG4 Meeting #98e	C4-203425

# E-Meeting, 02nd – 12th June 2020

# Title:	LS on Clarification on event of UE Reachability

# Release:	Rel-15

# Work Item:	5GS_Ph1

Source:	CT4

To:	SA2

Cc:

Contact Person:

Name:	Lu Yunjie

Tel. Number:

E-mail Address:	jones.lu@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:

1. Overall Description:

CT4 would like to inform SA2 that CT4 has agreed TS 29.518 CR #370 (attached) in Rel-15, which complementing the UE Reachability event on AMF Event Exposure service, as specified in TS 23.502 v15.9.0. During the implementation, CT4 however discovered certain aspects that are not clearly specified in Stage 2:

TS 23.502 clause 4.2.3.3 has specified following descriptions:

…

If the AMF has determined the UE is unreachable for the SMF (e.g., due to the UE in MICO mode, the UE using extended idle mode DRX or the UE is only registered over non-3GPP access and its state is CM-IDLE), then the AMF rejects the request from the SMF. The AMF may include in the reject message an indication that the SMF need not trigger the Namf_Communication_N1N2MessageTransfer Request to the AMF, if the SMF has not subscribed to the event of the UE reachability….

…

If the SMF receives an indication from the AMF that the UE is unreachable or reachable only for regulatory prioritized service and the SMF determines that Extended Buffering does not apply, the SMF may, based on network policies, either:

-    indicate to the UPF to stop sending Data Notifications;

-    indicate to the UPF to stop buffering DL data and discard the buffered data;

-    indicate to the UPF to stop sending Data Notifications and stop buffering DL data and discard the buffered data; or

-    refrains from sending further Namf_Communication_N1N2MessageTransfer message for DL data to the AMF while the UE is unreachable.

Observation-1:

If the SMF does not subscribe to the UE reachability event and it refrains from sending subsequent N1N2MessageTransfer requests to AMF (e.g. DDN) upon receiving the indication of UE unreachable, the SMF cannot be informed later on that the UE is reachable again and it will not triggering N1N2MessageTransfer request any longer and the data service would be stuck.

CT4 assumes that if an SMF refraining from sending N1N2MessageTransfer requests when UE is not reachable or in not allowed area, the SMF shall subscribe to the AMF event exposure for UE reachability. Please confirm CT4's understanding on this requirement or please clarify otherwise if CT4's understanding is not correct.

Observations-2:

The AMF informs notify the subscribers with UE "UNREACHABLE" event state when the UE reachability maintained in AMF transit from Reachable to Unreachable, e.g. when the Mobile Reachable Timer expires for 3GPP access, as specified in TS 23.501 clause 5.4. But AMF also identifies the UE is not reachable in above scenarios like the UE becomes CM-IDLE in MICO mode or the UE enters eDRX. A complete and exact list of triggers on AMF sending UE "UNREACHABLE" event state will be very helpful to avoid potential AMF misbehaviours among different implementations.

2. Actions:

To SA WG2 group.

ACTION: 	CT2 kindly asks SA2 to take the above observations into consideration and provide further clarifications on the mentioned concerns in stage 2.

3. Date of Next CT4 Meetings:

3GPP TSG CT4#99e	08/2020	E-Meeting