3GPP TSG-CT WG4 Meeting #98e	C4-203

# E-Meeting, 02nd – 12th June 2020									was C4-203123

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.509 | CR | 0094 | rev | 1 | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | N5GC device Authentication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, CableLabs, Charter Communications |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5WWC |  |  |  |  |  | Date: |  |  | 2020-06-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For Non-5G Capable (N5GC) devices an indication within Authentication Info is needed; see TS 33.501 Annex O. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add an N5GC Indicator to AuthenticationInfo |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment with stage 2 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 5.2.2.2.3.3, 6.1.6.2.2, 6.1.6.2.7, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible new features to the Nausf_UEAuthentication API. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Begin of Change * * * *

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AMF	Access and Mobility Management Function

API	Application Programming Interface

AUSF	Authentication Server Function

FN-RG	Fixed Network RG

MAC	Message Authentication Code

N5GC	Non-5G-Capable

NF	Network Function

RG	Residential Gateway

SEAF	SEcurity Anchor Function

SoR	Steering of Roaming

URI	Uniform Resource Identifier

UPU	UE Parameters Update

W-AGF	Wireline Access Gateway Function

* * * * Next of Change * * * *

###### 5.2.2.2.3.3	EAP method: EAP-TLS

The EAP-TLS method can be used in private networks as an EAP method (see 3GPP TS 33.501 [8] Annex B.1). The corresponding stage 3 implementation is described in Annex B.

The EAP-TLS method is applicable for N5GC devices behind Cable RGs in private networks or in deployment scenarios with wireline access; see 3GPP TS 33.501 [8] Annex O.

* * * * Next of Change * * * *

##### 6.1.6.2.2	Type: AuthenticationInfo

Table 6.1.6.2.2-1: Definition of type AuthenticationInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supiOrSuci | SupiOrSuci | M | 1 | Contains the SUPI or SUCI of the UE. |
| servingNetworkName | ServingNetworkName | M | 1 | Contains the Serving Network Name. |
| resynchronizationInfo | ResynchronizationInfo | O | 0..1 | Contains RAND and AUTS; see 3GPP 33.501 [8] clause 9.4. |
| pei | Pei | O | 0..1 | Permanent Equipment Identifier |
| traceData | TraceData | O | 0..1 | Contains TraceData provided by the UDM to the AMF |
| udmGroupId | NfGroupId | O | 0..1 | Identity of the UDM group serving the SUPI |
| routingIndicator | String | O | 0..1 | When present, it shall indicate the Routing Indicator of the UE. Pattern: '^[0-9]{1,4}$' |
| cagId | CagId | O | 0..1 | CAG ID of the CAG cell. |
| n5gcInd | boolean | O | 0..1 | N5GC device indicator (see 3GPP TS 33.501 [8]) When present, this IE shall be set as follows: - true: authentication is for a N5GC device; - false (default): authentication is not for a N5GC device. See NOTE |
| NOTE: The attribute n5gcInd is used for EAP-TLS, which is described in the informative annex O of 3GPP TS 33.501 [8] and is not mandatory to support. |  |  |  |  |

* * * * Next of Change * * * *

##### 6.1.6.2.7	Type: EapSession

Table 6.1.6.2.7-1: Definition of type EapSession

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| eapPayload | EapPayload | M | 1 | Contains the EAP packet (see IETF RFC 3748 [18]). If no EAP packet has been provided by the UE the null value is conveyed to the AUSF. |
| kSeaf | Kseaf | C | 0..1 | Shall be absent for N5GC device authentication; otherwise: If the authentication is successful, the Kseaf shall be included |
| _links | map(LinksValueSchema) | C | 1..N | If the EAP session requires another exchange e.g. for EAP-AKA' notification, this IE shall contain a member whose name is "eap-session" and the URI to continue the EAP session. See NOTE. |
| authResult | AuthResult | C | 0..1 | Indicates the result of the authentication. |
| supi | Supi | C | 0..1 | If the authentication is successful and if the AMF had provided a SUCI, this IE shall contain the SUPI of the UE. |
| NOTE: In the current version of this API, only 0 or 1 hypermedia link is provided. |  |  |  |  |

* * * * Next of Change * * * *

## A.2	Nausf_UEAuthentication API

openapi: 3.0.0

**************text not shown for clarity**************

AuthenticationInfo:

type: object

properties:

supiOrSuci:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupiOrSuci'

servingNetworkName:

$ref: 'TS29503_Nudm_UEAU.yaml#/components/schemas/ServingNetworkName'

resynchronizationInfo:

$ref: 'TS29503_Nudm_UEAU.yaml#/components/schemas/ResynchronizationInfo'

pei:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Pei'

traceData:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TraceData'

udmGroupId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfGroupId'

routingIndicator:

type: string

pattern: '^[0-9]{1,4}$'

cagId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/CagId'

n5gcInd:

type: boolean

default: false

required:

- supiOrSuci

- servingNetworkName

**************text not shown for clarity**************

* * * * End Of Change * * * *