| ETSI TC SCP Meeting #52 Turin, Italy, 28-30 Sept 2011 |  |  |  |  |  |  |  |  |  |  |  |  |  | SCP(11)0289r1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ETSI TC SCP TEC Meeting #39 Sophia Antipolis, France, 06 - 08 June 2011 |  |  |  |  |  |  |  |  |  |  |  |  |  | SCPTEC(11)0115r1 |  |  |  |  |  |  |  |
| TC SCP CR-Form-v1.8.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 102 705 |  |  |  | CR | 017 |  |  |  rev |  |  |  |  Current version: |  |  |  |  | 9.2.0 |  |  |  |
| For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects:  |  |  |  |  | smart card |  | X | terminal |  |  |  |  | server / network entity |  |  |  |  |  |  |  |  |
| Title:  |  | Clarification of HCIMessage.getReceiveBuffer(), getReceiveOffset() and getReceiveLength() |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source:  |  | SCP TEC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item name:  |  | TEI9 |  |  |  |  |  |  |  |  |  |  |  | Date:  |  | 31/08/2011 |  |  |  |  |  |
| Category:  |  | F |  |  |  |  |  |  |  |  |  |  |  | Release:  |  | Rel-9 |  |  |  |  |  |
|  |  | Use one of the following categories: F (essential correction) A (corresponds to a correction in an earlier release) B (Addition of feature), C (Functional modification of feature) D (Editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: REL-6 (Release 6) REL-7 (Release 7) REL-8 (Release 8) REL-9 (Release 9) REL-10 (Release 10) REL-11 (Release 11) |  |  |  |  |  |  |
| TC SCP use of the status "Accepted" for change requests: - At the TC SCP working group level, the status "Accepted" means that a change request is accepted for presentation to TC SCP Plenary. - At the TC SCP Plenary level, the status "Accepted" means that a change request is accepted for implementation in the targeted specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reason for change:  |  |  | According to section 4.1 an HCIMessage object encapsulates an HCP message which consists of the one byte header and the data part. The methods HCIMessage.getReceiveBuffer(), getReceiveOffset() and getReceiveLength() refer to the data part of the HCP message. This is not clearly described. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change:  |  |  | Add a statement in the description of the methods that these refer to data part of the HCP message. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if  not accepted: |  |  | Potential interoperability issues. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| New tag value defined within the CR? |  |  |  | If ticked, add document number of related CR to TS 101 220: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected:  |  |  | Annex A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs  |  |  |  | Other core specifications  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Affected: |  |  |  | Test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Annex A (normative):

Java Card™ Platform HCI API for the UICC

HCIMessage.java:

package uicc.hci.framework;

[…]

public interface HCIMessage {

[…]

/**

* Return a reference to the underlying receive buffer. The length of this

* buffer is system dependent; it is guaranteed to be at least 270 bytes.

*

* <P>When the listener is invoked with an incoming message the usable data

* are contained in the buffer returned by this method

.

*

* <ul>

* <li>The framework shall copy data into the receive buffer up to the end of the buffer.

* When the HCP message is longer then the available buffer length the HCIMessage shall be set

* as not complete.

*

* <li> The application may use the whole receive buffer for

* its internal purposes. If the buffer is used for manipulation of sensitive

* data it shall be cleared by the Applet before returning to the contactless framework.

*

* <li>The current fragment of the data part of the HCP message starts at the location indicated by getReceiveOffset() and has the length indicated by getReceiveLength()

*

* <li>The content of the buffer outside of the part which is used for the current fragment of the data part of the HCP message is undefined.

* </ul>

*

* <p>

* Note:

* <ul>

* <li><em>References to the receive buffer byte array

* may be stored in local variables or method parameters.</em>

* <li><em>References to the receive buffer byte array

* cannot be stored in class variables or instance variables or array components.</em>

* </ul>

*

* @return the buffer holding the current fragment of the HCP message.

*/

public byte[] getReceiveBuffer();

/**

* This method returns the offset of the data part of the received HCP message in the receive buffer.

*

* @return the offset into the buffer, retrieved via <code>getReceiveBuffer()</code>.

*/

public short getReceiveOffset();

/**

* This method returns the length of the data part of the received HCP message. If the message is

* not complete then the returned value is the length of the current fragment of the data part the HCP message.

*

* @return the size of the current fragment of the data part of the HCP message available in the receive buffer. If the Message object

*         delivered to the Listener interface resulted from a previous call to the

*         <code>requestCallbackNotification</code> method the return value shall be zero.

*/

public short getReceiveLength();

}