| ETSI TC SCP Meeting #52 Torino, Italy, 28 - 30 September 2011 |  |  |  |  |  |  |  |  |  |  |  |  |  | Tdoc SCP(11)0318 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TC SCP CR-Form-v1.8.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 102 705 |  |  |  | CR | 023 |  |  |  rev |  |  |  |  Current version: |  |  |  |  | 10.0.0 |  |  |  |
| For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects:  |  |  |  |  | smart card |  | X | terminal |  |  |  |  | server / network entity |  |  |  |  |  |  |  |  |
| Title:  |  | Correction of several inaccuracies in HCIMessage.isHeading() and isComplete() |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source:  |  | SCP TEC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item name:  |  | TEI9 |  |  |  |  |  |  |  |  |  |  |  | Date:  |  | 29/09/2011 |  |  |  |  |  |
| Category:  |  | A |  |  |  |  |  |  |  |  |  |  |  | Release:  |  | Rel-10 |  |  |  |  |  |
|  |  | Use one of the following categories: F (essential correction) A (corresponds to a correction in an earlier release) B (Addition of feature), C (Functional modification of feature) D (Editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: REL-6 (Release 6) REL-7 (Release 7) REL-8 (Release 8) REL-9 (Release 9) REL-10 (Release 10) REL-11 (Release 11) |  |  |  |  |  |  |
| TC SCP use of the status "Accepted" for change requests: - At the TC SCP working group level, the status "Accepted" means that a change request is accepted for presentation to TC SCP Plenary. - At the TC SCP Plenary level, the status "Accepted" means that a change request is accepted for implementation in the targeted specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reason for change:  |  |  | There are several inaccuracies in the description of the methods HCIMessage.isComplete() and HCIMessage.isHeading() which are caused by the use of the term HCI message which is undefined. There is even an obsolete editors note from the drafting phase of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change:  |  |  | The description of the methods HCIMessage.isComplete() and HCIMessage.isHeading() is improved. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if  not accepted: |  |  | Inaccurate and confusing specification resulting in potential interoperability issues. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| New tag value defined within the CR? |  |  |  | If ticked, add document number of related CR to TS 101 220: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected:  |  |  | Annex A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs  |  |  |  | Other core specifications  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Affected: |  |  |  | Test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Annex A (normative):

### Java Card™ Platform HCI API for the UICC

### In HCIMessage.java:

/******************************************************************************

*

******************************************************************************/

package uicc.hci.framework;

/**

* An instance of <code>HCIMessage</code> is used to get access to the content of a received

* HCI message.

*

* Every HCP message that is not longer then 270 bytes has to be presented to the

* Applet as a complete message. In this case the contactless framework is responsible

* to manage the defragmentation.

*

* The contactless framework shall support outgoing HCIMessages of at least 256 bytes.

*

* <P>Sub interfaces will typically have additional methods for sending HCI service

* specific HCI messages.

* <P>Instances of <code>HCIMessage</code> hide the fragmentation of messages on the HCI layer

* (see TS 102 622). However, as receive and send buffers with limited capacity will

* be used it provides methods to check if

* a message is complete (see <code>isHeading()</code> and <code>isComplete()</code>) in order to support

* streaming of long messages.

*

* <P>Instances of this object shall be temporary JCRE entry point objects.

*

* <P>

* When the applet is invoked by an event not related to an incoming HCI message (e.g. after

* <code>requestCallbackNotification</code> method), <code>HCIMessage</code>) content is undefined.

*

* @see uicc.hci.framework.HCIListener

**/

public interface HCIMessage {

[…]

/**

* Checks if the current content contains the heading part of an HCP message

*

* The heading information indicates the first part of an HCP message; with the

* <code>isComplete()</code> method it is possible to check whether the complete HCP message

* has already been received.

*

* @return true if the current message is the heading part of the message

*/

public boolean isHeading();

/**

* Checks for the completeness of the HCP message, i.e. the last received

* HCP packet had the chaining bit CB set to 1

*

* @return true if the message is complete

*/

public boolean isComplete();

[…]

}