| ETSI TC SCP Meeting #56 Freising, Germany, 27-28 September 2012 |  |  |  |  |  |  |  |  | SCP(12)000218 |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ETSI TC SCP TEC Meeting #44 Sophia Antipolis, France, 10-13 Sept 2012 |  |  |  |  |  |  |  |  | Tdoc SCPTEC(12)000131 |  |  |  |  |  |  |  |  |  |  |  |  |
| TC SCP CR-Form-v1.8.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 102 705 |  |  |  | CR | 035 |  |  |  rev |  |  |  |  |  Current version: |  |  |  | 10.0.0 |  |  |  |
| For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects:  |  |  |  |  | smart card |  | x | terminal |  |  |  |  |  | server / network entity |  |  |  |  |  |  |  |
| Title:  |  | Activation of events |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source:  |  | SCP TEC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item name:  |  | TEI9 |  |  |  |  |  |  |  |  |  |  | Date:  |  |  | 13/09/2012 |  |  |  |  |  |
| Category:  |  | A |  |  |  |  |  |  | Release:  |  |  |  |  |  |  | REL-10 |  |  |  |  |  |
|  |  | Use one of the following categories: F (essential correction) A (corresponds to a correction in an earlier release) B (Addition of feature), C (Functional modification of feature) D (Editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: REL-6 (Release 6) REL-7 (Release 7) REL-8 (Release 8) REL-9 (Release 9) REL-10 (Release 10) REL-11 (Release 11) |  |  |  |  |  |  |
| TC SCP use of the status "Accepted" for change requests: - At the TC SCP working group level, the status "Accepted" means that a change request is accepted for presentation to TC SCP Plenary. - At the TC SCP Plenary level, the status "Accepted" means that a change request is accepted for implementation in the targeted specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reason for change:  |  |  | SCP TEST requested a clarification about the activation of event and the registration of Listener interfaces during the lifetime of the applet |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change:  |  |  | Clarify that events can always be activated by the applet, but will only be sent when the corresponding Listener interface is registered. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if  not accepted: |  |  | Difficult to understand the specification for applet developers and SCP TEST can not complete test specification for REL-9. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| New tag value defined within the CR? |  |  |  | If ticked, add document number of related CR to TS 101 220: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected:  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs  |  |  |  | Other core specifications  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Affected: |  |  |  | Test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

package uicc.hci.framework;

/**

* This interface is the base interface for any HCI service.

* <P>

* An HCI service instance shall be a permanent JCRE Entry Point Object

* <P>

* It provides the means for an Applet instance to get access to the resources

* of a specific HCI service. An Applet instance can retrieve an implementation of

* a specific service via the <code>HCIDevice</code>.

* <P>

* For a service object that is implementing one of the subinterfaces defined in this API

* the Applet can only register the corresponding Listener defined in the same package

* (e.g. a <code>CardEmulationService</code> shall only accept the regsitration of a

* <code>CardEmulationListener</code>.

* <P>

* An Applet instance shall only be able to register one type of listener per service

* at any time. Listener registration is persistent.

* <P>

* To receive events via the registered Listeners an Applet instance shall activate the events.

* All events shall be deactivated by default. Event activation is persistent.

**/

public interface HCIService {

/**

* This method registers a Listener object to the Service instance

*

* @param listener listener to be registered with the given service

* @exception HCIException with reason code

*            <ul>

*            <li><code>HCIException.HCI_WRONG_LISTENER_TYPE</code> in case the listener Object registerd to the service

*            is not implementing the corresponding interface of the HCIService instance.

* <ul>

* <li>a <code>ReaderListener</code> can only be registered to a <code>ReaderService</code>,

* <li>a <code>CardEmulationListener</code> can only be registered to a <code>CardEmulationService</code>,

* <li>a <code>ConnectivityListener</code> can only be registered to a <code>ConnectivityService</code>

* </ul>

*           <li><code>HCIException.HCI_LISTENER_ALREADY_REGISTERED</code> in case a listener Object of the same type has already been registered.

* </ul>

*/

public void register(HCIListener listener) throws HCIException;

/** This method unregisters the given listener from the service.

* <P>

*  Calling <code>deregister</code> with an argument that does not identify a

*  currently registered <code>HCIListener</code> has no effect.

*

* @param listener listener to be unregistered from the service

*/

public void deregister(HCIListener listener);

/**

* This method shall not be used. It is retained for binary interoperability

* reasons only and will throw an exception when invoked

*

* @param event without any meaning (as any invocation of the method will throw and exception)

* @throws HCIEception with reason code <code>HCI_ACCESS_NOT_GRANTED</code>

*/

public void requestCallbackNotification(byte event) throws HCIException;

/**

* This method is used to activate an event.

* <p>

* The Applet can at any time activate an event, it will receive

* this event only when the corresponding <code>Listener</code>

* sub-interface is registered.

* </p>

*

* <ul>

* <li><P>Possible values for the parameter <code>event</code> for the different Service objects are:

* <ul>

* <li>For all *Service interfaces:

* <ul>

* <li>HCIListenr.EVENT_HCI_TRANSMISSION_FAILED

* <li>HCIListener.EVENT_HCI_RECEPTIONS_FAILED

* </ul>

* <li>for CardEmulationService:

* <ul>

* <li><code>CardEmulationListener.EVENT_GET_PARAMETER_RESPONSE</code>,

* <li><code>CardEmulationListener.EVENT_ON_SEND_DATA</code>,

* <li><code>CardEmulationListener.EVENT_FIELD_OFF</code>.

* </ul>

* <li>for ReaderService:

* <ul>

* <li><code>ReaderListener.EVENT_GET_PARAMETER_RESPONSE</code>,

* <li><code>ReaderListener.EVENT_WRITE_EXCHANGE_DATA_RESPONSE</code>,

* <li><code>ReaderListener.EVENT_TARGET_DISCOVERED</code>.

* </ul>

* <li>for ConnectivityService:

* <ul>

* <li><code>ConnectivityListener.EVENT_STANDBY</code>.

* </ul>

* </ul>

* @param event event to be activated

* @throws HCIEception with reason code

*     <ul>

*      <li><code>HCI_WRONG_EVENT_TYPE<code> in case a wrong event was activated for this Service instance

*      <li><code>HCI_CONDITIONS_NOT_SATISFIEDE<code> if one or more conditions to activate the event are not satisfied

*      <li><code>HCI_CURRENTLY_DISABLED</code> if the interface to the contactless frontend (CLF) is currently disabled and the

*          event is <code>ReaderListener.EVENT_TARGET_DISCOVERED</code>

*     </ul>

*/

public void activateEvent(byte event) throws HCIException;

/**

* This method is used to deactivate an event from the list of activated events.

*

* <ul>

* <li><P>Possible values for the parameter <code>event</code> for the different Service objects are:

* <ul>

* <li>For all *Service interfaces:

* <ul>

* <li>HCIListenr.EVENT_HCI_TRANSMISSION_FAILED

* <li>HCIListener.EVENT_HCI_RECEPTIONS_FAILED

* </ul>

* <li>for CardEmulationService:

* <ul>

* <li><code>CardEmulationListener.EVENT_GET_PARAMETER_RESPONSE</code>,

* <li><code>CardEmulationListener.EVENT_ON_SEND_DATA</code>,

* <li><code>CardEmulationListener.EVENT_FIELD_OFF</code>.

* </ul>

* <li>for ReaderService:

* <ul>

* <li><code>ReaderListener.EVENT_GET_PARAMETER_RESPONSE</code>,

* <li><code>ReaderListener.EVENT_WRITE_EXCHANGE_DATA_RESPONSE</code>,

* <li><code>ReaderListener.EVENT_TARGET_DISCOVERED</code>.

* </ul>

* <li>for ConnectivityService:

* <ul>

* <li><code>ConnectivityListener.EVENT_STANDBY</code>.

* </ul>

* </ul>

* @param event event to be deactivated

* @throws HCIEception with reason code <code>HCIException.HCI_WRONG_EVENT_TYPE<code> in case a wrong event was deactivated for this Service instance

*          or in case the event wasn't activated before.

*/

public void deactivateEvent(byte event) throws HCIException;

/**

* return the activation state of an event.

*

* <ul>

* <li><P>Possible values for the parameter <code>event</code> for the different Service objects are:

* <ul>

* <li>For all *Service interfaces:

* <ul>

* <li>HCIListenr.EVENT_HCI_TRANSMISSION_FAILED

* <li>HCIListener.EVENT_HCI_RECEPTIONS_FAILED

* </ul>

* <li>for CardEmulationService:

* <ul>

* <li><code>CardEmulationListener.EVENT_GET_PARAMETER_RESPONSE</code>,

* <li><code>CardEmulationListener.EVENT_ON_SEND_DATA</code>,

* <li><code>CardEmulationListener.EVENT_FIELD_OFF</code>.

* </ul>

* <li>for ReaderService:

* <ul>

* <li><code>ReaderListener.EVENT_GET_PARAMETER_RESPONSE</code>,

* <li><code>ReaderListener.EVENT_WRITE_EXCHANGE_DATA_RESPONSE</code>,

* <li><code>ReaderListener.EVENT_TARGET_DISCOVERED</code>.

* </ul>

* <li>for ConnectivityService:

* <ul>

* <li><code>ConnectivityListener.EVENT_STANDBY</code>.

* </ul>

* </ul>

*

* @param event event to request the activation state

* @return true if the event is activated for this listener, else false

* @throws HCIEception with reason code <code>HCIException.HCI_WRONG_EVENT_TYPE<code>  in case the event was not one of the possible values.

*/

public boolean getEventNotificationStatus(byte event) throws HCIException;

}