| ETSI TC SCP Meeting #52 Torino, Italy, 28 - 30 September 2011 |  |  |  |  |  |  |  |  |  |  |  |  |  | Tdoc SCP(11)0319 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ETSI TC SCP TEC Meeting #40 Sophia Antipolis, France, 29 Aug. - 01 Sept. 2011 |  |  |  |  |  |  |  |  |  |  |  |  |  | Tdoc SCPTEC(11)0113r1 |  |  |  |  |  |  |  |
| TC SCP CR-Form-v1.8.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 102 705 |  |  |  | CR | 024 |  |  |  rev | - |  |  |  Current version: |  |  |  |  | 10.0.0 |  |  |  |
| For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects:  |  |  |  |  | smart card |  | X | terminal |  |  |  |  | server / network entity |  |  |  |  |  |  |  |  |
| Title:  |  | Deprecation of requestCallbackNotification method |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source:  |  | SCP TEC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item name:  |  | TEI 9 |  |  |  |  |  |  |  |  |  |  |  | Date:  |  | 29/09/2011 |  |  |  |  |  |
| Category:  |  | A |  |  |  |  |  |  |  |  |  |  |  | Release:  |  | Rel-10 |  |  |  |  |  |
|  |  | Use one of the following categories: F (essential correction) A (corresponds to a correction in an earlier release) B (Addition of feature), C (Functional modification of feature) D (Editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: REL-6 (Release 6) REL-7 (Release 7) REL-8 (Release 8) REL-9 (Release 9) REL-10 (Release 10) REL-11 (Release 11) |  |  |  |  |  |  |
| TC SCP use of the status "Accepted" for change requests: - At the TC SCP working group level, the status "Accepted" means that a change request is accepted for presentation to TC SCP Plenary. - At the TC SCP Plenary level, the status "Accepted" means that a change request is accepted for implementation in the targeted specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reason for change:  |  |  | The use of the HCI API method “requestCallbackNotification” of the “Interface HCIService” by an application may interfere with the operation of an application assigned for communication over the HCI interface by the framework, i.e. a reader mode application in Life Cycle State activated or a selected card emulation mode application. Also, the original use case for the method, to allow a reader mode application get access to the HCI interface through the message object returned by that method does no longer apply because this is now done by activating a dedicated event using the “activateEvent” method. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change:  |  |  | Deprecate the method “requestCallbackNotification” in Interface HCIService. The method is retained for binary compatibility reasons and an “HCIException” is thrown whenever it is invoked. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if  not accepted: |  |  | Applications not assigned for contactless communication by the HCI framework can cause interference with contactless transactions. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| New tag value defined within the CR? |  |  |  | If ticked, add document number of related CR to TS 101 220: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected:  |  |  | Annex A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs  |  |  |  | Other core specifications  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Affected: |  |  |  | Test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### In HCIService.java

/**

* @deprecated  This method shall not be used. It is retained for binary interoperability reasons only and

* will throw an exception when invoked.

* @param event without any meaning (as any invocation of the method will throw an exception)

* @throws HCIEception with reason code <code>HCI_ACCESS_NOT_GRANTED</code>

*/

@Deprecated public void requestCallbackNotification(byte event) throws HCIException;