| ETSI TC SCP Meeting #52 Turin, Italy, 28-30 Sept 2011 |  |  |  |  |  |  |  |  |  |  |  |  |  | SCP(11)0290r1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ETSI TC SCP TEC Meeting #40 Sophia Antipolis, France, 29 Aug. - 01 Sept 2011 |  |  |  |  |  |  |  |  |  |  |  |  |  | SCPTEC(11)0116r1 |  |  |  |  |  |  |  |
| TC SCP CR-Form-v1.8.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 102 705 |  |  |  | CR | 018 |  |  |  rev |  |  |  |  Current version: |  |  |  |  | 9.2.0 |  |  |  |
| For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects:  |  |  |  |  | smart card |  | X | terminal |  |  |  |  | server / network entity |  |  |  |  |  |  |  |  |
| Title:  |  | Missing constant in HCIMessage corresponding to HCI response ANY_E_PIPE_NOT_OPENED |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source:  |  | SCP TEC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item name:  |  | TEI9 |  |  |  |  |  |  |  |  |  |  |  | Date:  |  | 31/08/2011 |  |  |  |  |  |
| Category:  |  | F |  |  |  |  |  |  |  |  |  |  |  | Release:  |  | Rel-9 |  |  |  |  |  |
|  |  | Use one of the following categories: F (essential correction) A (corresponds to a correction in an earlier release) B (Addition of feature), C (Functional modification of feature) D (Editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: REL-6 (Release 6) REL-7 (Release 7) REL-8 (Release 8) REL-9 (Release 9) REL-10 (Release 10) REL-11 (Release 11) |  |  |  |  |  |  |
| TC SCP use of the status "Accepted" for change requests: - At the TC SCP working group level, the status "Accepted" means that a change request is accepted for presentation to TC SCP Plenary. - At the TC SCP Plenary level, the status "Accepted" means that a change request is accepted for implementation in the targeted specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reason for change:  |  |  | Missing constant for HCI response ANY_E_PIPE_NOT_OPENED in HCIMessage |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change:  |  |  | According to TS 102 622 section 6.2 Table 16 a possible value of the response to an HCI command is ANY_E_PIPE_NOT_OPENED. The corresponding Java constant in the API is missing. A reference to the RESP_ANY_* constants is added to the description of the method HCIMessage.getInstruction(). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if  not accepted: |  |  | Incomplete support of the HCI specification TS 102 622. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| New tag value defined within the CR? |  |  |  | If ticked, add document number of related CR to TS 101 220: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected:  |  |  | Annex A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs  |  |  |  | Other core specifications  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Affected: |  |  |  | Test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Annex A (normative):

### Java Card™ Platform HCI API for the UICC

### In HCIMessage.java:

/******************************************************************************

*

******************************************************************************/

package uicc.hci.framework;

/**

* An instance of <code>HCIMessage</code> is used to get access to the content of a received

* HCP message.

*

* Every HCI message that is not longer then 270 bytes has to be presented to the

* Applet as a complete message. In this case the contactless framework is responsible

* to manage the defragmentation.

*

* The contactless framework shall support outgoing HCIMessages of at least 256 bytes.

*

* <P>Sub interfaces will typically have additional methods for sending HCI service

* specific HCI messages.

* <P>Instances of <code>HCIMessage</code> hide the fragmentation of messages on the HCI layer

* (see TS 102 622). However, as receive and send buffers with limited capacity will

* be used it provides methods to check if

* a message is complete (see <code>isHeading()</code> and <code>isComplete()</code>) in order to support

* streaming of long messages.

*

* <P>Instances of this object shall be temporary JCRE entry point objects.

*

* <P>

* When the applet is invoked by an event not related to an incoming HCI message (e.g. after

* <code>requestCallbackNotification</code> method), <code>HCIMessage</code>) content is undefined.

*

* @see uicc.hci.framework.HCIListener

**/

public interface HCIMessage {

// ---------  Message types -------------

/** Indicates the HCI message type "command" */

public static final byte TYPE_COMMAND = (byte) 0x00;

/** Indicates the HCI message type "event" */

public static final byte TYPE_EVENT = (byte) 0x40;

/** Indicates the HCI message type "response" */

public static final byte TYPE_RESPONSE = (byte) 0x80;

// --------- All Gate Message response -------------

/** command completed successfully (with optional parameters)*/

public static final byte RESP_ANY_OK = (byte) 0x00;

/** the destination host is not connected*/

public static final byte RESP_ANY_E_NOT_CONNECTED = (byte) 0x01;

/** command was rejected and/or not completed */

public static final byte RESP_ANY_E_NOK = (byte) 0x03;

/** the registry parameter identifier is either unknown to the registry or an optional registry parameter is not implemented*/

public static final byte RESP_ANY_E_REG_PAR_UNKNOWN = (byte) 0x05;

/** the pipe is not open*/

public static final byte RESP_ANY_E_PIPE_NOT_OPENED = (byte) 0x06;

/** the command is not supported by the gate*/

public static final byte RESP_ANY_E_CMD_NOT_SUPPORTED = (byte) 0x07;

/** command is inhibited due to failure of lower layer identity check*/

public static final byte RESP_ANY_E_INHIBITED = (byte) 0x08;

/** permission denied to write/read a value to/from a registry */

public static final byte RESP_ANY_E_REG_ACCESS_DENIED = (byte) 0x0A;

[…]

/**

* Returns the type of the incoming HCI message

*

* <P>

* In the case of a fragmented incoming message this method shall return

* the HCI message type coded in the first part of the HCI message.

*

* @return the HCI message type, it shall be one of constant values

*         <code>TYPE_*</code> defined in this interface

*/

public byte getType();

/**

* Returns the instruction part of the HCP message header of the current

* incoming HCP message, see TS 102 622 for further information.

* The RESP_ANY_* constants defined in this interface may be used to check

* the response which is contained in the instruction part.

*

* <P>

* In the case of a fragmented incoming message this method shall return

* the HCI message instruction coded in the first part of the HCI message.

*

* @return the  instruction in the HCP message header, b8,b7 of the return value are set to zero.

*/

public byte getInstruction();

[…]

}