| ETSI TC SCP #66 Beijing, China, 4-5 Dec 2014 |  |  |  |  |  |  |  |  |  |  |  |  |  | SCP(14)000288 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ETSI TC SCP TEC #55 Sophia-Antipolis, France, 27-31 Oct 2014 |  |  |  |  |  |  |  |  |  |  |  |  |  | SCPTEC(14)000189r1 |  |  |  |  |  |  |  |
| TC SCP CR-Form-v1.8.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 102 705 |  |  |  | CR | 046 |  |  |  rev |  |  |  |  Current version: |  |  |  |  | 12.0.0 |  |  |  |
| For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects:  |  |  |  |  | smart card |  | X | terminal |  |  |  |  | server / network entity |  |  |  |  |  |  |  |  |
| Title:  |  | Correction of description of error handling |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source:  |  | SCP TEC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item name:  |  | TEI9 |  |  |  |  |  |  |  |  |  |  |  | Date:  |  | 29/10/2014 |  |  |  |  |  |
| Category:  |  | A |  |  |  |  |  |  |  |  |  |  |  | Release:  |  | REL-12 |  |  |  |  |  |
|  |  | Use one of the following categories: F (essential correction) A (corresponds to a correction in an earlier release) B (Addition of feature), C (Functional modification of feature) D (Editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: REL-6 (Release 6) REL-7 (Release 7) REL-8 (Release 8) REL-9 (Release 9) REL-10 (Release 10) REL-11 (Release 11) REL-12(Release 12) |  |  |  |  |  |  |
| TC SCP use of the status "Accepted" for change requests: - At the TC SCP working group level, the status "Accepted" means that a change request is accepted for presentation to TC SCP Plenary. - At the TC SCP Plenary level, the status "Accepted" means that a change request is accepted for implementation in the targeted specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reason for change:  |  |  | The description of the error handling suggest that HCI transmission errors are propagated to the application layer. Also it is currently not clear in which case RF errors are forwarded to the application layer. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change:  |  |  | Stating that HCI communication is reliable therefore errors are not reported to the application layer, and deprecate the corresponding events in the API. Define the behaviour of the contactless runtime environment in case of RF errors for CardEmulation Mode and ReaderMode. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if  not accepted: |  |  | Interoperability issues and not possible to develop test for the related functionality. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| New tag value defined within the CR? |  |  |  | If ticked, add document number of related CR to TS 101 220: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected:  |  |  | 4.1,4.2,4.3,uicc.hci.framework.HCIListener |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs  |  |  |  | Other core specifications  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Affected: |  |  |  | Test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## 4.1	Architecture

The present document describes an API and a Contactless Framework that enables Java CardTM Platform based Applets, defined in [Error: Reference source not found], [Error: Reference source not found] and [Error: Reference source not found], to send and receive messages using the HCI protocol as specified in TS 102 622 [Error: Reference source not found] and to act as contactless Applets. The Contactless Framework shall support card emulation mode and reader mode as specified in the HCI protocol (TS 102 622 [Error: Reference source not found]) specification.

Figure 1

The functionality of the Contactless Framework and the configuration of contactless parameters and the management of contactless Applets in card emulation mode are based on the functionality provided by the Contactless Registry Service (CRS), the related APIs, the CRS Application and other features and concepts which are defined in the "GlobalPlatform Amendment C" [Error: Reference source not found] and the related APIs "Java Card API and Export File for Card Specification v2.2.1 (org.globalplatform) v1.5" [Error: Reference source not found] and "Java Card Contactless API and Export File for Card Specification v2.2.1 (org.globalplatform.contactless)" [Error: Reference source not found].

The API is event driven and based on the Observer/Listener pattern. Every HCI service is encapsulated by a dedicated Service interface. These Service interfaces shall allow the registration of Listener Interfaces and the activation of events. The Listener Interfaces shall be implemented by Java objects to receive HCI messages and events in the onCallback method. The Registration of Listener Interfaces and activation of events shall be persistent.

An HCIMessage object shall encapsulates one HCP message according to the HCI protocol as specified in TS 102 622 [Error: Reference source not found]. HCI message for the different contactless modes shall be identified by different types of interfaces. It is not guaranteed that any Applet originated HCI messages are sent before the completion of the execution of the current Applet. The Contactless Framework sends the Applet originated HCI messages in the same order as they are submitted by the Applet.

NOTE 1:	The Contactless Framework may not have enough resources to send several HCI messages submitted during the same onCallback method execution. The Applet should be aware of this limitation (e.g. use suitable error handling strategy, or send only one HCI message in the onCallback method at a time).

Any onCallback() method of a Listener interface shall not be invoked again while another onCallback() method is still being executed. The Contactless Framework shall be able to receive one or more HCI messages while waiting for a response related to a command originated by the Applet (e.g. processing a request for parameters) especially for the EVT_FIELD_OFF case.

The HCI event EVT_FIELD_OFF shall be buffered and sent by the Contactless Framework as soon as the Contactless Framework becomes the current context.

All other HCI messages shall be delivered to the Applet instance in the same order as they were received by the Contactless Framework.

Contactless State is the logical state of the Contactless Framework it can take the value enabled and disabled. It refers to the "contactless functionality in the UICC" as used in TS 102 223 [Error: Reference source not found]

This state can be changed with the mechanisms defined in TS 102 223 [Error: Reference source not found], and by the method setCommunicationInterface() API method of "GlobalPlatform Amendment C" [Error: Reference source not found].

The Contactless State applies only to the Card Emulation Mode and the Reader Mode, and it does not apply to the Connectivity service.

When the Contactless State is disabled, the Contactless Framework shall throw an HCIException with reason code HCI_CURRENTLY_DISABLED when an Applet invokes a method which requires that the Contactless State is enabled.

When the Contactless State is enabled and the state of the SWP [Error: Reference source not found] interface is DEACTIVATED and when the Contactless Framework needs to send data over the SWP [Error: Reference source not found] interface then it shall send the proactive command ACTIVATE defined in TS 102 223 [Error: Reference source not found] if supported by the terminal.

NOTE 2: 	An Applet may use the method HCIDevice.isHCIServiceAvailable() to check if the Contactless Framework supports sending the ACTIVATE command on pre Rel-11 implementations.

The underlying HCI communication layer as defined in TS 102 622[4] provides reliable message transfer. Therefore no errors can be reported to the application layer. For this reason no error reporting and recovery mechanism related to HCI communication are defined in this specification.

The API is split into two parts. One is a generic framework that provides a factory class to retrieve the different Service instances that are provided by the HCI implementation, and that allows discovery of whether the UICC is inserted into a HCI network. The second part of the API implements the Services that are defined for the HCI protocol, card emulation mode, reader mode and connectivity service.

4.2	Card Emulation Mode

In card emulation mode there exist two exclusive ways to exchange messages over the HCP [Error: Reference source not found]. The first is based on APDUs provided to the Applet through its process() method as specified in "Application Programming Interface, Java Card™ Platform, 3.0 Classic Edition" [Error: Reference source not found]. The second is made available by the package uicc.hci.services.cardemulation defined in this specification.

The uicc.hci.services.cardemulation package shall provide the communication technologies for the card emulation mode defined by the HCP as specified in [Error: Reference source not found]. The Contactless Framework shall bind the services defined in the uicc.hci.services.cardemulation package to the underlying HCI resources (e.g. gates and pipes) defined in the HCI architecture as specified in [Error: Reference source not found]. The parameters to be used by the HCI layer may be provided to the framework as defined in "GlobalPlatform Amendment C" [Error: Reference source not found].

In case of a communication error on the RF interface (i.e. the RF error indicator is set), messages are not propagated to the application layer in CardEmulation Mode.

For the API defined in this specification the card emulation capability shall be provided to Applets through a service interface implemented by the Contactless Framework. Applet instances shall receive CardEmulationMessages after the registration of a CardEmulationListener interface to a CardEmulationService only if the EVENT_ON_SEND_DATA is activated for the Applet instance. If the EVENT_ON_SEND_DATA is deactivated for the Applet instance and an APDU is received via the EVT_SEND_DATA, the javacard.framework.APDU class and the process() method of the Applet instance shall be invoked.

It shall not be possible to switch between the usage of the CardEmulationListener interface and the invocation through the process() method within a contactless application session, i.e. not before the Applet has been deselected and selected again. Applets communicating through the process() method shall also be able to use the API services defined in this specification which do not require a CardEmulationListener registration (e.g. requesting the power mode or connectivity service).

If the current application was selected through a SELECT by DF name, the Contactless Framework shall handle an application session termination according to TS 102 221 [Error: Reference source not found] independent of the interface used for APDU exchange.

Applet selection and deselection shall be performed by the Contactless Framework according to the rules defined in the "Java Card™ Runtime Environment Specification, 3.0.1 Classic Edition" [Error: Reference source not found] and in "GlobalPlatform Amendment C" [Error: Reference source not found].

The select() method of the Applet instance shall always be invoked for an Applet selection according to the rules given in "Java Card™ Runtime Environment Specification, 3.0.1 Classic Edition" [Error: Reference source not found].

In case the Applet instance has registered the CardEmulationListener and has activated the EVENT_ON_SEND_DATA the process() method of this Applet instance shall not be invoked during the selection. The CardEmulationListener.onCallback method shall be called by the Contactless Framework. The HCP message that resulted in the selection of this Applet according to the rules defined in "GlobalPlatform Amendment C" [Error: Reference source not found] shall be provided by the CardEmulationMessage.

If the HCI event EVT_FIELD_OFF or EVT_CARD_DEACTIVATED defined by the HCP specified in TS 102 622 [Error: Reference source not found] is received by the Contactless Framework and the UICC is still powered, the Applet instance shall be deselected according to "GlobalPlatform Amendment C" [Error: Reference source not found] by invocation of the deselect() method.

When the HCI event EVT_FIELD_OFF is received and if the Applet instance has activated this event the Contactless Framework shall raise an EVENT_FIELD_OFF before the invocation of the deselect() method of the Applet instance.

After the deselection of the Applet instance, it shall not be invoked by any other event defined in the interface CardEmulationListener until the Applet instance is selected again.

## 4.3	Reader Mode

The functionality to support the reader mode is provided in the package uicc.hci.services.reader. In reader mode the communication technologies defined by the contactless platform for reader mode [Error: Reference source not found] are supported. The Contactless Framework shall bind the services defined in uicc.hci.services.reader to the corresponding resources (e.g. gates and pipes) defined by the contactless platform for reader mode [Error: Reference source not found].

In case of a communication error on the RF interface (i.e. the RF error indicator is set), messages are propagated to the application layer in Reader Mode.

An Applet has to be in the selectable state (according to the Java Card™ specification [Error: Reference source not found], [Error: Reference source not found] and [Error: Reference source not found]) to act as a contactless Applet in reader mode.

Reader mode Applets shall follow the extended lifecycle model that is defined in "GlobalPlatform Amendment C" [Error: Reference source not found] for contactless Applets in card emulation mode (i.e. following Application Availability States and the related transition rules).

Per RF technology there shall be only one reader mode Applet in the state ACTIVATED (according to "GlobalPlatform Amendment C" [Error: Reference source not found]) at any time.

The installation parameters for contactless reader mode applications are specified in TS 102 226 [Error: Reference source not found].

When the state of a reader mode Applet changes to lifecycle ACTIVATED (according to "GlobalPlatform Amendment C" [Error: Reference source not found]) the Contactless Framework shall ensure that the HCI gates and pipes are setup for the RF technologies that are supported by the reader mode Applet.

To be able to receive and send messages over the contactless interface in reader mode the Applet shall activate the ReaderListener.EVENT_TARGET_DISCOVERED. An Applet shall only be able to activate this event or to use the restartReadermodeProcedure method if it is in lifecycle state ACTIVATED. To release the CLF control at the end of a transaction an Applet shall deactivate the ReaderListener.EVENT_TARGET_DISCOVERED.

When an Applet lifecycle state changes from ACTIVATED to DEACTIVATED the Contactless Framework shall enforce that the ReaderListener.EVENT_TARGET_DISCOVERED is deactivated.

The Contactless Framework shall request the reader mode control on the CLF by sending the HCI events EVT_READER_REQUESTED and EVT_END_OPERATION according to the state of the reader mode Applet. The EVT_READER_REQUESTED shall be sent by the Contactless Framework if an Applet instance activates the event ReaderListener.EVENT_TARGET_DISCOVERED and no other Applet instance has the event activated, i.e. it shall not be sent if the Contactless Framework has earlier sent an EVT_READER_REQUESTED due to the request from another Applet instance, which was not yet ended by an EVT_END_OPERATION. The HCI event EVT_END_OPERATION shall be sent to the CLF when an Applet instance or the Contactless Framework deactivates the event ReaderListener.EVENT_TARGET_DISCOVERED. The Contactless Framework shall resend the EVT_READER_REQUESTED to the CLF if another Applet instance exists with the ReaderListener.EVENT_TARGET_DISCOVERED event activated.

The Contactless Framework shall inform the Applet instance which has activated the ReaderListener.EVENT_TARGET_DISCOVERED when a target is discovered on one of the RF technologies the Applet instance is registered to with its installation parameters as specified in TS 102 226 [Error: Reference source not found].

The Contactless Framework shall ensure that the ReaderListener.EVENT_TARGET_DISCOVERED is deactivated for all Applets when access to the interface is disabled on the UICC level.

### ……

package uicc.hci.framework;

import javacard.framework.Shareable;

/** The base interface for all types of HCI listeners.

* <P>

* An Object shall not implement this interface but only implement one of the sub-interfaces.

*

* <P>

* An Applet instance implementing a <code>*Listener</code> interface shall only

* receive events defined in the same <code>*Listener</code> interface or in the

* <code>HCIListner</code> interface.

* <p>

* If a Class is implementing more then one <code>HCIListener</code> sub-interfaces

* the Applet should check the type of the <code>HCIMesage</code> by using

* <code>instanceof</code> in order to identify the source of the notification.

* </p>

* <P>

* Messages received through the inherited method <code>HCIListener.onCallback</code>

* shall be of the message type defined in the same package as the <code>*Listener</code> interface

* <P>

*

* @see uicc.hci.services.cardemulation.CardEmulationListener

* @see uicc.hci.services.connectivity.ConnectivityListener

* @see uicc.hci.services.readermode.ReaderListener

**/

public interface HCIListener extends Shareable {

/**

* @deprecated

* This value is used to notify the Applet when the UICC failed to transmit a message to the CLF.

*

* The Applet has to activate this event (for a specific service) if it wants to be notified

* about data transmission errors (that occur for a specific service):

*/

public final static byte EVENT_HCI_TRANSMISSION_FAILED = (byte) 0x81;

/**

* @deprecated

* This value is used to notify the Applet when the UICC failed to receive a message sent by the CLF.

*

* The Applet has to activate this event (for a specific service) if it wants to be notified

* about data transmission errors (that occur for a specific service):

*/

public final static byte EVENT_HCI_RECEPTION_FAILED = (byte) 0x82;

/** This method is called by the HCI framework to inform the Listener

*  Object about a specific event and pass the corresponding <code>HCIMessage</code> to

*  the Listener Object.

*

* In the case of an <code>EVENT_HCI_TRANSMISSION_FAILED</code> the message

* which could not be transmitted shall be made available in the <code>HCIMessage</code> object.

*

* If an Applet has requested the framework to send more than one message during a single

* <code>onCallback</code> method execution and a transmission to the CLF failed the framework

* shall not attempt to send any of the remaining messages.

*

* The <code>EVENT_HCI_RECEPTION_FAILED</code> shall not be raised in the case of a

* failure to receive a command from an external reader in CardEmulation Mode due to a problem

* on the data link layer

*

* @param  event that triggers the invocation of the method

* @param  message the HCIMessage

*/

public void onCallback(byte event, HCIMessage message);

}