| ETSI TC SCP Meeting #66 Beijing, China, 4-5 Dec 2014 |  |  |  |  |  |  |  |  |  |  |  |  |  | SCP(14)000348 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ETSI TC SCP TEC #55 Sophia-Antipolis, France, 27 - 31 Oct 2014 |  |  |  |  |  |  |  |  |  |  |  |  |  | Tdoc SCPTEC(14)000190r1 |  |  |  |  |  |  |  |
| TC SCP CR-Form-v1.8.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 102 705 |  |  |  | CR | 048 |  |  |  rev |  |  |  |  Current version: |  |  |  |  | 12.0.0 |  |  |  |
| For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects:  |  |  |  |  | smart card |  | X | terminal |  |  |  |  | server / network entity |  |  |  |  |  |  |  |  |
| Title:  |  | Optional support for uicc.hci.services.readermode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source:  |  | SCP TEC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item name:  |  | TEI12 |  |  |  |  |  |  |  |  |  |  |  | Date:  |  | 29/10/2014 |  |  |  |  |  |
| Category:  |  | C |  |  |  |  |  |  |  |  |  |  |  | Release:  |  | 12 |  |  |  |  |  |
|  |  | Use one of the following categories: F (essential correction) A (corresponds to a correction in an earlier release) B (Addition of feature), C (Functional modification of feature) D (Editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: REL-6 (Release 6) REL-7 (Release 7) REL-8 (Release 8) REL-9 (Release 9) REL-10 (Release 10) REL-11 (Release 11) REL-12 (Release 12) |  |  |  |  |  |  |
| TC SCP use of the status "Accepted" for change requests: - At the TC SCP working group level, the status "Accepted" means that a change request is accepted for presentation to TC SCP Plenary. - At the TC SCP Plenary level, the status "Accepted" means that a change request is accepted for implementation in the targeted specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reason for change:  |  |  | Reader mode is not widely supported therefore the support for it in the API can be made optional. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change:  |  |  | Make the support of the package uicc.hci.services.readermode optional. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if  not accepted: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| New tag value defined within the CR? |  |  |  | If ticked, add document number of related CR to TS 101 220: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected:  |  |  | 4.1, package.html of the uicc.hci.services.readermode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs  |  |  |  | Other core specifications  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Affected: |  |  |  | Test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## 4.1	Architecture

The present document describes an API and a Contactless Framework that enables Java CardTM Platform based Applets, defined in [Error: Reference source not found], [Error: Reference source not found] and [Error: Reference source not found], to send and receive messages using the HCI protocol as specified in TS 102 622 [Error: Reference source not found] and to act as contactless Applets. The Contactless Framework shall support card emulation mode and reader mode as specified in the HCI protocol (TS 102 622 [Error: Reference source not found]) specification.

Figure 1

The functionality of the Contactless Framework and the configuration of contactless parameters and the management of contactless Applets in card emulation mode are based on the functionality provided by the Contactless Registry Service (CRS), the related APIs, the CRS Application and other features and concepts which are defined in the "GlobalPlatform Amendment C" [Error: Reference source not found] and the related APIs "Java Card API and Export File for Card Specification v2.2.1 (org.globalplatform) v1.5" [Error: Reference source not found] and "Java Card Contactless API and Export File for Card Specification v2.2.1 (org.globalplatform.contactless)" [Error: Reference source not found].

The API is event driven and based on the Observer/Listener pattern. Every HCI service is encapsulated by a dedicated Service interface. These Service interfaces shall allow the registration of Listener Interfaces and the activation of events. The Listener Interfaces shall be implemented by Java objects to receive HCI messages and events in the onCallback method. The Registration of Listener Interfaces and activation of events shall be persistent.

An HCIMessage object shall encapsulates one HCP message according to the HCI protocol as specified in TS 102 622 [Error: Reference source not found]. HCI message for the different contactless modes shall be identified by different types of interfaces. It is not guaranteed that any Applet originated HCI messages are sent before the completion of the execution of the current Applet. The Contactless Framework sends the Applet originated HCI messages in the same order as they are submitted by the Applet.

NOTE 1:	The Contactless Framework may not have enough resources to send several HCI messages submitted during the same onCallback method execution. The Applet should be aware of this limitation (e.g. use suitable error handling strategy, or send only one HCI message in the onCallback method at a time).

Any onCallback() method of a Listener interface shall not be invoked again while another onCallback() method is still being executed. The Contactless Framework shall be able to receive one or more HCI messages while waiting for a response related to a command originated by the Applet (e.g. processing a request for parameters) especially for the EVT_FIELD_OFF case.

The HCI event EVT_FIELD_OFF shall be buffered and sent by the Contactless Framework as soon as the Contactless Framework becomes the current context.

All other HCI messages shall be delivered to the Applet instance in the same order as they were received by the Contactless Framework.

Contactless State is the logical state of the Contactless Framework it can take the value enabled and disabled. It refers to the "contactless functionality in the UICC" as used in TS 102 223 [Error: Reference source not found]

This state can be changed with the mechanisms defined in TS 102 223 [Error: Reference source not found], and by the method setCommunicationInterface() API method of "GlobalPlatform Amendment C" [Error: Reference source not found].

The Contactless State applies only to the Card Emulation Mode and the Reader Mode, and it does not apply to the Connectivity service.

When the Contactless State is disabled, the Contactless Framework shall throw an HCIException with reason code HCI_CURRENTLY_DISABLED when an Applet invokes a method which requires that the Contactless State is enabled.

When the Contactless State is enabled and the state of the SWP [Error: Reference source not found] interface is DEACTIVATED and when the Contactless Framework needs to send data over the SWP [Error: Reference source not found] interface then it shall send the proactive command ACTIVATE defined in TS 102 223 [Error: Reference source not found] if supported by the terminal.

NOTE 2: 	An Applet may use the method HCIDevice.isHCIServiceAvailable() to check if the Contactless Framework supports sending the ACTIVATE command on pre Rel-11 implementations.

The API is split into two parts. One is a generic framework that provides a factory class to retrieve the different Service instances that are provided by the HCI implementation, and that allows discovery of whether the UICC is inserted into a HCI network. The second part of the API implements the Services that are defined for the HCI protocol, card emulation mode, reader mode and connectivity service. The support of the package implementing reader mode, uicc.hci.services.readermode, is optional.

As stated in TS 102 622 [Error: Reference source not found], a reset of the underlying data link layer shall be transparent to the application layer if the data link layer recovers without any loss of data. In the case of a data loss an error shall be reported back to the Application which has initiated the data transmission. In case of a communication in CardEmulation Mode where a command from an external reader cannot be forwarded to an Application due to a data link layer problem the Contactless Framework shall not take any action but wait for the reader to resolve the problem.

…..

<!DOCTYPE HTML PUBLIC "-//W3C//DTD HTML 4.01 Transitional//EN">

<html>

<head>

<title></title>

<meta http-equiv="Content-Type" content="text/html; charset=UTF-8">

</head>

<body>

This optional package defines the interfaces for the reader mode of the HCI protocol.

</body>

</html>