| ETSI TC SCP Meeting #54 Sophia Antipolis, France, 29 - 30 March 2012 |  |  |  |  |  |  |  |  |  |  |  |  |  | SCP(12)000018 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ETSI TC SCP TEC Meeting #42 Sophia Antipolis, France, 20 - 24 February 2012 |  |  |  |  |  |  |  |  |  |  |  |  |  | SCPTEC(11)0181r2 |  |  |  |  |  |  |  |
| TC SCP CR-Form-v1.8.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 102 705 |  |  |  | CR | 027 |  |  |  rev |  |  |  |  Current version: |  |  |  |  | 10.0.0 |  |  |  |
| For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects:  |  |  |  |  | smart card |  | X | terminal |  |  |  |  | server / network entity |  |  |  | X |  |  |  |  |
| Title:  |  | Update of references to GlobalPlatform specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source:  |  | SCP TEC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item name:  |  | TEI10 |  |  |  |  |  |  |  |  |  |  |  | Date:  |  | 23/02/2012 |  |  |  |  |  |
| Category:  |  | F |  |  |  |  |  |  |  |  |  |  |  | Release:  |  | Rel-10 |  |  |  |  |  |
|  |  | Use one of the following categories: F (essential correction) A (corresponds to a correction in an earlier release) B (Addition of feature), C (Functional modification of feature) D (Editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: REL-6 (Release 6) REL-7 (Release 7) REL-8 (Release 8) REL-9 (Release 9) REL-10 (Release 10) REL-11 (Release 11) |  |  |  |  |  |  |
| TC SCP use of the status "Accepted" for change requests: - At the TC SCP working group level, the status "Accepted" means that a change request is accepted for presentation to TC SCP Plenary. - At the TC SCP Plenary level, the status "Accepted" means that a change request is accepted for implementation in the targeted specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reason for change:  |  |  | New versions of several GlobalPlatform specifications which are referenced in TS 102 705 were published. Those new versions include corrections that should be taken into account in TS 102 705 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change:  |  |  | The references to the following GlobalSpecifications are updated: - GlobalPlatform: "GlobalPlatform Card Specification Version 2.2, Amendment C: Contactless Services" Version 1.0. new version V1.0.1 February 2012 New references to GlobalPlatform APIs: - Java Card API and Export File for Card Specification v2.2.1 (org.globalplatform) v1.5 - Java Card Contactless API and Export File for Card Specification v2.2.1 (org.globalplatform.contactless) v1.1 February 2012 New text is inserted in clause 4.1 which refers to the aforementioned GlobalPlatform specifications because the discussion in SCP TEC showed that many delegates have the feeling that a normative reference to an external specification in clause 1 “Scope” is not sufficient. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if  not accepted: |  |  | The reader of TS 102 226 may not be aware of clarifications and corrections which were made in GlobalPlatform specifications. This may potentially result in interoperability problems and erroneous implementations. The card may not pass the compliance test suite of GlobalPlatform for the UICC Configuration. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| New tag value defined within the CR? |  |  |  | If ticked, add document number of related CR to TS 101 220: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected:  |  |  | 1, 2.1, 4.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs  |  |  |  | Other core specifications  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Affected: |  |  |  | Test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

# 1	Scope

The present document describes the UICC Application Programming Interface for Java Card™ for contactless Applications. Its purpose is to provide access for a contactless Applet to the services provided by the HCI protocol defined in [4] for the communication via the CLF. In the scope of this document contactless means support for the RF Technologies referenced by the HCI specification [4]. Low level functionality to manage gates and pipes as defined in the HCI specification [4] is not in the scope of this document. Registration of contactless parameters and management of contactless Applets in card emulation mode is defined in " GlobalPlatform Card Specification Amendmentt C" [8] . Related APIs are provided in " Java Card API and Export File for Card Specification v2.2.1 (org.globalplatform) v1.5" [X1] and " Java Card Contactless API and Export File for Card Specification v2.2.1 (org.globalplatform.contactless)" [X2]

## 2.1	Normative references

The following referenced documents are necessary for the application of the present document.

[1]	ISO/IEC 7816-3:2006: "Identification cards - Integrated circuit cards - Part 3: Cards with contacts - Electrical interface and transmission protocols".

[2]	ETSI TS 102 221: "Smart Cards; UICC-Terminal interface; Physical and logical characteristics".

[3]	ETSI TS 101 220: "Smart Cards; ETSI numbering system for telecommunication application providers".

[4]	ETSI TS 102 622: "Smart Cards; UICC - Contactless Front-end (CLF) Interface; Host Controller Interface (HCI)".

[5]	ETSI TS 102 241: "Smart Cards; UICC Application Programming Interface (UICC API) for Java Card (TM)".

[6]	ETSI TS 102 223: "Smart Cards; Card Application Toolkit (CAT)".

[7]	ETSI TS 102 226: "Smart Cards; Remote APDU structure for UICC based applications".

[8]	GlobalPlatform: "GlobalPlatform Card Specification Version 2.2, Amendment C: Contactless Services" Version 1.0.1

NOTE:	See http://www.globalplatform.org/.

[9]	Sun Microsystems "Application Programming Interface, Java Card™ Platform, 3.0.1 Classic Edition".

[10]	Sun Microsystems "Runtime Environment Specification, Java Card™ Platform, 3.0.1 Classic Edition".

[11]	Sun Microsystems "Virtual Machine Specification Java Card™ Platform, 3.0.1 Classic Edition".

NOTE:	SUN Java Card Specifications can be downloaded at http://java.sun.com/products/javacard/specs.html.

[X1]	Java Card API and Export File for Card Specification v2.2.1 (org.globalplatform) v1.5

[X2]	Java Card Contactless API and Export File for Card Specification v2.2.1 (org.globalplatform.contactless) v1.1

## 4.1	Architecture

The present document describes an API and a Contactless Framework that enables Java CardTM Platform based Applets, defined in [9], [10] and [11], to send and receive messages using the HCI protocol as specified in TS 102 622 [4] and to act as contactless Applets. The Contactless Framework shall support card emulation mode and reader mode as specified in the HCI protocol (TS 102 622 [4]) specification.

Figure 1

The functionality of the Contactless Framework and the configuration of contactless parameters and the management of contactless Applets in card emulation mode are based on the functionality provided by the Contactless Registry Service (CRS), the related APIs, the CRS Application and other features and concepts which are defined in the "GlobalPlatform Amendment C" [8] and the related APIs " Java Card API and Export File for Card Specification v2.2.1 (org.globalplatform) v1.5" [X1] and " Java Card Contactless API and Export File for Card Specification v2.2.1 (org.globalplatform.contactless)" [X2].

The API is event driven and based on the Observer/Listener pattern. Every HCI service is encapsulated by a dedicated Service interface. These Service interfaces shall allow the registration of Listener Interfaces and the activation of events. The Listener Interfaces shall be implemented by Java objects to receive HCI messages and events in the onCallback method. The Registration of Listener Interfaces and activation of events shall be persistent.

An HCIMessage object shall encapsulates one HCP message according to the HCI protocol as specified in TS 102 622 [4]. HCI message for the different contactless modes shall be identified by different types of interfaces. It is not guaranteed that any Applet originated HCI messages are sent before the completion of the execution of the current Applet.

Any onCallback() method of a Listener interface shall not be invoked again while another onCallback() method is still being executed. The Contactless Framework shall be able to receive one or more HCI messages while waiting for a response related to a command originated by the Applet (e.g. processing a request for parameters) especially for the EVT_FIELD_OFF case.

The HCI event EVT_FIELD_OFF shall be buffered and sent by the Contactless Framework as soon as the Contactless Framework becomes the current context.

All other HCI messages shall be delivered to the Applet instance in the same order as they were received by the Contactless Framework.

When the contactless interface is disabled (cf. "state of contactless functionality" in TS 102 223 [6] and setCommunicationInterface() API method of "Java Card Contactless API and Export File for Card Specification v2.2.1 (org.globalplatform.contactless)" [X1]), the Contactless Framework shall throw an HCIException with reason code HCI_CURRENTLY_DISABLED.

The API is split into two parts. One is a generic framework that provides a factory class to retrieve the different Service instances that are provided by the HCI implementation, and that allows discovery of whether the UICC is inserted into a HCI network. The second part of the API implements the Services that are defined for the HCI protocol, card emulation mode, reader mode and connectivity service.

As stated in TS 102 622 [4], a reset of the underlying data link layer shall be transparent to the application layer if the data link layer recovers without any loss of data. In the case of a data loss an error shall be reported back to the Application which has initiated the data transmission. In case of a communication in CardEmulation Mode where a command from an external reader can not be forwarded to an Application due to a data link layer problem the Contactless Framework shall not take any action but wait for the reader to resolve the problem.