| ETSI TC SCP #55 Osaka, Japan, 30 May - 2 June 2012 |  |  |  |  |  |  |  |  |  |  | Tdoc SCP(12)000102 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ETSI TC SCP TEC #43 Sophia-Antipolis, France, 23 - 27 April 2012 |  |  |  |  |  |  |  |  |  |  | Tdoc SCPTEC(12)000045r3 |  |  |  |  |  |  |  |  |  |  |
| TC SCP CR-Form-v1.8.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 102 705 |  |  |  | CR | 029 |  |  |  rev |  |  |  |  |  Current version: |  |  |  | 11.0.0 |  |  |  |
| For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects:  |  |  |  |  | smart card |  | X | terminal |  |  |  |  |  | server / network entity |  |  |  |  |  |  |  |
| Title:  |  | Clarifications related to the use of the proactive command ACTIVATE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source:  |  | SCP TEC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item name:  |  | TEI11 |  |  |  |  |  |  |  |  |  |  | Date:  |  |  | 26/04/2012 |  |  |  |  |  |
| Category:  |  | C |  |  |  |  |  |  |  |  | Release:  |  |  |  |  | Rel-11 |  |  |  |  |  |
|  |  | Use one of the following categories: F (essential correction) A (corresponds to a correction in an earlier release) B (Addition of feature), C (Functional modification of feature) D (Editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: REL-6 (Release 6) REL-7 (Release 7) REL-8 (Release 8) REL-9 (Release 9) REL-10 (Release 10) REL-11 (Release 11) |  |  |  |  |  |  |
| TC SCP use of the status "Accepted" for change requests: - At the TC SCP working group level, the status "Accepted" means that a change request is accepted for presentation to TC SCP Plenary. - At the TC SCP Plenary level, the status "Accepted" means that a change request is accepted for implementation in the targeted specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reason for change:  |  |  | The proactive commands ACTIVATE was introduced in TS 102 223 with CR 161 (SCP-070422/ SCPt070880) in 2007. By means of this proactive command the UICC is able to instruct the terminal to activate the SWP interface. Although the proactive command ACTIVATE is defined on the interface level it is not described in any SCP specification which entity in the UICC is responsible to issue this command. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change:  |  |  | This CR clarifies that the Contactless Framework is responsible to send the command to the terminal if the SWP interface needs to be activated. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if  not accepted: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| New tag value defined within the CR? |  |  |  | If ticked, add document number of related CR to TS 101 220: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected:  |  |  | 2.1, 3.1, 3.2, 4.1, Annex A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs  |  |  |  | Other core specifications  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Affected: |  |  |  | Test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## 2.1	Normative references

The following referenced documents are necessary for the application of the present document.

[1]	ISO/IEC 7816-3:2006: "Identification cards - Integrated circuit cards - Part 3: Cards with contacts - Electrical interface and transmission protocols".

[2]	ETSI TS 102 221: "Smart Cards; UICC-Terminal interface; Physical and logical characteristics".

[3]	ETSI TS 101 220: "Smart Cards; ETSI numbering system for telecommunication application providers".

[4]	ETSI TS 102 622: "Smart Cards; UICC - Contactless Front-end (CLF) Interface; Host Controller Interface (HCI)".

[5]	ETSI TS 102 241: "Smart Cards; UICC Application Programming Interface (UICC API) for Java Card (TM)".

[6]	ETSI TS 102 223: "Smart Cards; Card Application Toolkit (CAT)".

[7]	ETSI TS 102 226: "Smart Cards; Remote APDU structure for UICC based applications".

[8]	GlobalPlatform: "GlobalPlatform Card Specification Version 2.2, Amendment C: Contactless Services" Version 1.0.

NOTE:	See http://www.globalplatform.org/.

[9]	Sun Microsystems "Application Programming Interface, Java Card™ Platform, 3.0.1 Classic Edition".

[10]	Sun Microsystems "Runtime Environment Specification, Java Card™ Platform, 3.0.1 Classic Edition".

[11]	Sun Microsystems "Virtual Machine Specification Java Card™ Platform, 3.0.1 Classic Edition".

NOTE:	SUN Java Card Specifications can be downloaded at http://java.sun.com/products/javacard/specs.html.

[12]			ETSI TS 102 613: "Smart Cards; UICC – Contactless Front-end (CLF) Interface, Part 1: Physical and data link layer characteristics"

….

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the following terms and definitions apply:

contactless mode: is used as a generic term for "Card Emulation Mode" and "Reader Mode"

Contactless State: corresponds to the logical state of the contactless framework.

HCP message: message as specified in TS 102 622 [4]

NOTE:	An HCP message can be of type "command", "event" or "response to a command"

RF Technology: radio frequency technology supported by the HCI (TS 102 622 [4]) protocol specification

## 3.2	Abbreviations

For the purposes of the present document, the following abbreviations apply:

APDU	Application Protocol Data Unit

NOTE:	According to ISO/IEC 7816-3 [1].

API	Application Programming Interface

CLF	Contactless Front-end

NOTE:	According to TS 102 622 [4].

HCI	Host Controller Interface

NOTE:	According to TS 102 622 [4].

HCP	Host Controller Protocol

NOTE:	According to TS 102 622 [4].

RF	Radio Frequency

SWP	Single Wire Protocol

NOTE:	According to TS 102 613[13]

…..

## 4.1	Architecture

The present document describes an API and a Contactless Framework that enables Java CardTM Platform based Applets, defined in [9], [10] and [11], to send and receive messages using the HCI protocol as specified in TS 102 622 [4] and to act as contactless Applets. The Contactless Framework shall support card emulation mode and reader mode as specified in the HCI protocol (TS 102 622 [4]) specification.

Figure 1

The API is event driven and based on the Observer/Listener pattern. Every HCI service is encapsulated by a dedicated Service interface. These Service interfaces shall allow the registration of Listener Interfaces and the activation of events. The Listener Interfaces shall be implemented by Java objects to receive HCI messages and events in the onCallback method. The Registration of Listener Interfaces and activation of events shall be persistent.

An HCIMessage object shall encapsulates one HCP message according to the HCI protocol as specified in TS 102 622 [4]. HCI message for the different contactless modes shall be identified by different types of interfaces. It is not guaranteed that any Applet originated HCI messages are sent before the completion of the execution of the current Applet. The Contactless Framework sends the Applet originated HCI messages in the same order as they are submitted by the Applet.

NOTE:	The Contactless Framework may not have enough resources to send several HCI messages submitted during the same onCallback method execution. The Applet should be aware of this limitation (e.g. use suitable error handling strategy, or send only one HCI message in the onCallback method at a time).

Any onCallback() method of a Listener interface shall not be invoked again while another onCallback() method is still being executed. The Contactless Framework shall be able to receive one or more HCI messages while waiting for a response related to a command originated by the Applet (e.g. processing a request for parameters) especially for the EVT_FIELD_OFF case.

The HCI event EVT_FIELD_OFF shall be buffered and sent by the Contactless Framework as soon as the Contactless Framework becomes the current context.

All other HCI messages shall be delivered to the Applet instance in the same order as they were received by the Contactless Framework.

Contactless State is the logical state of the Contactless Framework it can take the value enabled and disabled. It refers to the “contactless functionality in the UICC” as used in TS 102 223[6]

This state can be changed with the mechanisms defined in TS 102 223[6], and by the method setCommunicationInterface() API method of "GlobalPlatform Amendment C" [8].

The Contactless State applies only to the Card Emulation Mode and the Reader Mode, and it does not apply to the Connectivity service.

When the Contactless State is disabled the Contactless Framework shall throw an HCIException with reason code HCI_CURRENTLY_DISABLED when an Applet invokes a method which requires that the Contactless State is enabled

When the Contactless State is enabled and the state of the SWP[12] interface is DEACTIVATED and when the Contactless Framework needs to send data over the SWP[12] interface then it shall send the proactive command ACTIVATE defined in TS 102 223[6] if supported by the terminal. An Applet may use the method HCIDevice.isHCIServiceAvailable() to check if the Contactless Framework supports sending the ACTIVATE command.

The API is split into two parts. One is a generic framework that provides a factory class to retrieve the different Service instances that are provided by the HCI implementation, and that allows discovery of whether the UICC is inserted into a HCI network. The second part of the API implements the Services that are defined for the HCI protocol, card emulation mode, reader mode and connectivity service.

As stated in TS 102 622 [4], a reset of the underlying data link layer shall be transparent to the application layer if the data link layer recovers without any loss of data. In the case of a data loss an error shall be reported back to the Application which has initiated the data transmission. In case of a communication in CardEmulation Mode where a command from an external reader can not be forwarded to an Application due to a data link layer problem the Contactless Framework shall not take any action but wait for the reader to resolve the problem.

# Annex A (normative):

# Java Card™ Platform HCI API for the UICC

In file uicc\hci\framework\HCIDevice.java:

/******************************************************************************

*

******************************************************************************/

package uicc.hci.framework;

import javacard.framework.*;

/**

* This class provides a factory to retrieve object instances that implement sub

* interfaces of <code>HCIService</code> and a method to check if certain features

* are supported by the UICC or the terminal.

*

**/

public final class HCIDevice {

/** Identifies the service object for reader mode functionality */

public final static short READER_SERVICE_ID = (short)2;

/** Identifies the service object for card emulation mode functionality */

public static final short CARD_EMULATION_SERVICE_ID = (short)3;

/** Identifies the service object for connectivity functionality */

public final static short CONNECTIVITY_SERVICE_ID = (short)4;

/** Identifies the feature of automated sending auf the proactive command ACTIVATE */

public final static short AUTO_ACTIVATE_SERVICE_ID = (short)5;

/** UICC is activated in low power mode */

public final static byte LOW_POWER_MODE = (byte)0x00;

/** UICC is activated in full power mode */

public final static byte FULL_POWER_MODE = (byte)0x01;

[…]

/**

* This method is used to retrieve information about the availability of a HCIService

* or about the availability of a feature of the contactless framework.

* For the AUTO_ACTIVATE_SEVICE_ID only bit 0,1,4 of the return value apply

* @param serviceID shall be one of the constants (*)_SERVICE_ID defined in this class

* @return a byte with the

bit values set according to the following list:

*              <ul>

*              <li> bit 0 set to 1: interface not supported by terminal

*              <li> bit 1 set to 1: service not supported by terminal

*              <li> bit 2 set to 1: access not allowed for Applet

*              <li> bit 3 set to 1: HCI interface currently disabled in UICC

*              <li> bit 4 set to 1: service not supported in UICC

*              <li> bit 5 set to 0 (rfu)

*              <li> bit 6 set to 0 (rfu)

*              <li> bit 7 set to 0 (rfu)

*              </ul>

* @throws HCIException with reason:

* <ul>

* <li>HCI_SERVICE_NOT_AVAILABLE if the requested service type or feature is not known to the contactless runtime environment

* </ul>

*/

public static byte isHCIServiceAvailable(short serviceID) throws HCIException { return 0x00; }

}