| ETSI TC SCP Meeting #55 Osaka, Japan, 30 May - 1 June 2012 |  |  |  |  |  |  |  |  |  |  |  | Tdoc SCPTEC(12)000103 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ETSI TC SCP TEC Meeting #43 Sophia Antipolis, France, 23 - 27 April 2012 |  |  |  |  |  |  |  |  |  |  |  | Tdoc SCPTEC(12)000061r3 |  |  |  |  |  |  |  |  |  |
| TC SCP CR-Form-v1.8.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 102 705 |  |  |  | CR | 030 |  |  |  rev |  |  |  |  Current version: |  |  |  |  | 11.0.0 |  |  |  |
| For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects:  |  |  |  |  | smart card |  | X | terminal |  |  |  |  | server / network entity |  |  |  |  |  |  |  |  |
| Title:  |  | Define the relationship between the access to the contactless interface and the install parameters |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source:  |  | SCP TEC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item name:  |  | TEI11 |  |  |  |  |  |  |  |  |  |  |  | Date:  |  | 26/04/2012 |  |  |  |  |  |
| Category:  |  | C |  |  |  |  |  |  |  |  |  |  |  | Release:  |  | REL-11 |  |  |  |  |  |
|  |  | Use one of the following categories: F (essential correction) A (corresponds to a correction in an earlier release) B (Addition of feature), C (Functional modification of feature) D (Editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: REL-6 (Release 6) REL-7 (Release 7) REL-8 (Release 8) REL-9 (Release 9) REL-10 (Release 10) REL-11 (Release 11) |  |  |  |  |  |  |
| TC SCP use of the status "Accepted" for change requests: - At the TC SCP working group level, the status "Accepted" means that a change request is accepted for presentation to TC SCP Plenary. - At the TC SCP Plenary level, the status "Accepted" means that a change request is accepted for implementation in the targeted specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reason for change:  |  |  | With installation parameters in GlobalPlatform and TS 102 226 it is possible to restrict the access to the contactless interface for an Applet instance. The relationship between these installation parameters and the indication given by the API has to be defined. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change:  |  |  | Clarification of the description of the methods getHCIService and isHCIServiceAvailable in HCIDevice with respect to the situation when the installation parameters of the Applications indicate that the calling Applet has no access to the contactless interface. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if  not accepted: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| New tag value defined within the CR? |  |  |  | If ticked, add document number of related CR to TS 101 220: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected:  |  |  | HCIDevice.java |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs  |  |  |  | Other core specifications  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Affected: |  |  |  | Test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### uicc.hci.framework.HCIDevice

/**

* This method is used to retrieve an instances that implement one of <code>HCIService</code>

* subinterfaces defined in this specification.

* <P>

* This method returns <code>null</code> if the <code>Applet.register()</code> method has not yet

* been invoked.

* <p>

* If the Applet instance invoking this method passes <code>CARD_EMULATION_SERVICE_ID</code> as parameterand theCommunication Interface Access Configuration of this Applet instance does not allow "Proximity-based communication" according to  clause 5 of GlobalPlatform Card Specification Amendment C [8] a <code>HCIException</code> with reason code <code>HCI_ACCESS_NOT_GRANTED</code> shall be thrown.

*<p>

* If no "Parameters for contactless applications" as defined in ETSI TS 102 226 have been assigned to the Applet instance invoking this method and passing <code>READER_SERVICE_ID</code> as parameter, a <code>HCIException</code> with reason code <code>HCI_ACCESS_NOT_GRANTED</code> shall be thrown.

*

*

* @param serviceID shall be one of the constants (*)_SERVICE_ID defined

* in this class.

* @return reference to the permanent JCRE entry point object of the <code>HCIService</code> or

*         <code>null</code> if not yet registered.

* @throws HCIException with reason:

* <ul>

* <li>HCI_ACCESS_NOT_GRANTED if access to the indicated service is

* not permitted for the Applet

* <li>HCI_SERVICE_NOT_AVAILABLE if the requested service type is not available

* </ul>

* @exception SystemException with the following reason code: <ul>

*      <li><code>ILLEGAL_VALUE</code> <ul><li>if the serviceID does not match with the predefined values</li></ul>

*

*/

public static HCIService getHCIService(short serviceID) throws HCIException, SystemException {

return null;

}

……..

/**

* This method is used to retrieve information about the availability of a HCIService

* more related to user interface

* <p>

* If the Applet instance invoking this method passes <code>CARD_EMULATION_SERVICE_ID</code> as parameter and the Communication Interface Access Configuration of this Applet instance does not allow "Proximity-based communication" according to clause 5 of GlobalPlatform Card Specification Amendment C [8], the return value shall indicate "access not allowed for Applet".

*<p>

* If no "Parameters for contactless applications" as defined in ETSI TS 102 226 have been assigned to the Applet instance invoking this method and passing <code>READER_SERVICE_ID</code> as parameter the return value shall indicate "access not allowed for Applet".

*

*

* @param serviceID shall be one of the constants (*)_SERVICE_ID defined in this class

* @return (byte)0x00 if the service is available and can be used

*         otherwise bit values set to

*              <ul>

*              <li> bit 0 set: interface not supported by terminal

*              <li> bit 1 set: service not supported by terminal

*              <li> bit 2 set: access not allowed for Applet

*              <li> bit 3 set: HCI interface currently disabled in UICC

*              </ul>

*/

public static byte isHCIServiceAvailable(short serviceID) { return 0x00; }