| ETSI TC SCP Meeting #56 Munich, Germany, 27-28 September 2012 |  |  |  |  |  |  |  |  |  |  | Tdoc SCP(12)000161 |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ETSI TC SCP TEC Meeting #44 Sophia Antipolis, France, 10 - 13 Sept 2012 |  |  |  |  |  |  |  |  |  |  | Tdoc SCPTEC(12)000109r1 |  |  |  |  |  |  |  |  |  |  |  |  |
| TC SCP CR-Form-v1.8.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 102 705 |  |  |  | CR | 032 |  |  |  rev |  |  |  |  |  |  |  Current version: |  |  |  | 9.2.0 |  |  |  |
| For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects:  |  |  |  |  | smart card |  | x | terminal |  |  |  |  |  |  |  | server / network entity |  |  |  |  |  |  |  |
| Title:  |  | Clarification of service availability |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source:  |  | SCP TEC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item name:  |  | TEI9 |  |  |  |  |  |  |  |  |  |  |  |  | Date:  |  |  | 13/09/2012 |  |  |  |  |  |
| Category:  |  | F |  |  |  |  |  |  |  | Release:  |  |  |  |  |  |  |  | REL-9 |  |  |  |  |  |
|  |  | Use one of the following categories: F (essential correction) A (corresponds to a correction in an earlier release) B (Addition of feature), C (Functional modification of feature) D (Editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: REL-6 (Release 6) REL-7 (Release 7) REL-8 (Release 8) REL-9 (Release 9) REL-10 (Release 10) REL-11 (Release 11) |  |  |  |  |  |  |
| TC SCP use of the status "Accepted" for change requests: - At the TC SCP working group level, the status "Accepted" means that a change request is accepted for presentation to TC SCP Plenary. - At the TC SCP Plenary level, the status "Accepted" means that a change request is accepted for implementation in the targeted specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reason for change:  |  |  | SCP TEST requested a clarification about the difference between availability of services and unsupported interface. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change:  |  |  | Clarify that the indication is given for requested services that are supported in the terminal and the CLF and for the SWP/HCI interface. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if  not accepted: |  |  | Difficult to understand the specification for applet developers and SCP TEST can not complete test specification for REL-9. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| New tag value defined within the CR? |  |  |  | If ticked, add document number of related CR to TS 101 220: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected:  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs  |  |  |  | Other core specifications  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Affected: |  |  |  | Test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

/******************************************************************************

*

******************************************************************************/

package uicc.hci.framework;

import javacard.framework.*;

/**

* This class provides a factory to retrieve objects instances that implement sub

* interfaces of <code>HCIService</code>.

*

**/

public final class HCIDevice {

/** Identifies the service object for reader mode functionality */

public final static short READER_SERVICE_ID = (short)2;

/** Identifies the service object for card emulation mode functionality */

public static final short CARD_EMULATION_SERVICE_ID = (short)3;

/** Identifies the service object for connectivity functionality */

public final static short CONNECTIVITY_SERVICE_ID = (short)4;

/** UICC is activated in low power mode */

public final static byte LOW_POWER_MODE = (byte)0x00;

/** UICC is activated in full power mode */

public final static byte FULL_POWER_MODE = (byte)0x01;

/**

* private constructor

**/

private HCIDevice() {

}

/**

* This method is used to retrieve an instances that implement one of <code>HCIService</code>

* subinterfaces defined in this specification.

* <P>

* This method returns <code>null</code> if the <code>Applet.register()</code> method has not yet

* been invoked.

*

* @param serviceID shall be one of the constants (*)_SERVICE_ID defined

* in this class.

* @return reference to the permanent JCRE entry point object of the <code>HCIService</code> or

*         <code>null</code> if not yet registered.

* @throws HCIException with reason:

* <ul>

* <li>HCI_ACCESS_NOT_GRANTED if access to the indicated service is

* not permitted for the Applet

* <li>HCI_SERVICE_NOT_AVAILABLE if the requested service type is not available

* </ul>

* @exception SystemException with the following reason code: <ul>

*      <li><code>ILLEGAL_VALUE</code> <ul><li>if the serviceID does not match with the predefined values</li></ul>

*

*/

public static HCIService getHCIService(short serviceID) throws HCIException, SystemException {

return null;

}

/**

* This method returns the current power mode of the UICC as defined in TS 102 613.

* Possible values are:

* <ul>

* <li>LOW_POWER_MODE</li>

* <li>FULL_POWER_MODE</li>

* </ul>

* @return the current power mode or -1 if the power mode can not be retrieved

*/

public static byte getPowerMode() { return (byte)-1; }

/**

* This method is used to retrieve information about the availability of a HCIService

* more related to user interface

*

* @param serviceID shall be one of the constants (*)_SERVICE_ID defined in this class

* @return (byte)0x00 if the service is available and can be used

*         otherwise bit values set to

*              <ul>

*              <li> bit 0 set: SWP/HCI interface not supported by terminal

*              <li> bit 1 set: requested service not supported by terminal

*              <li> bit 2 set: access not allowed for Applet

*              <li> bit 3 set: HCI interface currently disabled in UICC

*              </ul>

*/

public static byte isHCIServiceAvailable(short serviceID) { return 0x00; }

}