| ETSI TC SCP Meeting #56 Munich, Germany, 27-28 September 2012 |  |  |  |  |  |  |  |  |  |  | Tdoc SCP(12)000162 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ETSI TC SCP TEC Meeting #44 Sophia Antipolis, France, 10 - 13 Sept 2012 |  |  |  |  |  |  |  |  |  |  | Tdoc SCPTEC(12)000110r1 |  |  |  |  |  |  |  |  |  |  |
| TC SCP CR-Form-v1.8.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 102 705 |  |  |  | CR | 033 |  |  |  rev |  |  |  |  |  Current version: |  |  |  | 9.2.0 |  |  |  |
| For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects:  |  |  |  |  | smart card |  | X | terminal |  |  |  |  |  | server / network entity |  |  |  |  |  |  |  |
| Title:  |  | Correct usage of Listener Interfaces |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source:  |  | SCP TEC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item name:  |  | TEI9 |  |  |  |  |  |  |  |  |  |  | Date:  |  |  | 13/09/2012 |  |  |  |  |  |
| Category:  |  | F |  |  |  |  |  |  | Release:  |  |  |  |  |  |  | REL-9 |  |  |  |  |  |
|  |  | Use one of the following categories: F (essential correction) A (corresponds to a correction in an earlier release) B (Addition of feature), C (Functional modification of feature) D (Editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: REL-6 (Release 6) REL-7 (Release 7) REL-8 (Release 8) REL-9 (Release 9) REL-10 (Release 10) REL-11 (Release 11) |  |  |  |  |  |  |
| TC SCP use of the status "Accepted" for change requests: - At the TC SCP working group level, the status "Accepted" means that a change request is accepted for presentation to TC SCP Plenary. - At the TC SCP Plenary level, the status "Accepted" means that a change request is accepted for implementation in the targeted specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reason for change:  |  |  | SCP TEST identified a potential issue caused by the assignment of the same numerical values to constants in three subinterfaces of HCIListener. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change:  |  |  | Events and Listener interfaces are related to each other in this specification. The same event values are used for different types of Listener interfaces. To identify these events on the applet level the application code has to check the type of the HCIMessage if one Class implements more then one interface. A sentence is added to make the developer aware of this. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if  not accepted: |  |  | Issues on the Applet level to identify the source of events |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| New tag value defined within the CR? |  |  |  | If ticked, add document number of related CR to TS 101 220: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected:  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs  |  |  |  | Other core specifications  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Affected: |  |  |  | Test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

/******************************************************************************

*

******************************************************************************/

package uicc.hci.framework;

import javacard.framework.Shareable;

/** The base interface for all types of HCI listeners.

* <P>

* An Object shall not implement this interface but only implement one of the subinterfaces.

*

* <P>

* An Applet instance implementing a <code>*Listener</code> interface shall only

* receive events defined in the same <code>*Listener</code> interface or in the

* <code>HCIListener</code> interface.

*<p>

* If a Class is implementing more then one <code>HCIListener</code>  subinterface the applet code should check the type of the  <code>HCIMessage</code> by using <code>instanceof</code> in order to identify the source of the notification.

* <P>

* Messages received through the inherited method <code>HCIListener.onCallback</code>

* shall be of the message type defined in the same package as the <code>*Listener</code> interface

* <P>

*

* @see uicc.hci.services.cardemulation.CardEmulationListener

* @see uicc.hci.services.connectivity.ConnectivityListener

* @see uicc.hci.services.readermode.ReaderListener

**/

public interface HCIListener extends Shareable {

/**

* This value is used to notify the Applet when the UICC failed to transmit a message to the CLF.

*

* The Applet has to activate this event (for a specific service) if it wants to be notified

* about data transmission errors (that occur for a specific service):

*/

public final static byte EVENT_HCI_TRANSMISSION_FAILED = (byte) 0x81;

/**

* This value is used to notify the Applet when the UICC failed to receive a message sent by the CLF.

*

* The Applet has to activate this event (for a specific service) if it wants to be notified

* about data transmission errors (that occur for a specific service):

*/

public final static byte EVENT_HCI_RECEPTION_FAILED = (byte) 0x82;

/** This method is called by the HCI framework to inform the Listener

*  Object about a specific event and pass the corresponding <code>HCIMessage</code> to

*  the Listener Object.

*

* In the case of an <code>EVENT_HCI_TRANSMISSION_FAILED</code> the message

* which could not be transmitted shall be made available in the <code>HCIMessage</code> object.

*

* If an Applet has requested the framework to send more than one message during a single

* <code>onCallback</code> method execution and a transmission to the CLF failed the framework

* shall not attempt to send any of the remaining messages.

*

* The <code>EVENT_HCI_RECEPTION_FAILED</code> shall not be raised in the case of a

* failure to receive a command from an external reader in CardEmulation Mode due to a problem

* on the data link layer

*

* @param  event that triggers the invocation of the method

* @param  message the HCIMessage

*/

public void onCallback(byte event, HCIMessage message);

}