TD <>

draft ETSI TS 102 705 V<1.3.0

> (2008-12)

Technical Specification

Smart Cards;

UICC Application Programming Interface for Java Card™ for Contactless Applications

(Release 9)

Reference

DTS/SCP-THCIAPI

Keywords

API, smart card, Java Card, contactless, UICC, Java

ETSI

650 Route des Lucioles

F-06921 Sophia Antipolis Cedex - FRANCE

Tel.: +33 4 92 94 42 00   Fax: +33 4 93 65 47 16

Siret N° 348 623 562 00017 - NAF 742 C

Association à but non lucratif enregistrée à la

Sous-Préfecture de Grasse (06) N° 7803/88

Important notice

Individual copies of the present document can be downloaded from:
http://www.etsi.org

The present document may be made available in more than one electronic version or in print. In any case of existing or perceived difference in contents between such versions, the reference version is the Portable Document Format (PDF). In case of dispute, the reference shall be the printing on ETSI printers of the PDF version kept on a specific network drive within ETSI Secretariat.

Users of the present document should be aware that the document may be subject to revision or change of status. Information on the current status of this and other ETSI documents is available at http://portal.etsi.org/tb/status/status.asp

If you find errors in the present document, please send your comment to one of the following services:
http://portal.etsi.org/chaircor/ETSI_support.asp

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© European Telecommunications Standards Institute yyyy.

All rights reserved.

DECTTM, PLUGTESTSTM, UMTSTM, TIPHONTM, the TIPHON logo and the ETSI logo are Trade Marks of ETSI registered for the benefit of its Members.
3GPPTM is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners.

Contents

Contents	4

Intellectual Property Rights	5

Foreword	5

1	Scope	5

2	References	5

2.1	Normative references	6

2.2 Informative references	6

3	Definitions, symbols and abbreviations	6

3.1	Definitions	6

3.2	Abbreviations	6

4	Description	7

4.1	Architecture	7

4.2 Card Emulation Mode	7

4.3 Reader  Mode	8

4.5 Connectivity Service	8

5 Interaction with	Proactive Functionality	8

6	Java Card Resource Handling	8

Annex A (normative): Java Card™ Platform HCI API for the UICC	9

Annex B (normative): Java Card ™ Platform HCI API for the UICC identifiers	10

History	10

# Intellectual Property Rights

IPRs essential or potentially essential to the present document may have been declared to ETSI. The information pertaining to these essential IPRs, if any, is publicly available for ETSI members and non-members, and can be found in ETSI SR 000 314: "Intellectual Property Rights (IPRs); Essential, or potentially Essential, IPRs notified to ETSI in respect of ETSI standards", which is available from the ETSI Secretariat. Latest updates are available on the ETSI Web server (http://webapp.etsi.org/IPR/home.asp).

Pursuant to the ETSI IPR Policy, no investigation, including IPR searches, has been carried out by ETSI. No guarantee can be given as to the existence of other IPRs not referenced in ETSI SR 000 314 (or the updates on the ETSI Web server) which are, or may be, or may become, essential to the present document.

# Foreword

This Technical Specification (TS) has been produced by ETSI Technical Committee Smart Card Platform (SCP).

# 1	Scope

The present document describes the UICC Application Programming Interface for Java Card(TM) for contactless Applications. Its purpose is to provide access for a contactless Applet to the services provided by the HCI protocol defined in [1] for the communication via the CLF. In the scope of this document contactless means support for the RF Technologies referenced by the HCI specification [1]. Low level functionality to manage gates and pipes as defined in the HCI specification [1] is not in the scope of this document. Registration of contactless parameters and management of contactless Applets in card emulation mode is defined in GlobalPlatform Amendment C [4] .

# 2	References

References are either specific (identified by date of publication and/or edition number or version number) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- Non-specific reference may be made only to a complete document or a part thereof and only in the following cases:

- if it is accepted that it will be possible to use all future changes of the referenced document for the purposes of the referring document;

- for informative references.

Referenced documents which are not found to be publicly available in the expected location might be found at http://docbox.etsi.org/Reference.

For online referenced documents, information sufficient to identify and locate the source shall be provided. Preferably, the primary source of the referenced document should be cited, in order to ensure traceability. Furthermore, the reference should, as far as possible, remain valid for the expected life of the document. The reference shall include the method of access to the referenced document and the full network address, with the same punctuation and use of upper case and lower case letters.

NOTE:	While any hyperlinks included in this clause were valid at the time of publication ETSI cannot guarantee their long term validity.

## 2.1	Normative references

The following referenced documents are indispensable for the application of the present document. For dated references, only the edition cited applies. For non-specific references, the latest edition of the referenced document (including any amendments) applies.

[1]	ETSI TS 102 622: "Smart card; UICC-Contactless Front-end (CLF) Interface, Host Controller Interface (HCI)"

[2]	ETSI TS 102 241: "Smart Cards; UICC Application Programming Interface (UICC API) for Java Card™

[3] 	ETSI TS 102 226: "Smart Cards; Remote APDU structure for UICC based applications"

[4]	GlobalPlatform (October 2009): "Contactless Services Card Specification V2.2 - Amendment C Version 0.87" (Note: This specification is in draft state. It is not published. This reference needs to be updated when newer versions of GP Amendment C are available.)

[5]	Sun Microsystems  "Application Programming Interface, Java Card™ Platform, 3.0 Classic Edition"

[6]	Sun Microsystems "Runtime Environment Specification, Java Card™ Platform, 3.0 Classic Edition"

[7]	Sun Microsystems "Virtual Machine Specification Java Card™ Platform, 3.0 Classic Edition"

NOTE: SUN Java Card Specifications can be downloaded at http://java.sun.com/products/javacard/specs.html

## Editor: I think we can skip this it is referenced via the Amendment C

## 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the following terms and definitions  apply:

contactless mode: is used as a generic term for "Card Emulation Mode" and "Reader  Mode"

RF Technology: radio frequency technology supported by the HCI [1] protocol specification.

## 3.2	Abbreviations

For the purposes of the present document, the following abbreviations apply

APDU		Application Programming Data Unit according to ISO/IEC 7816-3

HCI		Host Controller Interface according to TS 102 622 [1]

JCRE 		Java Card Runtime Environment according to [6]

# 4	Description

## 4.1	Architecture

The present document describes an API and a Contactless Framework that enables Java Card(TM) Platform based Applets, defined in [5] [6] [7] to send and receive messages using the HCI protocol defined in [1] and to act as contactless Applets. The Contactless Framework supports card emulation mode and reader mode as defined in the HCI protocol [1] specification.

The API is event driven and based on the Observer/Listener pattern. Every HCI service is encapsulated by a dedicated Service interface. This interface allows the registration of Listener Interfaces that have to be implemented by Java objects to receive HCI messages or events. The different HCI message types are identified by different types of interfaces

API methods that send a command will throw an exception in case the method is invoked a second time before the response to the last command is received (that has to be covered in the API with the HCI exception)

Applet originated HCI messages (events, commands, responses) are not guaranteed to be sent before the completion of the execution of the current Applet.

This system is guaranteed to be non reentrant.

a. An Applet shall not be invoked by a callback method during an ongoing Applet execution.

b. A onCallBackTriggering method shall not be invoked during an ongoing Applet execution.

c. A onCallBackTriggering method shall not be invoked while another Applet execution is ongoing.

d. A onCallBackTriggering method shall not be invoked while the execution of any earlier invocation of the same Applet is still ongoing.

e. Any onCallBackTriggering method shall not be invoked again while one of them is still being executed.

It is not required for the Contactless Framework to queue more then one incoming or outgoing HCI messages.

Clarification: same statement for outgoing messages, in case discarding which message gets discarded first or last

that would result     No queuing messages may be discarded

The API is split into two parts. One is a generic framework that provides a factory class to retrieve the different Service instances that are provided by the HCI implementation, and that allows to discover if the UICC is inserted into a HCI network. The second part of the API implements the Services that are defined for the HCI protocol, card emulation modereader mode and connectivity service.

Editor’s note: provide architectural diagram (like in TS 102 241, 31.130, …)

## 4.2 Card Emulation Mode

In card emulation mode there exist two exclusive ways to exchange messages over the HCI [1] protocol. The first is based on APDUs provided to the Applet through its "process" method as specified in "Application Programming Interface, Java Card™ Platform, 3.0 Classic Edition" [5]. The second is made available by the package uicc.hci.services.cardemulation defined in this specification.

The uicc.hci.services.cardemulation package shall provide the communication technologies for the card emulation mode defined in HCI[1]. The contactless runtime environment shall bind the services defined in the package to the underlying HCI resources (e.g. Gates and Pipes) as defined in HCI[1]. The parameters to be used by the HCI layer may be provided to the framework as defined in GlobalPlatform Amendment C[4].

For the API defined in this specification the card emulation capability shall be provided to Applets through a service interface implemented by the framework. Applet Instances shall receive CardEmulationMessages after the registration of a CardEmulationListener interface for the service. If no CardEmulationListener is registered by the Applet instance the javacard.framework.APDU class shall be used.

APDU go to the process method if the events EVENT_ON_SEND_DATA is deactivated

The select() and process() method are always invoked for a select command according to the JavaCard rules,  in case no APDU is used to select an Applet the process() method is not called because not SELECT BY AID command was submitted (check JCRE spec PICC selection for correct wording)

? what happens if the Applet is selected not with a APDU but with a pattern

It shall not be possible to switch between card emulation listener mode and JavaCard APDU mode (invocation through process) within a contactless session, i.e. not before the RF field has been turned off and on again. Editor’s note: Explore what needs to be added to the API to prevent switching between these modes (e.g. HCI exception in deregister method).Applets communicating through the process method shall also be able to use API services defined in this specification which do not require a CardEmulationListener registration. (e.g. request the power mode, connectivity service)

Applet selection and deselect shall be performed by the framework according to the rules defined in the "Java Card™ Runtime Environment Specification,  3.0 Classic Edition" [6] and in "GlobalPlatform Amendment C" [3].

Editor note: cover the case that for Selection of the Applet first the select method is and then the process() method is invoced after that all HCI Messages go to the CardEmulationListener if it is registered.

Editor’s note: The HCI event EVT_FIELD_OFF may be useful for applets even in process()-mode. If the RF field is off, the Applet is deselected according to GP Amendment C, the framework shall raise after the invocation of the deselect() method an EVT_FIELD_OFF if the event is activated for this Applet.

Editor’s note: We forgot to provide some mechanisms to support type F or CLT-mode. Editor: seams we need a special mechanism to inform an Applet that work over Type F that it is activated

Editor’s note: When the applet is invoked in the HCI mode (the non-process() mode), is the applet then selected? Does Amd-C apply at all when the applet is not selected? Will the select() and deselect() methods be called (by OPEN??)? Editor: My understand is that Amendmend C at the moment only cares about Applet where select and deselect are invoked, we may need a special method in CardEmulationListener like activate() deactivate()

## 4.3 Reader  Mode

The functionality to support the reader  mode is provided in the package uicc.hci.services.reader. In reader  mode  the communication technologies defined in HCI [1] for this mode are supported. The contactless runtime environment shall bind the services defined in  uicc.hci.services.reader to the corresponding resources (e.g. Gates and Pipes) defined in HCI [1] for the reader  mode.

An Applet has to be in the selectable state (according to the Java Card specification) to act as a contactless Applet in reader mode.

Reader mode Applets shall follow the extended lifecycle model that is defined in "GlobalPlatform Amendment C" [4] for contactless Applets in card emulation mode (i.e following Application Availability States and the  related transition rules).

The contactless registry parameters for reader mode Applets are specified in TS 102 226 [3].

When the state of a reader mode Applet changes to ACTIVATED (according to [4]) the framework shall ensure that the HCI gates and pipes are setup for the RF technologies that are supported by the reader mode Applet.

Per RF technology there shall be only one reader mode Applet in the state ACTIVATED (according to [4]) at any time.

## 4.4 Connectivity Service

The functionality to support the connectivity service as defined in HCI [1] is provided in the package uicc.hci.services.connectivity. The contactless runtime environment shall bind the services defined in uicc.hci.services.connectivity to the corresponding resources (e.g. Gates and Pipes) defined in HCI [1] for the connectivity service.

Editor’s note: Check if it makes sense to migrate the methods from ConnectivityMessage to ConnectivityService (for sake of simplicity and to avoid problems with sending connectivity events from the deselect() method (which is not possible with the current API.)).

# 5 Interaction with	Proactive Functionality

The ProactiveHandler defined in TS 102 241 [2] shall not be available when the contactless Applet is invoked with the callback methods defined in this specification, or when the Applet is invoked with the method process() of the Applet class defined in Application Programming Interface, Java Card™ Platform [5] (in card emulation mode). If the Applet wants to use proactive functionality it shall use the Connectivity Service functionality of the HCI [1] specification to trigger the terminal. All the proactive functionality of the CAT specification and the UICC API are then available to the Applet.

## 6	Java Card Resource Handling

The Runtime Environment triggers a contactless applet by calling call back method of the respective contactless service interface As a consequence all the rules defined in "Java Card™ 3.0 Classic (JCRE) Runtime Environment Specification" [6] apply (e.g. access to CLEAR_ON_DESELECT transient objects, context switch, multi selectable).

At the invocation of the call back method there shall be no transaction in progress.

The context as defined in Java Card shall be set to the context of the contactless applet. The previous context (context of the caller) shall be the context of the Runtime Environment.

# Annex A (normative):
Java Card™ Platform HCI API for the UICC

The source files for the UICC Application Programing Interface for Java Card™ for contactless Applets (102705_Annex_A_Java.zip and 102705_Annex_A_HTML.zip) are contained in draftTS-102705-Api-v002p0.zip, which accompanies the present document.

- Annex B (normative):
Java Card ™ Platform HCI API for the UICC identifiers

The export files for the uicc.hci.* package are contained in draftTS-102-XXX-Api-v002p0.zip, which accompanies the present document.

- Annex C (normative):
HCI API package version management

Table C.1 describes the relationship between each TS 102 705 specification version and its HCI API packages AID and Major, Minor versions defined in the export files.

Table C.1

| TS 102 705 |  | uicc.hci.framework package |  |  |
| --- | --- | --- | --- | --- |
|  | AID |  | Major, Minor |  |
| 9.0.0 |  | A0 00 00 00 09 00 05 FF FF FF FF 89 16 01 00 00 |  | 1.0 |

Table C.2

| TS 102 241 | uicc.hci.services.cardemulation package |  |
| --- | --- | --- |
|  | AID | Major, Minor |
| 9.0.0 | A0 00 00 00 09 00 05 FF FF FF FF 89 16 02 01 00 | 1.0 |

Table C.3

| TS 102 241 | uicc.hci.services.connectivity package |  |
| --- | --- | --- |
|  | AID | Major, Minor |
| 9.0.0 | A0 00 00 00 09 00 05 FF FF FF FF 89 16 02 02 00 | 1.0 |

Table C.4

| TS 102 241 | uicc.hci.services.readermode package |  |
| --- | --- | --- |
|  | AID | Major, Minor |
| 9.0.0 | A0 00 00 00 09 00 05 FF FF FF FF 89 16 02 03 00 | 1.0 |

The package AID coding is defined in TS 101 220 [Error: Reference source not found]. The HCI API packages' AID are not modified by changes to Major or Minor Version.

The Major Version shall be incremented if a change to the specification introduces byte code incompatibility with the previous version.

The Minor Version shall be incremented if a change to the specification does not introduce byte code incompatibility with the previous version.

# History

| Document history |  |  |
| --- | --- | --- |
| 0.0.2 | Nov-2008 | initial draft |
| 0.0.1 | Dec-2008 | Presented to ETSI SCP TEC #25 |
| 0.0.2 | 1st April-2009 | Send to ETSI SCP TEC e-mails list |
| 0.0.3 | 25 April-2009 | Updated based on feedback and based on GP discussions |
| 0.0.4 | 13 May-2009 | Version developed during the SCP TEC #27 |
| 0.0.5 | Sept-2009 | Version submitted to SCP TEC#28 |
| 0.0.6 | Sept-2009 | Created during SCP TEC #28 |
| 0.0.7 | October 2009 | Submitted to SCP TEC #29 |
| 0.0.8 | October 2009 | Created during SCP TEC #29 |
| 0.0.9 | October 2009 | Created during SCP TEC #29 |
| ?? |  |  |
| 1.0.0 | October 2009 | Sent ‘for information’ to SCP #43 |
| 1.1.0 | December 2009 | Created during SCP TEC #30 in document SCPt090458 |