| ETSI TC SCP Meeting #57 San Diego, US, 13-14 December 2012 |  |  |  |  |  |  |  |  |  |  |  |  | Tdoc SCP(12)000261 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ETSI TC SCP TEC Meeting #45 Sophia Antipolis, France, 26 - 29 Nov 2012 |  |  |  |  |  |  |  |  |  |  |  |  | Tdoc SCPTEC(12)000142r2 |  |  |  |  |  |  |  |  |  |
| TC SCP CR-Form-v1.8.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 102 241 |  |  |  | CR | 131 |  |  |  rev |  |  |  |  |  Current version: |  |  |  |  | 11.0.0 |  |  |  |
| For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects:  |  |  |  |  | smart card |  |  | terminal |  |  |  |  |  | server / network entity |  |  |  |  |  |  |  |  |
| Title:  |  | API services for high activity arrays |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source:  |  | SCP TEC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item name:  |  | M2M API |  |  |  |  |  |  |  |  |  |  |  |  | Date:  |  | 29/11/2012 |  |  |  |  |  |
| Category:  |  | B |  |  |  |  |  |  |  |  |  |  |  |  | Release:  |  | REL-11 |  |  |  |  |  |
|  |  | Use one of the following categories: F (essential correction) A (corresponds to a correction in an earlier release) B (Addition of feature), C (Functional modification of feature) D (Editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: REL-6 (Release 6) REL-7 (Release 7) REL-8 (Release 8) REL-9 (Release 9) REL-10 (Release 10) REL-11 (Release 11) |  |  |  |  |  |  |
| TC SCP use of the status "Accepted" for change requests: - At the TC SCP working group level, the status "Accepted" means that a change request is accepted for presentation to TC SCP Plenary. - At the TC SCP Plenary level, the status "Accepted" means that a change request is accepted for implementation in the targeted specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reason for change:  |  |  | SCP approved the CR SCP(11)0131 with a new requirement to allow the creation of objects with extended endurance in terms of update operations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change:  |  |  | A new class containing a set of methods for the creation of arrays suitable for intense update operations. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if  not accepted: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| New tag value defined within the CR? |  |  | x | If ticked, add document number of related CR to TS 101 220: |  |  |  |  |  |  |  |  |  | SCPTEC(12)000168 |  |  |  |  |  |  |  |  |
| Clauses affected:  |  |  | Clause 2.1, new chapter added , Annex C, new source file (in 102241_Annex_A_Java.zip), |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs  |  |  |  | Other core specifications  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Affected: |  |  |  | Test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## 2.1	Normative references

The following referenced documents are necessary for the application of the present document.

[1]	Void.

[2]	Sun Microsystems "Application Programming Interface, Java Card™ Platform, 3.0.1 Classic Edition".

[3]	Sun Microsystems "Runtime Environment Specification, Java Card™ Platform, 3.0.1 Classic Edition".

[4]	Sun Microsystems "Virtual Machine Specification Java Card™ Platform, 3.0.1 Classic Edition".

NOTE:	SUN Java Card Specifications can be downloaded at http://java.sun.com/products/javacard/specs.html.

[5]	ETSI TS 101 220: "Smart cards; ETSI numbering system for telecommunication application providers".

[6]	ETSI TS 102 221: "Smart cards; UICC-Terminal interface; Physical and logical characteristics".

[7]	ETSI TS 102 223: "Smart cards; Card Application Toolkit (CAT)".

[8]	ETSI TS 102 222: "Integrated Circuit Cards (ICC); Administrative commands for telecommunications applications".

[9]	ETSI TS 102 225: "Smart cards; Secured packet structure for UICC based applications".

[10]	ETSI TS 102 226: "Smart Cards; Remote APDU structure for UICC based applications".

[11]	ETSI TS 102 240: "Smart Cards; UICC Application Programming Interface and Loader Requirements; Service description".

[12]	ETSI TS 123 040: "Digital cellular telecommunications system (Phase 2+); Universal Mobile Telecommunications System (UMTS); Technical realization of Short Message Service (SMS) (3GPP TS 23.040 version 6.6.0 Release 6)".

[13]	ETSI TS 102 241: "Smart cards; UICC Application Programming Interface (UICC API) for Java Card (TM)".

[XX]	ETSI TS 102 671: "Smart Cards; Machine to Machine UICC; Physical and logical characteristics".

[YY] 				GlobalPlatform: "GlobalPlatform Card Specification, Version 2.2.1"

NOTE:	See http://www.globalplatform.org/.

[ZZ]				GlobalPlatform: Java Card API and Export File for Card Specification v2.2.1 (org.globalplatform)

v1.5

NOTE:	See http://www.globalplatform.org/.

[...]

# X	UICC Java Card Services

UICC Java Card Services are implemented as GlobalPlatform Global Services Applications according to the GlobalPlatform Card Specification [YY].  A unique service name identifies each service. Applets request a reference to a UICC Java Card Service through the following method defined in the GlobalPlatform API [ZZ]:

org.globalplatform.GPSystem.getService(javacard.framework.AID serverAID, short sServiceName)

The service names constant values are defined in the uicc.system.servicesConstants interface . The support for any of the UICC Java Card Services defined in this document is optional. In case a specific service is not supported, the getService() method invoked with the corresponding service name shall return null.

## X.X  High update arrays

The uicc.services.highupdatearray.HighUpdateArrayBuilder shareable interface is an optional UICC Java Card Service that provides the creation of Java Card arrays, called high update arrays, which support a specified number of update operations, which is expected to be higher than required for general data. Each update action on the array, regardless of the position and number of the updated element(s), is counted as one update operation. The service name to be used to obtain a reference to the HighUpdateArrayBuilder object is SERVICE_ID_HIGH_UPDATE_ARRAY_BUILDER.

The unavailability of this Service does not exclude that the platform may perform the management of frequently updated data transparently. In that case, the application may use the standard array feature.

If the high update arrays service is available, the UICC shall be classified with the JX property representing its update performance with reference to high update arrays, in a similar way as described in TS 102 671 [XX]. The JX property value indicates the UICC's expected minimum number of update operations supported for a high update array. The following JX property values are defined:

- JA: UICCs indicating JA as their minimum number of update operations property shall be able to update an high update array 100 000 times without failure; loss of information due to time factors is excluded from this property.

- JB: UICCs indicating JB as their minimum number of update operations property shall be able to update an high update array 500 000 times without failure; loss of information due to time factors is excluded from this property.

- JC: UICCs indicating JC as their minimum number of update operations property shall be able to update an high update array 1 000 000 times without failure; loss of information due to time factors is excluded from this property.

Applets can query the JX property value of the UICC by using the method HighUpdateArrayBuilder.makeHighUpdateObjectArray().

Annex C (normative):
UICC API package version management

[...]

Table C.x

| TS 102 241 [13] | uicc.services.highupdatearray package |  |
| --- | --- | --- |
|  | AID | Major, Minor |
| 11.0.0 | A0 00 00 00 09 00 05 FF FF FF FF 89 18 01 00 00 | 1.0 |

[...]

New folders in 102241_Annex_A_Java: uicc\services, uicc\services\highupdatearray

New file in 102241_Annex_A_Java: uicc\services\ highupdatearray\HighUpdateArrayBuilder.java

// PACKAGE DEFINITION

package uicc.services.highupdatearray;

//IMPORTS

import javacard.framework.*;

/**

* The <code>HighUpdateArrayBuilder</code> interface provides methods for creation of

* arrays supporting high usage of update operations with respect to

* Java Card conventional arrays. The minimum number of update operations

* is specified for the UICC by the JX property.

* These arrays are intended to contain data for which the update operations is expected to be

* higher than required for general data.

*/

public interface HighUpdateArrayBuilder extends Shareable{

/** JX Property value : High Update Array minimum update property, value JA corresponding to 100,000 update operations. Constant = (short)1;

*/

public static final short HIGH_UPDATE_ARRAY_MIN_UPDATE_JA           = (short)1;

/** JX Property value : High Update Array minimum update property, value JB corresponding to 500,000 update operations. Constant = (short)2;

*/

public static final short HIGH_UPDATE_ARRAY_MIN_UPDATE_JB           = (short)2;

/** JX Property value : High Update Array minimum update property, value JC corresponding to 1,000,000 update operations. Constant = (short)3;

*/

public static final short HIGH_UPDATE_ARRAY_MIN_UPDATE_JC           = (short)3;

/**

* Create a persistent <code>boolean</code> array with the specified array length and with

* extended endurance in terms of update operations with respect to arrays created

* by means of the Java <code>new</code> keyword.

*

* @param length   the length of the boolean array

* @exception SystemException with the following reason code: <ul>

*      <li><code>SystemException.NO_RESOURCE</code> if there are not enough resources in the

*	card to allocate the array</li>

*      </ul>

* @exception NegativeArraySizeException if the length parameter is negative

* @return    the new high update boolean array

*/

public boolean[] makeHighUpdateBooleanArray(short length);

/**

* Create a persistent <code>byte</code> array with the specified array length and with

* extended endurance in terms of update operations with respect to arrays created

* by means of the Java <code>new</code> keyword

*

* @param length   the length of the byte array

* @exception SystemException with the following reason code: <ul>

*      <li><code>SystemException.NO_RESOURCE</code> if there are not enough resources in the

*	              card to allocate the array</li>

*      </ul>

* @exception NegativeArraySizeException if the length parameter is negative

* @return    the new high update byte array

*/

public byte[] makeHighUpdateByteArray(short length);

/**

* Create a persistent <code>Object</code> array with the specified array length and with

* extended endurance in terms of update operations with respect to arrays created

* by means of the Java <code>new</code> keyword.

*

* @param length   the length of the Object array

* @exception SystemException with the following reason code: <ul>

*      <li><code>SystemException.NO_RESOURCE</code> if there are not enough resources in the

*	              card to allocate the array</li>

*      </ul>

* @exception NegativeArraySizeException if the length parameter is negative

* @return    the new high update Object array

*/

public Object[] makeHighUpdateObjectArray(short length);

/**

* Create a persistent <code>short</code> array with the specified array length and with

* extended endurance in terms of update operations with respect to arrays created

* by means of the Java <code>new</code> keyword.

*

* @param length   the length of the short array

* @exception SystemException with the following reason code: <ul>

*      <li><code>SystemException.NO_RESOURCE</code> if there are not enough resources in the

*	              card to allocate the array</li>

*      </ul>

* @exception NegativeArraySizeException if the length parameter is negative

* @return    the new high update short array

*/

public short[] makeHighUpdateShortArray(short length);

/**

* Return the value of minimum number of update operations property (JX property)

* for the high update arrays supported by the UICC.

*

* @return    a constant HIGH_UPDATE_ARRAY_MIN_UPDATE_Jx

*            indicating the JX property

*/

public short   getHighUpdateMinimumUpdateProperty();

}

New file in 102241_Annex_A_Java: uicc\system\servicesConstants.java

// PACKAGE DEFINITION

package uicc.system;

public interface servicesConstants

{

// --------------------------- Services Constants --------------------------

/** Service : High Update Array Builder service ID  = (short)0xA1WW;<br>

*            To be used with

*            org.globalplatform.GPSystem.getService(javacard.framework.AID serverAID,

*            short sServiceName)

*/

public static final short SERVICE_ID_HIGH_UPDATE_ARRAY_BUILDER           = (short)0xA1WW;

}